3GPP TSG-WG2 Meeting #160AH	S2-2401404

# Emeeting, 22- 29 Jan, 2024 	(was S2-2401104r02)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1261 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of UPF reporting of QoS Monitoring packet delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on LS in CT4 (S2-2311961 / C4-234616) when the PCF reports the measurement result needs to be further clarified when Minimum Waiting time is received for "event triggered" reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PCF shall not report the result during the Minimum Waiting time. The PCF sends the report that matches or exceeds the Reporting Threshold when the Minimum Waiting time is over. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement result reporting from PCF may be inaccurate since the result is received during the Minimum Waiting time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 6.1.3.26	Packet Delay Variation monitoring and reporting

Th AF may send a request to PCF for Packet Delay Variation (i.e. between UE to PSA UPF) monitoring and reporting, together with the request for packet delay monitoring and reporting. The PCF may generate the authorized QoS Monitoring policy for the service data flow based on both, Packet Delay Variation monitoring request and the QoS Monitoring request for packet delay.

The Packet Delay Variation monitoring request includes the following:

-	Packet Delay Variation (UL, DL or RT packet delay variation);

-	frequency of reporting (event triggered or periodic):

-	if the reporting frequency is event triggered:

-	the corresponding reporting thresholds;

-	minimum waiting time between subsequent reports.

NOTE 1:	The reporting period for the Packet Delay Variation monitoring is the same as the reporting period of the packet delay monitoring.

Based on the above Packet Delay Variation monitoring request, the QoS Monitoring request for packet delay received from the AF and any other AF requirements related to QoS monitoring, the PCF generates the authorized QoS monitoring policy for packet delay monitoring. The PCF shall subscribe to receive QoS Monitoring reports from SMF by setting the QoS Monitoring Policy Control Request Trigger. Then, the SMF reports the measured packet delay values to the PCF, the PCF derives the Packet Delay Variation and reports to the AF both, the packet delay measurements and Packet Delay Variation results under consideration of the respective thresholds and minimum waiting times (if they are applicable).

NOTE 2:	If event triggered reporting is used for packet delay monitoring and Packet Delay Variation monitoring, the PCF report can contain only packet delay measurements or Packet Delay Variation.

If the Reporting frequency indicates "event triggered", PCF shall send a report when the measurement result matches or exceeds the indicated Reporting threshold. Subsequent reports shall not be sent during the Minimum waiting time. After the Minimum waiting time is over, the PCF shall report if  measurement results are produced that match or exceed the indicated Reporting threshold.

# * * * * End of changes * * * *