SA WG2 Meeting #160-Ad Hoc-e 	S2-2401569

# Electronic meeting, 2024-01-22 – 2024-01-29

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5217 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDN Selection for URSP delivery in EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2024-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1) After 5GS to EPS handover the UE has no information about whether the SMF+PGW-C handling the transferred PDN connections support or not URSP delivery in EPS. Then, when there is a new PDN connectivity request after the 5GS to EPS handover, the UE doesn’t know whether to include or not the indication of URSP Provisioning Support in EPS and potentially transfer the UE Policy Container (UE STATE INDICATION) over the new PDN Connection. There could be cases where the transferring of such UE Policy Container for a new PDN connection could enable the possibility for the NW to deliver URSP updates in EPS by triggering the establishment of a UE Policy Association between the PCF for the PDU session and the PCF for the UE. E.g. if during the 5GS to EPS handover there is no any SMF+PGW-C supporting URSP delivery in EPS for the transferred PDN connections the UE Policy Association is not established and the PCF for the UE is not able to deliver any URSP update to the UE. Another example is in case the transferred PDN connections handled by SMF+PGW-C supporting URSP delivery in EPS are terminated before the establishment of a new PDN connection (in such a case the UE Policy Association is terminated between the PCF for the PDU session and the PCF for the UE). There could be also cases where if upon the new PDN Connection requests after 5GS to EPS handover, the UE includes the indication, the UE might send unnecessarily a UE Policy Container (UE STATE INDICATION) upon the reception of the Indication of URSP Provisioning Support in EPS included in ePCO in the PDN Connectivity Accept message. For example, this might happen if at least one SMF+PGW-C handling a transferred PDN connection supports URSP delivery in EPS (in such a case the PCF for the PDU session already established a UE Policy Association towards the PCF for a UE during 5GS to EPS handover). 2) If the UE has established several PDN connections with a SMF+PGW-C that supports URSP delivery in EPS but the PDN connection that was used for the UE for the delivery of the UE Policy Container (UE STATE INDICATION) is terminated, according to the current definition the PCF cannot deliver URSP updates to the UE over the remaining PDN connections with that SMF+PGW-C 3) During 5GS to EPS there could be multiple PCFs for the PDU session handling transferred PDN connections for the same UE and establishing UE Policy associations towards the PCF for the UE, but this scenario is not considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - During the establishment of a PDU session in 5GS the SMF+PGW-C provides to the UE the Indication of URSP Provisioning Support in EPS included in PDU Session Establishment Accept message. The UE checks later such indications after 5GS to EPS when triggering a new PDN Connectivity request in EPS to determine whether to include or not the indication of URSP Provisioning Support in EPS in PCO/ePCO. - Upon the reception of a UE Policy Container from the PCF for the UE, the PCF for the PDU session may select any of the ongoing PDN connections for which the SMF+PGW-C supports URSP delivery in EPS to deliver the UE Policy Container to the UE. The UE can receive ePCO with UE Policy Container over any of the established PDN connections. - During 5GS to EPS the PCF for the UE may handle multiple UE Policy Associations for the same UE established from different PCFs for the PDU session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Undefined behavior of the UE upon new PDN Connectiviy requests after 5GS to EPS handover, that in some cases might prevent the possibility for the NW to deliver URSP updates to the UE. Lack of support for URSP delivery in EPS when multiple PDN connections are established and one them is terminated. Undefined behavior for URSP delivery in EPS after 5GS to EPS handover with multiple PCFs for the PDU session handling transferred PDN connections for a UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 5.17.8	URSP Provisioning in EPS

When the UE registers in 5GS, the UE includes the Indication of URSP Provisioning Support in EPS in the UE Policy Container carried in Registration Request. On receiving this indication in the UE Policy Container, the PCF will enable the provisioning of the URSP to UE in EPS.

The UE may include the indication of URSP Provisioning Support in EPS in PCO or ePCO to the SMF+PGW-C during:

-	Initial Attach with default PDN connection establishment (according to clause 5.3.2.1 of TS 23.401 [26]).

-	Any UE requested PDN connectivity request to an additional PDN (according to clause 5.10.2 of TS 23.401 [26].

If the SMF+PGW-C supports URSP provisioning in EPS, it provides the Indication of URSP Provisioning Support in EPS in ePCO in the Create Session Response message. The SMF+PGW-C may also provide the Indication of URSP Provisioning Support in EPS during PDU session establishment in 5GS within PDU Session Establishment Accept message.

When the UE receives an indication of URSP provisioning support in EPS in the PDN Connectivity Accept message or in PDU Session Establishment Accept and this PDN connection or the corresponding transferred PDN connection is not released, then for any subsequent PDN connectivity requests the UE does not include an indication of URSP Provisioning Support in EPS.

When the UE receives the Indication of URSP Provisioning Support in EPS included in ePCO in the PDN Connectivity Accept message, then the UE initiates the UE requested bearer resource modification without QoS update procedure and includes the UE Policy Container ePCO in the Request Bearer Resource Modification message, the UE Policy Container in ePCO will be further forwarded by MME to SMF+PGW-C. When the UE Policy Container ePCO is received by SMF+PGW-C, it forwards transparently the UE Policy Container to PCF for the PDU Session, then the PCF for the PDU Session establishes the UE Policy Association with PCF for the UE. The PCF for the UE generates the corresponding URSP rules in a similar way as it is done in 5GS and sends the URSP rules to UE in the UE Policy Container as described in clause 4.11.0a.5 of TS 23.502 [3].

NOTE 1:	The UE includes the Indication of URSP Provisioning Support in EPS in PCO or ePCO in the PDN connectivity request according to clause 6.6.1.1 of TS 24.301 [13].

NOTE 2:	The MME and Serving-GW can be configured to prioritize the selection of a SMF+PGW-C that support URSP Rule provisioning in EPS.

If the UE does not receive the Indication of URSP Provisioning Support in EPS in ePCO in the PDN Connectivity Accept message, then the UE does not initiate the UE requested bearer resource modification procedure to send the UE Policy Container.

The PDN Connection used by UE and SMF+PGW-C to convey UE Policy Container PCO shall be kept when the UE is in the CONNECTED mode. When the UE is attached to EPS, the PCF for the PDU Session retrieves the PCRTs for UE Policy from PCF for the UE and subscribes to the applicable PCRTs to SMF+PGW-C. The PCF for the PDU Session may send the UE Policy Container in the same PDN connection in which the UE Policy Container was received or in any additional PDN connection established with a SMF+PGW-C supporting URSP delivery in EPS.

During EPS to 5GS mobility with N26, the UE Policy Association is terminated by PCF for PDU Session when it receives the indication of RAT type change from the SMF+PGW-C.

During 5GS to EPS mobility with N26, the PCF for the PDU Session determines whether the UE supports URSP delivery in EPS by checking UE context policy control subscription information in UDR. If the UE and at least one of the SMF+PGW-C for which the UE maintains a PDU session support URSP delivery in EPS the PCF for the PDU session establishes a UE Policy Association with the PCF for the UE. The PCF for the PDU Session discovers the address of PCF for the UE serving the UE by querying BSF. The PCF for the UE recovers the information about the PSI list in the UE and the subscribed PCRTs in 5GS from former UE Policy Association for the UE after receiving the UE Policy Association Establishment request including a UE Policy Container only including an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS mobility").

After the 5GS to EPS mobility, if PCF for the UE needs to provision the URSP to UE, the PCF for the UE sends the UE Policy Container in Npcf_UEPolicyControl_UpdateNotify Request to the PCF for the PDU Session as described in clause 4.11.0a.2a.10 of TS 23.502 [3]. In case there are more than one PDN connections for the UE, then the PCF for the PDU Session selects any of the ongoing PDN Connections via a SMF+PGW-C supporting URSP delivery in EPS for the UE. Then via the selected PDN Connection, the SMF+PGW-C sends the UE Policy Container via ePCO to UE by initiating the PDN GW initiated bearer modification without QoS update procedure as defined in clause 5.4.3 of TS 23.401 [26].

NOTE 3:	At 5GS to EPS mobility with N26, the guard timer in the AMF (as specified in clauses 4.11.1.2.1 and 4.11.1.3.2 of TS 23.502 [3]) ensures that the UE Policy Association remains until the PCF for the UE detects that a UE Policy Association establishment is received from a PCF for the PDU Session indicating 5GS to EPS mobility.

NOTE 4:	In the case that the UE is still registered and reachable over non-3GPP access, the PCF for UE can have two UE policy associations for one UE, and based on local configuration the PCF can decide to use one of the UE policy associations to update URSP rule to the UE. The UE can receive URSP Rules over any of these two accesses.

NOTE 5:	In the case that during the 5GS to EPS mobility there are more than one PCF for the PDU session maintaining PDN connections for the UE every involved PCF for the PDU session can establish a UE Policy Association towards the PCF for the UE. In such case when the PCF for the UE needs to provision the URSP to UE, the PCF for the UE first selects one of those UE Policy Associations for the delivery of URSP rules to the UE in EPS.

The UE can receive ePCO with UE Policy Container over any of the established PDN connections and then response over the same PDN connection.

When the PCF for the UE decides to update the URSPs in the UE via EPS, the PCF for the UE sends the updated URSP rules in UE Policy Container to the PCF for the PDU Session, then the PCF for the PDU Session forwards it to the SMF+PGW-C. The SMF+PGW-C transfers the received UE Policy Container via ePCO to the UE by triggering PDN-GW initiated Bearer without QoS Modification procedure as described in clause 5.4.3 of TS 23.401 [26]. After update the UE policy provided by the PCF, the UE response about the delivery result via ePCO to the network. The SMF+PGW-C transparently forwards the UE response to the PCF for the PDU Session via Update Bearer Response message, and then the PCF for the PDU Session forwards it to the PCF for the UE via Npcf_UEPolicyControl_Update Request. If the SMF+PGW-C receives a rejection for Update Bearer Request message (e.g. due to paging failure), the delivery failure result is sent to PCF for the PDU Session and the PCF for the UE. To request to forward the result of delivery of UE policies, "Result of UE Policy Container delivery via EPS" PCRT is applied to the PCF for the PDU Session and the SMF+PGW-C as described in clause 4.11.0a.2a.10 of TS 23.502 [3].

* * * End of Changes * * *