SA WG2 Meeting #S2-160	S2-2311994

13 - 17 November, 2023, Chicago, USA

Title:	[draft] Reply LS on LS from CT WG3: LS on PDN connection selection for URSP provisioning in EPS

Response to:	(S2-2311958/C3-234513) LS on PDN connection selection for URSP provisioning in EPS

Release:	Release 18

Work Item:	eUEPO

Source:	Samsung [to be: SA2]

To:	CT3

Cc:	CT1

Contact person:	DongYeon Kim

dy522.kim@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR 5071, 23.502 CR 4547 [to be: updated with the agreed CRs]

1	Overall description

SA2 thanks CT3 for their LS on PDN connection selection for URSP provisioning in EPS.

SA2 has discussed the questions raised by CT3:

In 3GPP TS 23.501 clause 5.17.8, it is specified that during 5GS to EPS mobility with N26 the PCF recovers the former 5GS UE Policy Association for the UE in EPS. However, in case URSP needs to change in EPS (after handover from 5GS) and a UE Policy Container needs to be sent to the UE it is unclear how the network selects a PDN connection for the delivery of a UE Policy Container towards the UE (e.g. the UE has more than one ongoing PDN connection).

Excerpt from 3GPP TS 23.501 clause 5.17.8:

During 5GS to EPS mobility with N26, the PCF for the PDU Session determines whether the UE supports URSP delivery in EPS by checking UE context policy control subscription information in UDR. The PCF for the PDU Session discovers the address of PCF for the UE serving the UE by querying BSF. The PCF for the UE recovers the information about the PSI list in the UE and the subscribed PCRTs in 5GS from former UE Policy Association for the UE after receiving the UE Policy Association Establishment request including a UE Policy Container only including an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS mobility").

Question to SA2:	For a UE in EPS with N26 after handover from 5GS, how does the network select an ongoing PDN connection for the delivery of a UE Policy Container towards the UE (e.g. the UE has more than one ongoing PDN connection).

And SA2 would like CT3 to consider the answer provided as below:

SA2 Answer:	During 5GS to EPS mobility with N26, the SMF+PGW-C selects one PDU Session among the PDU Sessions moved from 5GS and reports the RAT Type change, and the PDN Connection associated with that PDU Session is used for delivery of the UE Policy Container towards the UE.

2	Actions

To CT3

ACTION: 	SA2 kindly asks CT3 to take the above into account.

3	Dates of next TSG SA WG2 meetings

SA2#Ad Hoc	22nd   – 26th January 2024 		Online

SA2#161	26th  February  – 01st May 2024 		Athens, GR