3GPP SA WG2 Meeting #160	S2-2312026

# 13-17 November 2023, Chicago, IL, USA	(was S2-230xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5072 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provisioning separate DL and UL PDU Set QoS Parameters to NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed in S2-2312008, clause 5.7.7.2 states that "a QoS Flow is associated with only one PDU Set Delay Budget. The value of the PDU Set Delay Budget is the same in UL and DL." Equivalent text on PSER is included in clause 5.7.7.3. However, it appears that this text was copied from PDU QoS without much discussion because it is unrealistic to assume that PSDB, PSER and PSIHI will always be the same in uplink and downlink direction for an IP flow: - UE support for detecting PDU Sets is optional. Similarly, a UE application may not provide enough information to a UE to detect PDU Sets. As a result, UL PDU Set QoS cannot be supported. At the same time, it should still be possible to support PDU Set QoS in downlink direction even if a UE (or a UE application) does not support UL PDU Set QoS. This however requires the network to be able to signal PSDB, PSER and PSIHI applicable to downlink direction only to NG-RAN. - Furthermore, in general it cannot be assumed that PSDB and PSER would necessarily be the same in uplink and downlink for all XR applications. For instance, virtual reality (VR) applications are very downlink heavy due to the delivery of rendered images but only require very little uplink information, e.g., to convey head movement information. Also, it is not obvious that PDU Set QoS should be applied to the uplink traffic of such VR applications. - In line with this, also the PDU Set Integrated Handling Information may be different in UL and DL direction given that the application requirements in terms of whether all PDUs of a PDU Set are needed or not may vary in UL and DL direction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in clauses 5.7.7.2, 5.7.7.3, 5.7.7.4 that SMF provides PSDB, PSER and PSIHI separately for uplink and downlink direction to NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Separate DL and UL PDU Set QoS Parameters to NG-RAN not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.7.2, 5.7.7.3, 5.7.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 5.7.7.2	PDU Set Delay Budget

The PDU Set Delay Budget (PSDB) defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. the duration between the reception time of the first PDU (at the N6 termination point for DL or the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE for DL or N6 termination point for UL). PSDB applies to the DL PDU Set received by the PSA UPF over the N6 interface, and to the UL PDU Set sent by the UE.

NOTE:	To enable support for PSDB, it is required that a maximum inter arrival time between the first received PDU and the last received PDU of a PDU Set complies with SLA. This maximum inter arrival time does not exceed PSDB. NG-RAN behaviour when the SLA is not fulfilled is out of scope of this specification.

A QoS Flow is associated with one UL PDU Set Delay Budget value, one DL PDU Set Delay Budget value or both. DL PSDB and UL PSDB are optional parameters that may be provided by the PCF. The provided PSDBs can be used by the NG-RAN to support the configuration of scheduling and link layer functions.

When the PSDB is available, the PSDB supersedes the PDB for the given QoS Flow.

The AN PSDB is derived at NG-RAN by subtracting CN PDB (as described in clause 5.7.3.4) from the PSDB.

#### 5.7.7.3	PDU Set Error Rate

The PDU Set Error Rate (PSER) defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PSER defines an upper bound for a rate of non-congestion related PDU Set losses. The purpose of the PSER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access).

NOTE 1:	In this Release, a PDU Set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully.

NOTE 2:	How RAN enforces PSER is up to RAN implementation.

A QoS Flow is associated with one UL PDU Set Error Rate value, one DL PDU Set Error Rate value or both. PSER are optional parameters. If the PSER is available, the PSER supersedes the PER.

#### 5.7.7.4	PDU Set Integrated Handling Information

The PDU Set Integrated Handling Information (PSIHI) indicates separately in UL and DL direction whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side. PSIHI is an optional parameter.

# >>>>END OF CHANGES<<<<