SA WG2 Meeting #S2-160	S2-2312055

13 - 17 November, 2023, Chicago, USA

# Title:	[Draft] Reply LS on 5G ProSe UE-to-UE relay discovery with security aspects

# Response to:	S2-2311951(C1-237897)

# Release:	Rel-18

# Work Item:	5G_ProSe_Ph2

Source:	Interdigital (to be SA2)

To:	CT1, SA3

Cc:

Contact Person:

Name:	Jung Je Son

Tel. Number:	jungje dot son at interdigital dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR 0385

1. Overall Description:

SA2 would like to thank CT1 for their LS on 5G ProSe UE-to-UE relay discovery with security aspects.

SA2 would like CT1 to consider following answers.

Question from CT1 in C1-234362:

Question: How does the 5G ProSe UE-to-UE Relay UE send the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE to the corresponding discoverer 5G ProSe End UE, considering 1) the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in the protected direct discovery set are transparent to the 5G ProSe UE-to-UE Relay UE and 2) the 5G ProSe UE-to-UE Relay UE may serve more than one discoverer 5G ProSe End UE simultaneously?

Answer from SA2:

SA2 has acknowledged the issue from CT1 and agreed to the following solution (captured in the attached CR). Upon receiving a UE-to-UE Relay Discovery Solicitation message from a Discoverer End UE, the U2U Relay includes a Direct Discovery Solicitation ID in a UE-to-UE Relay Discovery Solicitation message sent to the Discoveree End UE. The U2U Relay associates the Direct Discovery Solicitation ID with the Discoverer End UE’s source L2 ID. The U2U Relay uses the Direct Discovery Solicitation ID included in a UE-to-UE Relay Discovery Response message to locate the Discoverer End UE’s source L2 ID so that the UE-to-UE Relay Discovery message is sent to the Discoverer UE.

2. Actions:

To CT1 and SA3 group

ACTION 1: SA2 kindly asks CT1 and SA3 to take the above information into account.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#160 Ad hoc-e | January 22 - 29th, 2024 | Online | Online |
| SA2#161 | February 26 th - March 01st, 2023 | Athens | Greece |

Please see https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/