SA WG2 Meeting #160	S2-2312077

# November 13-17, 2023; Chicago, IL, USA

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Key issue for WT4: Network information exposure enhancements

Document for: 	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM / Rel-19

Abstract of the contribution: Propose key issue for WT4.

# 1 Discussion

This key issue is related to WT4 as below.

WT#4 Network exposure: Study whether and how XR related network capability/information (e.g. if the QoS profile requested by AF cannot be met, network can indicate the alternative QoS profile) can be exposed towards the application layer.

To support network exposure enhancements towards application layer, the following issues need to be considered:

-	Whether and how to expose the network information e.g. QoS profile/Alternative QoS profile/Alternative PDU set QoS parameters to application server via user plane, to support the traffic rate/codec adaptation, with improving the E2E transmission performance of XR traffic.

# 2 Proposal

It is proposed to update TR 23.700-70 as follows.

# * * * * First change * * * *(all new texts)

## X.X	Key Issue #X: Enhancement for network information exposure

### X.X.1	Description

This key issue studies whether and how to enhance XR services related network information exposure to enable better awareness of network information or capability towards application layer, e.g. if the QoS profile requested by AF cannot be met, network can indicate the alternative QoS profile.

The objective of this Key Issue is to study whether and how to enhance network exposure mechanism to better support the network information/capability exposed to the application layer. In particular, the key issue includes the following aspects:

-	Whether and how to expose the network information e.g. QoS profile/Alternative QoS profile/Alternative PDU set QoS parameters to application server via user plane, to support the traffic rate/codec adaptation, with improving the E2E transmission performance of XR traffic.

# * * * * End of change * * * *