# 3GPP TSG-SA WG2 Meeting #160	S2-2312164

November 13 - 17, 2023, Chicago, USA	(Revision of S2-230xxxx)

Title:	[Draft]Reply LS on INACTIVE eDRX above 10.24sec and SDT

Response to:	S2-2311527/R3-235765, S2-2308320/R3-233347, S2-2311930/C4-233691

Release:	Rel-18

# Work Item:	     NR_REDCAP_Ph2, NR_redcap_enh-Core, NR_MT_SDT-Core

Source:	SA2

To:	RAN3, CT4

Cc:	RAN2

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.501 CR 4771 (S2-231xxxx), TS 23.502 CR 4323 (S2-231xxxx)

1. Overall Description:

SA2 thanks RAN3 for the LS (R3-233347) on “INACTIVE eDRX above 10.24sec and SDT” which lead to further investigation as indicated in SA2 reply LS S2-2309757 with the following:

Answer 2: When NGAP DL DATA NOTIFICATION is triggered by downlink data on user plane, the signalled 5QI is the same as the one stored in RAN UE context. For the case of NGAP DL DATA NOTIFICATION is triggered by MT signalling, SA2 will continue the discussion and provide further response.

Answer 3: SA2 discussed the issue and considers it’s technically feasible for UPF/SMF to provide the DL data size of the first downlink packet to NG-RAN via AMF. SA2 also discussed if more accurate data size is needed besides the first packet, but no consensus could be reached.

SA2 asks RAN3 to provide view regarding if indicating the first downlink packet data size by 5GC is sufficient for NG-RAN to decide the use of MT-SDT.

For A2 above, SA2 discussed further and concluded that when NGAP RAN Paging Request is triggered for MT signalling, the 5QI may be different from the one stored in RAN UE context. SA2 further agreed to introduce the signalling indication which can co-exists with the 5QI. See attached CRs

For A3 above, SA2 thanks again RAN3 for the further reply LS (R3-235765/ S2-2311527) on “INACTIVE eDRX above 10.24sec and SDT” and provided the following response to CT4 and RAN3 in reply LS S2-2311359 with following:

SA2 answer:

SA2 discussed potential solutions and would like to provide the following feedback:

- SA2 has concluded that providing accurate information to NG-RAN regarding the most recent cumulative data size of data buffered per QFI in the UPF has some impact on the N4 interface. Namely, the N4 procedures today (as SA2 understands them) allow the UPF to send the Downlink Data Notification only upon arrival of the first packet, which is insufficient for providing accurate information on the cumulative data size of data buffered per QFI at the UPF at the end of the long eDRX cycle.

SA2 would like to kindly ask CT4 to indicate whether SA2’s understanding of the N4 procedures today is correct and to provide feedback on the feasibility of N4 procedure enhancements that would allow for more accurate provision of the cumulative data size of the data buffered per QFI at the UPF, while taking into account the cost of such enhancements in term of specification impact and the incurred additional N4 signalling load.

SA2 discussed this further and concludes that providing most recent cumulative data size per QFI has major impact on the N4/N11 interfaces and NF selection functionality. Considering it’s end of the Rel-18 work and the limited benefit at RAN side with the cost of such major changes at the CN side, SA2 concluds that CN provides only data size information when the UPF triggers the downlink data notification based on existing functions specified for N4 interface. SA2 agreed the attached CRs based on above conclusion.

2. Actions:

To RAN3, CT4

ACTION: 	SA2 kindly asks RAN3 and CT4 to take above conclusion into consideration.

3. Date of Next SA2 Meeting:

SA2-Ad Hoc-e   		January 22 - 29, 2024 				E-meeting

SA2#161   			February 26 – March 1, 2024				Athens, GR