3GPP TSG-SA2 Meeting #160	S2-2312207

# November 13 – 17, 2023, Chicago, USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5090 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Support for ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.32.1, there are some typos and redundant and ambiguous sentences. This paper is to clarify the clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on clause 5.32.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | less understandable clause |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

5.32.1	General

The ATSSS feature is an optional feature that may be supported by the UE and the 5GC network.

The ATSSS feature enables a multi-access PDU Connectivity Service, which can exchange PDUs between the UE and a data network by simultaneously using one 3GPP access network and one non-3GPP access network and two independent N3/N9 tunnels between the PSA and RAN/AN. The multi-access PDU Connectivity Service is realized by establishing a Multi-Access PDU (MA PDU) Session, i.e. a PDU Session that may have user-plane resources on two access networks. This assumes both 3GPP access and non-3GPP access are allowed for the S-NSSAI of the PDU Session.

After the establishment of a MA PDU Session, and when there are user-plane resources on both access networks, the UE applies network-provided policy (i.e. ATSSS rules) and considers local conditions (such as network interface availability, radio signal conditions, user preferences, etc.) for deciding how to distribute the uplink traffic across the two access networks. Similarly, the PSA UPF of the MA PDU Session applies network-provided policy (i.e. N4 rules) and considers feedback information received from the UE (such as Availability of access network) for deciding how to distribute the downlink traffic across the two access networks via two N3/N9 tunnels. When there are user-plane resources on only one access network, the UE may establish or activate the user-plane resources over another access network based on the ATSSS rules and local conditions.

The type of a MA PDU Session may be set as one of the following types defined in clause 5.6.1; IPv4, IPv6, IPv4v6, and Ethernet. In this release of the specification, the Unstructured type is not supported. The clause 5.32.6.2.1, the clause 5.32.6.2.2, and the clause 5.32.6.3.1 define which Steering Functionalities can be used for each supported type of a MA PDU Session.

The 3GPP PS Data Off feature for the MA PDU Session is described in clause 5.24.

The ATSSS feature supportsany type of access networks, including untrusted and trusted non-3GPP access networks (see clauses 4.2.8 and 5.5), wireline 5G access networks (see clause 4.2.8), etc. as long as a MA PDU Session can be established over these types of access networks.

In this release of the specification, a MA PDU Session does not support IPv6 multi-homing (see clause 5.6.4.3) and UL Classifier (see clause 5.6.4.2).

In this release of the specification, for ATSSS feature it is assumed that the SMF Service Areas covers the whole PLMN or that a MA PDU Session is released over both access networks when the UE moves out of the SMF Service Areas.

A MA PDU Session does not support LADN. If the AMF receives a request to establish a MA PDU Session for a LADN DNN, the AMF shall reject the request. If the AMF receives a request to establish a PDU Session for a LADN DNN with "MA PDU Network-Upgrade Allowed" indication, the AMF shall not forward this indication to the SMF.

If the UE moves from the source AMF supporting the ATSSS feature to the target AMF which is not, the MA PDU Session is released as described in TS 23.502 [3].

NOTE 1:	Deployment of the ATSSS feature that is homogeneous per PLMN or network slice enables consistent behaviour. In the case of non-homogenous support of the ATSSS feature in a PLMN/slice (i.e. some NFs in a PLMN/slice may not support the ATSSS feature), the MA PDU Sessions can be released due to the UE mobility.

A MA PDU Session may, for the 3GPP access network and/or non-3GPP access network, use user-plane resources of an associated PDN Connection in EPC (e.g. one 3GPP access network path via EPC and one non-3GPP access network path via 5GC or one 3GPP access network path via 5GC and one non-3GPP access network path via ePDG/EPC). Such use of the ATSSS feature with EPS interworking may apply to the PDU Session of Ethernet, IPv4, IPv6, or IPv4v6 type and PDN Connection type.

NOTE 2:	Co-existence with NBIFOM is not defined. It is assumed that NBIFOM and the multi-access connectivity described in this clause are not deployed in the same network.

NOTE 3:	To the MME and SGW this is a regular PDN Connection and the support for the ATSSS feature is transparent to MME and SGW.

For a MA PDU Session for the Ethernet PDU Session type, if the UE has not indicated the support for Ethernet PDN connection type or if the network does not support Ethernet PDN connection type, when the 3GPP access network uses user-plane resources of an associated PDN Connection, the following takes place:

-	The SMF+PGW-C considers that the MA PDU Session is still using the Ethernet PDU Session / PDN Connection type but in a restricted mode which EPS signalling can only refer to non-IP PDN Connection type.

-	MAR rules in the UPF are still used for distributing DL traffic across the 3GPP access network and the non-3GPP access network.

-	For traffic on the 3GPP access network, the SMF may update N4 rules and QoS rules/EPS bearer contexts on the UE to take into account that no QoS differentiation is possible over the 3GPP access network.

Support of MA PDU Sessions using one leg associated with PDN Connection in EPC and the other leg associated with PDU Session in 5GC is further defined in TS 23.502 [3].

The following clauses specify the functionality that enables the ATSSS feature.

* * * End of Changes * * *