3GPP TSG-SA WG2 Meeting #160	S2-2312251

# Chicago, USA, November 13 – 17, 2023								      (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0395 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on sending U2U Relay Discovery Response message using unicast Layer-2 ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT1 sent SA2 the LS on 5G ProSe UE-to-UE relay discovery with security aspects (S2-2311951/C1-237897) to ask SA2 about the following question. - As specified in TS 33.503 clause 6.1.3.3.3.2, for 5G ProSe UE-to-UE Relay Discovery with Model B, the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE are included in a protected direct discovery set and sent between the discoverer 5G ProSe End UE and the discoveree 5G ProSe End UE via the 5G ProSe UE-to-UE Relay UE. Question: How does the 5G ProSe UE-to-UE Relay UE send the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE to the corresponding discoverer 5G ProSe End UE, considering 1) the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in the protected direct discovery set are transparent to the 5G ProSe UE-to-UE Relay UE and 2) the 5G ProSe UE-to-UE Relay UE may serve more than one discoverer 5G ProSe End UE simultaneously? As pointed out in the LS from CT1, since User Info IDs of the discoverer 5G ProSe End UE in the protected direct discovery set are transparent to 5G ProSe UE-to-UE Relay, it is not feasible for the UE-to-UE Relay to correlate a Discovery Response message to a specific discoverer End UE using a unicast Destination Layer-2 ID. To resolve this issue, it is proposed that the Destination Layer-2 ID is a unicast Layer-2 ID which is set to the Source Layer-2 ID of the received 5G ProSe UE-to-UE Relay Discovery Solicitation message as described in TS 23.304 clause 5.8.4.2 It is also proposed to clarify that UE-to-UE Relay Discovery Response message is sent to all discoverer UEs that have sent UE-to-UE Relay Discovery Solicitation message to the UE-to-UE Relay, if the RSC contained in the UE-to-UE Relay Discovery Response message received from the discoveree End UE matches any of the (pre)configured RSC(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that UE-to-UE Relay Discovery Response message is sent to all discoverer UEs that have sent UE-to-UE Relay Solicitation message to the UE-to-UE Relay, if the RSC contained in the UE-to-UE Relay Discovery Response message received from the discoveree End UE matches any of the (pre)configured RSC(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear stage 2 specification about sending UE-to-UE Relay Discovery Response message causes confusion to CT1 protocol design. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.3.2.4.3	Procedure for 5G ProSe UE-to-UE Relay Discovery with Model B

Depicted in Figure 6.3.2.4.3-1 is the procedure for 5G ProSe UE-to-UE Relay Discovery with Model B.

Figure 6.3.2.4.3-1: 5G ProSe UE-to-UE Relay Discovery with Model B

1.	The discoverer 5G ProSe End UE (UE-1) sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, User Info ID of itself, RSC and User Info ID of the discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

A 5G ProSe UE-to-UE Relays determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

2.	A 5G ProSe UE-to-UE Relay that matches the RSC sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, User Info ID of the discoverer 5G ProSe End UE (UE-1), User Info ID of UE-to-UE Relay, RSC and User Info ID of the discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

A 5G ProSe End UE determines the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

3.	The discoveree 5G ProSe End UE (UE-2) that matches the RSC and the User Info ID of the discoveree 5G ProSe End UE (UE-2) responds to the 5G ProSe UE-to-UE Relay with a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, RSC, User Info ID of the discoverer 5G ProSe End UE (UE-1) and User Info ID of discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4. If the discoveree 5G ProSe End UE (UE-2) receives multiple UE-to-UE Relay Discovery Solicitation messages from different 5G ProSe UE-to-UE Relays with the same RSC and the User Info ID of the discoveree 5G ProSe End UE (UE-2), it may choose to respond or not to a 5G ProSe UE-to-UE Relay (e.g. based on the PC5 signal strength of each message received).

4.	If the RSC contained in the Response message matches any of the (pre)configured RSC(s), as specified in clause 5.1.5.1, of the 5G ProSe UE-to-UE Relay, the 5G ProSe UE-to-UE Relay sends a 5G ProSe UE-to-UE Relay Discovery Response message to all discoverer 5G ProSe End UE(s) that have sent the 5G ProSe UE-to-UE Relay Discovery Solicitation message to the 5G ProSe UE-to-UE Relay as described in step 1. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, User Info ID of UE-to-UE Relay, RSC and User Info ID of the discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

* * * * End of Changes * * * *