3GPP TSG-WG SA2 Meeting #160	S2-2312266

Chicago, US, Nov 13 – 17, 2023

Title:	[DRAFT] Reply LS on Clarifications related to Network Slice Replacement feature

Response to:	LS (S2-2311957/C3-234473) on Clarifications related to Network Slice Replacement feature

Release:	Rel-18

Work Item:	eNS_Ph3

Source:	[Huawei, ZTE to be] 3GPP SA2

To:	3GPP CT3

Cc:

Contact Person:

Name:	Haiyang Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.502 CR 4451

1. Overall Description:

SA2 would like to thank CT3 for their LS on Clarifications related to Network Slice Replacement feature.

Specific to the CT3 questions in C3-234473, SA2 would like to provide the following feedback:

In the current TS 23.503, clause 6.1.3.5:

| Network Slice Replacement | The SMF reports the event of change between S-NSSAI and Alternative S-NSSAI to PCF when the SMF determines that the PDU Session and SM Policy Association can be retained. The SMF provides Alternative S-NSSAI when the PDU Session is transferred from S-NSSAI to Alternative S-NSSAI. | Added | PCF |  |
| --- | --- | --- | --- | --- |

Question 1: When the original S-NSSAI is replaced by an Alternative S-NSSAI, should both the original S-NSSAI and the Alternative S-NSSAI be provided by the SMF to the PCF (and from PCF to BSF)?

Answer:

In case of Network Slice Replacement during PDU Session Establishment, the SMF provides both the original S-NSSAI and Alternative S-NSSAI to the PCF. In this case, the PCF registers the BSF using the original S-NSSAI since SA2 agrees that Network Slice Replacement procedure should be transparent to the Application Function

In case of Network Slice Replacement for the existing PDU Session and SMF determines that the existing PDU Session can be retained, the SMF provides the Alternative S-NSSAI to the PCF via the PCRT defined in clause 6.1.3.5 of TS 23.503 so that the PCF knows the S-NSSAI is to be replaced with the Alternative S-NSSAI. In this case, the PCF does not need to provide any information to the BSF.

Question 2: For the case where the original S-NSSAI is recovered and then the PDU session is transferred back to the original S-NSSAI, what information (if any) is expected that the SMF provides to the PCF (and from PCF to BSF)?

Answer:

If the SMF determines that the existing PDU Session can be retained, the original S-NSSAI is provided from SMF to the PCF so that the PCF knows the PDU session is transferred back from the Alternative S-NSSAI to the original S-NSSAI. In this case, the PCF doesn’t need to provide any information to the BSF.

In the current TS 23.501, clause 5.15.19:

The AMF determines the Alternative S-NSSAI for a UE registered with the S-NSSAI based on the notification from NSSF or PCF, or based on local configuration if the NSSF or PCF do not provide an alternative S-NSSAI. The Alternative S-NSSAI shall be supported in the UE Registration Area. If AMF cannot determine the Alternative S-NSSAI for the S-NSSAI, e.g. PCF or NSSF doesn't provide Alternative S-NSSAI, the AMF may further interact with the PCF to determine the Alternative S-NSSAI. The event trigger in AMF for interacting with PCF is described in clause 6.1.2.5 of TS 23.503 [45]

Question 3: In case of roaming LBO scenario, does the V-PCF need the mapping of V-PLMN S-NSSAI with H-PLMN S-NSSAI information for network slice replacement policy decision during roaming LBO scenario

Answer: In case of roaming, the AMF only reports the VPLMN S-NSSAI that requires slice replacement to the V-PCF to decide the V-PLMN Alternative S-NSSAI since the HPLMN S-NSSAI is not required to be replaced for LBO roaming. Therefore, the mapping of V-PLMN S-NSSAI with H-PLMN S-NSSAI is not needed for the V-PCF, i.e V-PLMN S-NSSAI value and corresponding mapped HPLMN S-NSSAI value.

Please see details in the attachment.

2. Actions:

To CT3 group.

ACTION: 	SA2 asks CT3 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #160-Ad Hoc-e	22nd – 29th January, 2024			Online

TSG-SA2 Meeting #161		26th February – 1st March, 2024		Athens, GR