3GPP TSG-SA2 Meeting #160	S2-2312311

Chicago, USA, November 13th -17th, 2023

Source:	Lenovo

Title:	Discussion on separating different kinds of service data flows for charging and data rate reporting

Document for:	Discussion

Agenda Item:	9.9.2

Work Item / Release:	AIMLsys/ Rel-18

Abstract of the contribution: Discussion on separating different kinds of service data flows for charging and data rate reporting

1	Discussion

As stated in the LS from SA5 (S2-2310135), SA5 assumes the existing Flow Based Charging (FBC) can be used, to apply differentiated charging for the AI/ML traffic from any other applications traffic, under appropriate data flow filters and dedicated AI/ML rating group (i.e. charging key) or rating group + service-id. However, to the best of SA5 knowledge, there is no capability specified, to allow on a per application basis, charging differentiation between the application usage and the AI/ML traffic the application is associated to.

It means there is no existing mechanism to differentiate the AI/ML traffic from other type of traffics sent from the same application and the charging differentiation cannot be enabled on a per application basis. To make this work, some information needs to be provided from the AF to separate different kind/type of traffics sent from the same application. The PCF can use this information and assign different charging using the existing Flow Based Charging, based on its configuration.

Proposal #1: The AF may provide the traffic type associated with the service data flow to the 5GC when sending a request to reserve resources for an AF session. The traffic type will indicate the traffic type of the current service data flow, e.g., AI/ML traffic, etc.

Besides, it is stated in the section 8.6.3 of TR 23.700-80 [1], the conclusion of Key Issue #6, that the 5GS should bind the AI/ML traffic to distinct QoS flow (i.e. not sharing QoS flow with other applications). It means no other service or applications should be bound or added to this QoS flow.

The existing data rate report is in the granularity of QoS flow. However, if AI/ML traffic and other type of traffics are all bound into the same QoS flow, the data rate calculation for AI/ML traffic could be mixed up and wrong. There are mainly two options to enable differentiated data rate reporting. Option 1 is to enhance the existing QoS flow based data rate report to SDF based data rate report, which has impact on PCF, SMF and UPF. Option 2 is to bind AI/ML traffic to an exclusive QoS flow by giving the traffic type of the service data flow, which has impact on PCF and SMF. In order to avoid more specification work, option 2 is preferred.

Proposal #2: The PCF may include the traffic type in the PCC rules provided to the SMF to bind the AI/ML traffic (indicated by the traffic type) to distinct QoS flow.

2	Proposals

Based on the discussion above, it is proposed to discuss and agree the following proposals:

Proposal #1: The AF may provide the traffic type associated with the service data flow to the 5GC when sending a request to reserve resources for an AF session. The traffic type will indicate the type of the current service data flow, e.g., AI/ML traffic, etc.

Proposal #2: The PCF may include the traffic type in the PCC rules provided to the SMF to bind the AI/ML traffic (indicated by the traffic type) to distinct QoS flow.

In line with the proposals, this paper proposes to approve the companion 23.502 and 23.503 CRs in S2-2312309 and S2-2312308.

3	Reference

[1] 3GPP TR 23.700-80, Study on 5G System Support for AI/ML-based Services, Release 18.