# SA WG2 Meeting #S2-160	S2-2312329

# 13 - 17 November, 2023, Chicago, US

# Title:	DRAFT: LS on Ranging/SL Positioning service exposure security and privacy check

# Response to:	-

# Release:	Release 18

# Work Item:	Ranging_SL

Source:	SA2

To:	SA3

Cc:	SA1, SA6

Contact Person:	Lars Nord

Lars dot Nord at Sony dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	 Add agreed CRs

1. Overall Description:

SA2 has discussed the procedures for authorization of UE for Ranging/SL positioning service exposure as specified in TS 33.533 v18.0.0. SA2 has observed that in this version of the specification one of the service exposure options specified in TS 23.586 v18.1.0 is not covered i.e., Ranging/SL Positioning service exposure through 5GC U-plane.

TS 33.533 clause 6.3.6.2 specifies that SL-MO-LR authorization of the SL Positioning Client UE is reused for Ranging/SL positioning service exposure through 5GC control plane. This authorization is based on AMF subscription check, to check whether the UE is allowed to act as SL Positioning Client UE. After the authorization is done the GMLC will perform SL-MT-LR procedure that includes the LCS privacy check. SA2 has identified that the UE LCS Privacy Profile needs to be updated with a list of approved SL Positioning Client UEs to make the LCS Privacy check efficient and not require the GMLC to check with the UEs whether each SL Positioning Client UE is allowed to access location information of the UEs. The agreed CR is attached.

SA2 has further discussed whether the principles for service exposure via the control plane can be applied to when the service exposure via the user plane/NEF is invoked by a third UE. SA2 concluded that:

- The AMF performs subscription check whether the UE is allowed to act as a SL Positioning Client UE during the registration before the PCF provisions the SL Positioning Client UE with the necessary Onboarding information. This would correspond to the AMF subscription check in the SL-MO-LR case.

- Both service exposure options (CP and UP) would trigger the GMLC to execute the SL-MT-LR procedure that includes the LCS Privacy check. If same LCS Privacy check is done, then the same level of subscriber aware control can be provided.

Based on these conclusions SA2 updated service exposure features and procedures, see attached CRs. SA2 has also updated the UE LCS privacy profile information as attached CR.

2. Actions:

To SA3

ACTION:

SA2 kindly asks SA3 to take above conclusions and updated solutions into consideration and provide feedback if necessary.

3. Date of Next SA2 Meetings:

SA2-Ad Hoc	29 January – 2 February 2023	E-meeting

SA2#161	26 February – 1 March 2023	Athens, GR