3GPP TSG-SA2 Meeting #160		S2-2312345

Chicago, USA, 13 – 17 November

Source:	vivo

Title:	Discussion on RAN1 AI progress and agreement and potential SA2 impact

Agenda Item:	5

Document for:	Information

1	Background

SA2 are discussing Rel.19 "SID on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML)" and S2-2310034 is technically endorsed as the baseline for further work at SA2.

The working task #1 as captured in S2-2310034 has strong dependency with RAN and SA WG2 sent LS out S2-2311921.zip to TSG RAN and RAN WGs, requesting feedback on whether there is any requirement for SA2 to support AI/ML for air interface and NG-RAN in RAN.

WT#1: AI/ML cross-domain coordination aspects

- Study enhancements to support AI enabled RAN based on conclusions of the RAN study. The WT will discuss whether and how to support the cross domain (i.e. UE, RAN, 5GC, OAM and AF) collaborative AI/ML mechanisms to support the UE, the RAN, the 5GC and the AF for the aspects described by the work tasks below. The WT will also discuss interaction/coordination with RAN to support the AI enabled RAN framework:

-	WT1.1 – Study enhancements to UE data collection framework. Study whether and how to enhance UE data collection framework to meet requirements for RAN AI support for air interface operation (for RAN). This includes identifying what benefit can be achieved from enhanced UE data collection for 5GC, and the potential impacts on the 5G framework, including potential enhancements to policy control. Regarding the radio related data collected from UE or RAN, e.g, channel status information and beam information, the WT will also discuss the data leakage from the operator's domain which should be avoided.

-	WT1.2 – Study 5GC support for AI/ML model and information sharing with the UE. Study whether (and how) to support model transfer/delivery to the UE according to RAN1/RAN2 considerations, including potential enhancements to policy control. Whether and what entities or functions transfer the AI/ML model or information to the UE will be studied as part of the work. This WT will also discuss the data leakage from the operator's domain which should be avoided.

-	WT1.3: Study whether and how to support the alignment of model identification and model management between SA2 and RAN. Work will be based on the possible requirements defined by RAN1 and RAN2.

- WT1.4: Study whether and how to support interaction/coordination with RAN3 to support the AI enabled NG-RAN framework (i.e. AI/ML for NG-RAN in Rel-18). Work will be based on possible requirements from RAN3.

- WT1.5: Study whether and how to consider enhancements to LCS to support AI/ML based Positioning.

This papers have two parts:

- Part A: Considering RAN may need some time to send overall response to SA2 (as LS to RAN groups touched quite a few aspects ), this paper tries to analyse whether RAN1 Positioning aspects may have SA impact (based on current RAN1 agreements and discussions, which is catured in TR 38.843).

- Part B: Other aspects (e.g. data collection and Model transfer/deliver) are not very clear at this moment and need further discussion in RAN groups in November meeting, therefore this paper just gives very draft oberservations in Annex for information.

2	Discussion

In this section, we mainly analyse AI/ML based Positioning which the RAN1 aspects may need SA work.

In RAN1#110b, 5 cases have been agreed for AI/ML based positioning accuracy enhancement and it is recommended to proceed with normative work for AI/ML based positioning.

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Potential SA impacts:

(related to WT1.5: Study whether and how to consider enhancements to LCS to support AI/ML based Positioning)

- Case 2a, 2b and 3b are directly related to LMF, which would have potential SA2 impact

Obersevation: RAN is expected to support AI/ML based positioning in Rel-19, which would have potential SA2 imapct and require a corresponding study in SA2 in Rel-19.

Please see RAN1 agreements of LMF related positioning which are listed in the following (clause 5.3 and 8, TR 38.843).

Agreement - Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning Agreement [Unrelated parts are omitted.] Measurements, signalling and procedures were studied to enable AI/ML for positioning accuracy enhancements with NR RAT-dependent positioning methods and is recommended to be further investigated in normative work, and specified if necessary. A variety of enhancements for measurements (e.g., based on extensions to current positioning measurements or with new measurements) were identified as potentially beneficial (e.g., trade-off positioning accuracy requirement and signalling overhead) and are recommended to be investigated further and if needed, specified during normative work. Based on conducted analysis, it is recommended to proceed with normative work for AI/ML based positioning. Conclusion For all five positioning cases (Case 1/2a/2b/3a/3b), RAN1 has not considered prioritization.

3 Summary

- Regarding AI/ML based postioning, this paper is to  to summary RAN1 agreements and discussions  and point out there are some potential impacts on SA2 e.g. Case 2a, 2b and 3b are directly related to LMF.

Annex

The following aspects e.g. Model transfer/deliver are not very clear at this moment and need further discussion in RAN groups in November meeting, therefore this secion only gives some very draft oberservations for information.

Model transfer/deliver

The agreements of model transfer/delivery are listed in the following:

- Collabration levels x, y and z have been identified in RAN1#109e.

- In RAN1#112, Cases y, z1 to z5 for model delivery/transfer have been agreed to facilitating discussion.

- Further details of Cases z4 and z5 have been clarified.

- Some observations of model transfer/delivery have been agreed and some pros and cons of model transfer/delivery are still under discussion.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Obervation1 (related to WT1.2: Study 5GC support for AI/ML model and information sharing with the UE)

- These model transfer/delivery cases consider different model delivery/transfer, model storage location and training location, and provide different representative scenarios for detailed study.

- These different scenarios are highly related to SA2 and corresponding feasibility need to be assessed by SA2.

The RAN1 agreements of model transfer/delivery are listed in the following (clause 4.2 and 4.3, TR 38.843).

Agreement Take the following network-UE collaboration levels as one aspect for defining collaboration levels - Level x: No collaboration - Level y: Signaling-based collaboration without model transfer - Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings. FFS: Clarification is needed for Level x-y boundary Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery. Agreement To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side. Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. Note: The Case definition is NOT intended to introduce sub-levels of Level z. Note: Other cases may be included further upon interest from companies. FFS: Z4 and Z5 boundary Agreement In model delivery/transfer Case z4, the “known model structure” means an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. In model delivery/transfer Case z5, the “unknown model structure” means any other model structure not covered in z4, including any model structure that is only partially known. Agreement - When a model of a known structure at UE (e.g., Case z4) is transferred from NW, the new model being identified (e.g., via Type B2) has the same structure as an previously identified model at the Network and UE - Note: the need of model transfer will be discussed separately

Data collection

Data collection has been identified as one of the main LCM aspects in early meetings. Potential specification impact related to data collection may be measurement configuration and reporting, contents, type and format of data, signaling of assistance information for categorizing the data and signaling for data collection procedure.

In previous reply of RAN2 LS (R1-2308730 and R1-2310681) on data collection requirements and assumptions, data colletion entities, data content, typical data size (per data sample) and typical latency requirement of different LCM prupose have been identified [1][2].

There are two main solutions of collected data report.

- One solution is the CP based reporting mainly for small number of data samples. Legacy CSI framework may be largely reused for this solution.

- The other solution is the UP based reporting mainly for large number of data samples. In the existing framework of the UP based solution, data would be collected from UE Application(s) and Event Exposure (EVEX) mechanism. The detailed UP based solution is worthy of study by SA.

Obervation 2 (related to WT1.1: Study enhancements to UE data collection framework.)

- UP based reporting for large number of data samples e.g. UE Application(s) and Event Exposure (EVEX) mechanism

- data colletion entities, data content, typical data size (per data sample)

The RAN1 agreements of data collection are listed in the following(clause 4.2, TR 38.843).

Conclusion Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) Agreement Consider at least the following aspects and if applicable, the corresponding potential specification impact related to data collection: - Measurement configuration and reporting - Contents, type and format of data including: - Data related to model input - Data related to ground truth - Quality of the data - Other information - Signaling of assistance information for categorizing the data - Note: The study should consider the feasibility of disclosure of proprietary information - Signaling for data collection procedure - Note 1: Use-case specific details can be studied in respective agenda items - Note 2: Signaling mechanism details can be studied by appropriate working groups.

Model/functionality identification

How model/functionality identification works have been discussed a lot and their denfinitions have been agreed in RAN1#112b

- Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

- Model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

  - Model ID in RAN1 discussion may or may not be globally unique, and different types of model IDs may be created for a single model for various LCM purposes.

Obervation 3 (related to WT1.3: Study whether and how to support the alignment of model identification and model management between SA2 and RAN)

- The definition of Model ID

- The information needed for alignment of model ID would need signaling exchange between network sided servers and UE sided servers

The RAN1 agreements of model/functionality identification are listed in the following (clause 4.2, TR 38.843).

Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). - FFS: relationship between functionality-based LCM and model-ID-based LCM - Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion. Conclusion From RAN1 perspective, it is clarified that an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. - When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model. Agreement For model identification of UE-side or UE-part of two-sided models, categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact (if any). - Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signaling - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signaling after model identification. - FFS: Spec impact to other WGs - Type B: Model is identified via over-the-air signaling, - Type B1: - Model identification initiated by the UE, and NW assists the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - FFS: details of steps - Type B2: - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - FFS: details of steps - Note: The support and applicability of each model identification Type is a separate discussion. This study does not imply that model identification is necessary. Agreement For functionality/model-ID based LCM, - Once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring. Agreement - Once models are identified, UE can indicate supported AI/ML model IDs for a given AI/ML-enabled Feature/FG in a UE capability report as starting point. - FFS: applicability to model identification, Type A, type B1 and type B2 - FFS: Using a procedure other than UE capability report - Note: model identification using capability report is not precluded for type B1 and type B2 Agreement - Model ID in RAN1 discussion may or may not be globally unique, and different types of model IDs may be created for a single model for various LCM purposes. - Note: Details can be studied in the WI phase.

References

Qualcomm Incorporated, R1-2308730, “Reply LS on Data Collection Requirements and Assumptions”, RAN1#114, Aug 21th - 25th, 2023.

Qualcomm Incorporated, R1-2310681, “Reply LS on Data Collection Requirements and Assumptions”, RAN1#114bis, Oct 9th - 13th, 2023.