3GPP TSG-WG2 Meeting #160	S2-2312466

# Chicago, USA, November 13-17 , 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4600 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reinstating Namf_Communication_NonUeN2MessageTransfer for TSS reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During CT4#118 meeting C4-234110 has been postponed, and this CR was replacing a trigger for NG-RAN to start reporting its timing synchronization status. The initial trigger (Namf_Communication_NonUeN2MessageTransfer message from TSCTSF that further leads to N2 message from the AMF to NG-RAN node(s)) was proposed to be replaced by Namf_Communication_NonUeN2InfoSubscribe, in which case the AMF could have an additional logic to determine whether there is a need to send an N2 meesage to the respective NG-RAN leading to some signalling savings in some (uncommon) scenarios. There is no unanimous support for such changes in Stage 3, and Stage 2 needs re-evaluate the need for such changes now when Stage 3 have expressed some concerns. Note that Namf_Communication_NonUeN2MessageTransfer has been removed by S2-2309819. # ---Some discussions-- In case of Namf_Communication_NonUeN2MessageTransfer, the AMF will transparently forward the NAGP message to the NG-RAN node(s) without considering e.g. DNN/S-NSSAI or which TSCTSF have made this request. There is no impact on AMF. In case of Namf_Communication_NonUeN2InfoSubscribe, the AMF could have an additional logic, determining which NG-RAN nodes have already started repoting their TSS information and to which TSCTSF. Moreover, if a NG-RAN already reports its TSS, there is no need for the AMF to send yet another N2 message as the NG-RAN has been already instructed to start the reporting. The info about TSCTSF needs to be stored by AMF so that it is able to stop the TSS reporting correctly when there is not a single TSCTSF with the subscription left in case when any of the TSCTSF-s decides to cancel the subscription (by invoking the Namf_Communication_NonUeN2InfoUnSubscribe) Finally, regarding multiple TSCTSF-s requesting the TSS reporting from NG-RAN node(s). Although hypothetically there could be such case, since “If the TSCTSF is locally configured in NFs, it shall be ensured that the same TSCTSF Set is configured in all NFs (e.g. NEF, AF and PCF) for the given DNN and S-NSSAI. NOTE: Thus, it is assumed that there is only one TSCTSF Set for a given DNN/S-NSSAI in this Release of the specification.” – clause 6.3.24 of TS 23.501 However, considering that this time synchronization feature exploits DNN and slice unspecific aspects, such as info broadcasting in SIB, it is uncommon scenarios. Hence, to our understanding, the potential gain (in signalling savings) is not worth complexity and impacts it imposes on AMF. # --- Proposal: Reinstate the use of Namf_Communication_NonUeN2MessageTransfer for triggering RAN timing synchronization status reporting towards the TSCTSF. Additinonally, we clarify in Step 3 that “As part of this subscription, the TSCTSF may specify TA(s) and/or NG-RAN node(s) the subscription needs to be applied. Based on local configuration and/or TA or NG-RAN node information as received from the TSCTSF, the AMF may subscribe to some or all NG-RAN nodes in the TA(s).” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Reinstating the Namf_Communication_NonUeN2MessageTransfer as a trigger for an N2 message that would serve as an indication to NG-RAN nodes to start TSS reporting to the AMF - Adding that the TSCTSF may specify TA(s) or NG-RAN nodes identifiers for the AMF about whose timing synchronization status it wants the subscription to be applied. - Clarifying that “Based on local configuration or TA or NG-RAN node information as received from the TSCTSF, the AMF may subscribe to some or all NG-RAN nodes in the TA(s).” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TSS reporting cannot be performed, incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.9.5.1, 5.2.2.2.17, 5.2.2.2.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

##### 4.15.9.5.1	Network timing synchronization status

Figure 4.15.9.5.1-1: Procedure for TSCTSF subscription to RAN and/or UPF/NW-TT timing synchronization status

0.	The AF requests creation or modification of ASTI or PTP based time synchronization service as described in clauses 4.15.9.4 and 4.15.9.3 including clock quality detail level and clock quality acceptance criteria (if applicable) in the request.

If the request is received at the NEF, it checks whether the AF is authorized to send the request and forwards the request to the TSCTSF.

If network timing synchronization status reports are provisioned using node-level signalling via control plane, the TSCTSF determines the serving AMF(s) and the UPF/NW-TT nodes (if applicable) for the UE(s) that needs to initiate network timing synchronization status monitoring.

Otherwise, if network timing synchronizations status reports are provisioned via OAM, steps 1-3 and 6-7 are skipped.

1-2.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution or by PTP instance activation, modification):

Upon the reception of the clock quality detail level and clock quality acceptance criteria (if applicable) in the AF request in step 0, the TSCTSF needs to be subscribed to NG-RAN timing synchronization status updates at the NG-RAN nodes that may provision access stratum time distribution information to the target UE. NG-RAN timing synchronization status updates provisioning may be configured via AMF (with node level signalling as illustrated in steps 1-3).

The TSCTSF sends an Namf_Communication_NonUeN2MessageTransfer containing the RAN timing synchronization status indication to the AMF; this triggers N2 messages to NG-RAN node(s), see Step 2, that may start reporting the gNB timing synchronization status information (specified in Table 5.27.1.12-1 of TS 23.501[1]) to the AMF.

3.	The TSCTSF invokes an Namf_Communication_NonUeN2InfoSubscribe to initiate the subscription for the NG-RAN timing synchronization status updates from the AMF. As part of this subscription, the TSCTSF may specify TA(s) and/or NG-RAN node(s) the subscription needs to be applied. Based on local configuration and/or TA or NG-RAN node information as received from the TSCTSF, the AMF may subscribe to some or all NG-RAN nodes in the TA(s).

4.	(When the procedure is triggered by the AF request for PTP instance activation, modification and if the UPF/NW-TT is involved in providing time information to DS-TT):

Upon the reception of the clock quality acceptance criteria in the AF request in step 0, the TSCTSF needs to be subscribed to UPF/NW-TT timing synchronization status updates at the UPF/NW-TT that may provision time information via PTP to the target UE. UPF/NW-TT timing synchronization status updates provisioning may be configured via OAM or via UMIC.

5.	The RAN node is pre-configured for the thresholds for each timing synchronization status attribute as described in clause 5.27.1.12 of TS 23.501 [2]. When the network timing synchronization status exceeds or meets again any of the pre-configured thresholds, the NG-RAN node detects a change on its timing synchronization status (e.g., degradation, failure, improvement).

6-7.	If the NG-RAN node detects a change on its timing synchronization status and timing synchronization status reporting is configured via the AMF in step 1, the NG-RAN node notifies the AMF providing a NG-RAN timing synchronization status update. The update can contain the information elements listed in Table 5.27.1.12-1 of TS 23.501 [2], and the scope of the timing synchronization status (as described in clause 5.27.1.12 of TS 23.501 [2]). The AMF forwards the update to the subscribed TSCTSF.

8.	If the UPF/NW-TT detects a change on its timing synchronization status and timing synchronization status reporting is configured via UMIC in step 4, the UPF/NW-TT notifies the TSCTSF providing a UPF/NW-TT timing synchronization status update via UMIC. The update can contain the information elements listed in Table 5.28.3.1-2 of TS 23.501 [2].

9.	Upon the reception of a change in the NG-RAN and/or NW-TT timing synchronization status update, the TSCTSF shall determine if the UE is impacted and whether the clock quality acceptance criteria can still be met. For each scope of the timing synchronization status received from the RAN in step 6, if the status indicates degradation or failure, TSCTSF may set the Area of Interest to gNB node ID(s) or Cell IDs in the scope of the timing synchronization status. The TSCTSF uses the resulted Area of Interest to subscribe for the UE presence in Area of Interest from the AMF that is serving the gNB node ID as described in clause 5.27.1.12 of TS 23.501 [2]. If the status indicates recovery, the TSCTSF may remove the corresponding scope from the subscription.

Upon reception of notification for the UE presence in the Area of Interest, if the TSCTSF determines that the UE is impacted for ASTI service, the TSCTSF performs steps 15-16 in clause 4.15.9.4 to notify the AF the acceptance criteria result.

Upon reception of notification for the UE presence in the Area of Interest, if the TSCTSF determines that the UE is impacted for PTP service, the TSCTSF performs steps 9-11 in clause 4.15.9.3.2 to notify the AF the acceptance criteria result.

>>>> Next Change <<<<

##### 5.2.2.2.17	Namf_Communication_NonUeN2MessageTransfer service operation

Service operation name: Namf_Communication_NonUeN2MessageTransfer

Description: NF Service Consumer requests to transfer a non-UE specific message to NG-RAN node(s) via N2.

Input, Required: N2 Message Container.

Input, Optional: TAI List, RAT Selector (ng-eNB or gNB), Global RAN Node List, Send Write-Replace-Warning-Indication, Send Stop-Warning-Indication, RAN timing synchronization status indication.

Output, Required: N2 Information Transfer Result.

Output, Optional: PWS Result Data.

PWS Result Data is included when parts of a PWS received message have not been comprehended or were missing, or if the message contained logical errors. It may contain the Unknown Tracking Area List which may be present in the associated PWS response message.

>>>> Next Change <<<<

##### 5.2.2.2.18	Namf_Communication_NonUeN2InfoSubscribe service operation

Service operation name: Namf_Communication_NonUeN2InfoSubscribe

Description: The NF Service Consumer invokes this service operation to subscribe to the delivery of non-UE specific information from the NG-RAN node sent via N2 to the AMF.

Input, Required: Notification Target Address, Notification Correlation ID, N2 information type to be subscribed.

Input, Optional: Timing synchronization status reporting indication, TAI(s), NG-RAN node identifier(s).

Output, Required: Subscription Correlation ID.

Output, Optional: None.

>>>> End of Changes <<<<