SA WG2 Meeting #S2-160	S2- 2312528

13th – 17th November, 2023, Chicago, USA

# Title:				Discussion on PDN selection for URSP delivery

# Source:				Ericsson

# Document for:				Discussion

Agenda Item:				9.25.2

Work Item / Release:	eUEPO

Abstract:	Discussion about the selection of PDN connection for URSP delivery.

# Discussion

This is a discussion about the procedure for selecting the PDN connection to use for URSP delivery, for both the UE and the PCF for a PDU session for the purpose of delivery of UE Policy Containers.

Based on current specification the UE only initiates the negotiation of URSP Provisioning Support in EPS with the SMF+PGW-C during the first PDN connectivity requests, either the one included during Initial Attach or during the first request for PDN connectivity if the default PDN connection is not established during Initial Attach procedure.

[5.17.8 in TS 23.501 V18.3.0]

During Initial Attach with default PDN connection establishment procedure in EPS, the UE provides the Indication of URSP Provisioning Support in EPS PCO in the PDN connectivity request to SMF+PGW-C. If the SMF+PGW-C supports URSP provisioning in EPS, it provides the Indication of URSP Provisioning Support in EPS PCO to UE in the PDN Connectivity Accept message.

[4.11.0a.5 in TS 23.502 V18.3.0]

To support URSP Provisioning in EPS, during Initial Attach with default PDN connection establishment procedure in EPS, the UE provides the Indication of URSP Provisioning Support in EPS PCO in the PDN connectivity request to SMF+PGW-C. If the SMF+PGW-C supports URSP Provisioning in EPS and ePCO capability, it provides the Indication of URSP Provisioning Support in EPS PCO in the Create Session Response. If the UE receives the Indication of URSP Provisioning Support in EPS PCO from SMF+PGW-C in the PDN Connectivity Accept message, the UE initiates the UE requested bearer resource modification procedure and includes the UE Policy Container ePCO in the Request Bearer Resource Modification message. If the default PDN connection is not established during Initial Attach procedure, the aforemetioned procedure happens during the first request for PDN connectivity. …

Observation #1: The UE only initiates the negotiation of URSP Provisioning Support in EPS with the SMF+PGW-C during the first PDN connectivity request.

In addition, the UE initiates the UE requested bearer resource modification procedure including the UE Policy container in ePCO (UE STATE INDICATION) if the negotiation of URSP Provisioning Support in EPS with the SMF+PGW-C was successful, i.e. upon the UE receives the URSP Provisioning Support in EPS PCO and ePCO support indication from EPC.

[5.17.8 in TS 23.501 V18.3.0]

When the UE receives the URSP Provisioning Support in EPS PCO and ePCO support indication from EPC, then the UE initiates the UE requested bearer resource modification procedure and includes the UE Policy Container ePCO in the Request Bearer Resource Modification message, the UE Policy Container ePCO will be further forwarded by MME to SMF+PGW-C. When the UE Policy Container ePCO is received by SMF+PGW-C, it forwards transparently the UE Policy Container to PCF for the PDU Session, then the PCF for the PDU Session establishes the UE Policy Association with PCF for the UE. The PCF for the UE generates the corresponding URSP rules in a similar way as it is done in 5GS and sends the URSP rules to UE in the UE Policy Container as described in clause 4.11.0a.5 of TS 23.502 [3].

Observation #2: The UE Provides UE Policy Container in ePCO towards the SMF+PGW-C (and this to the PCF for the PDU session) if the negotiation of URSP Provisioning Support in EPS with the SMF+PGW-C during the first PDN connectivity request is successful.

When the UE initially attaches in EPS the trigger for the PCF for the PDU session to establish a UE Policy Association towards the PCF for the UE and check for the need of any URSP update is the reception of UE Policy Container from the SMF+PGW-C.

[4.11.0a.2a.5 in TS 23.502 V18.3.0]

The PCF for the PDU Session determines the need of establishment of a UE Policy Association when:

-	A UE Policy Container is received from the UE during SM Policy Association Update at Initial Attach with default PDN connection establishment to EPC or at first PDN Connectivity Request as described in clause 4.11.0a.2a.2.1; or

-	During 5GS to EPS mobility as described in clause 4.11.0a.2a.2.2.

Note that before the reception of that first UE Policy Container the PCF doesn’t know whether the UE supports URSP provisioning in EPS or not, so the reception of the UE Policy Container is used to get that knowledge.

Observation #3: For UE initial attach in EPS the PCF for the PDU session establishes a UE policy association towards the PCF for the UE upon the reception of UE Policy Container from the SMF+PGW-C.

According to [4.11.0a.5 in TS 23.502 V18.3.0]

To support URSP Provisioning in EPS, during Initial Attach with default PDN connection establishment procedure in EPS, the UE provides the Indication of URSP Provisioning Support in EPS PCO in the PDN connectivity request to SMF+PGW-C. If the SMF+PGW-C supports URSP Provisioning in EPS and ePCO capability, it provides the Indication of URSP Provisioning Support in EPS PCO in the Create Session Response. If the UE receives the Indication of URSP Provisioning Support in EPS PCO from SMF+PGW-C in the PDN Connectivity Accept message, the UE initiates the UE requested bearer resource modification procedure and includes the UE Policy Container ePCO in the Request Bearer Resource Modification message. If the default PDN connection is not established during Initial Attach procedure, the aforemetioned procedure happens during the first request for PDN connectivity. …

Observation #4: The negotiation of URSP Provisioning Support in EPS with the SMF+PGW-C fails if either the SMF+PGW-C doesn’t support ePCO or URSP Provisioning in EPS, or the MME doesn’t support ePCO.

ISSUE#1: As consequence of Observation#1-4 if either the MME used by the UE for the Initial Attach in EPS doesn’t support ePCO or the SMF+PGW-C doesn’t support either ePCO or URSP Provisioning in EPS the PCF is not able to deliver URSP rules to the UE in EPS.

When the UE attaches initially in EPS and the PCF for the PDU session has received the first UE Policy Container (UE STATE INDICATION) from the UE over the first PDN connection the PCF for the PDU session may use that PDN connection to deliver to the UE the UE Policy Containers received from the PCF for the UE. The PCF shouldn’t use any other PDN connection for the delivery of URSP since it cannot ensure the SMF+PGW-C on that PDN connection supports URSP provisioning in EPS (due to at least the first UE Policy container was received from the UE over the first PDN connection it is assumed the MME supports ePCO)

[4.11.0a.2a.2.1 in TS 23.502 V18.3.0]

-	Step 4a: The PCF determines that the SM Policy Association is used to provision URSP rules to the UE by checking the UE Policy Container received from SMF+PGW-C in Npcf_SMPolicyControl_Update Request.

Observation #5: When the UE attaches initially in EPS, the PCF for the PDU session uses the same PDN connection in which the first UE Policy Container (UE STATE INDICATION) has been received to deliver UE Policy Containers from the PCF for the UE.

ISSUE#2: As consequence of Observation#5, if the PDN connection in which the UE Policy Container (UE STATE INDICATION) was received from the UE is terminated or lost the PCF is not able to deliver URSP rules to the UE in EPS any longer.

For a network deployment that supports EPC interworking it is assumed all MMEs supports ePCO, since ePCO is required for the EPC interworking solution. Then for the scenario of 5GS to EPS handover for a UE it is always assumed the MME supports ePCO.

For the scenario of EPS initial attach, ePCO support can be assumed for the MME if the network deployment supports EPC interworking.

Observation #6: For the scenario of EPS initial attach, the support of ePCO in MME can be assumed if the network deployment supports EPC interworking. For the scenario of 5GS to EPS handover the MME always supports ePCO.

For the scenario of 5GS to EPS handover the PCF for the PDU session triggers the establishment of the UE Policy association towards the PCF for the UE upon the reception of a SM Policy association update from the SMF+PGW-C notyfing of a change in RAT and Access-Type and after checking the UE supports URSP provisioning in EPS (by querying indication stored in UE Context Policy Subscription data in UDR).

[4.11.0a.2a.2.2in TS 23.502 V18.3.0]

In addition to step 4 in clause 4.16.5.1, the PCF determines that 5GS to EPS or EPS to 5GS mobility applies by checking a change in RAT and Access-Type. If 5GS to EPS mobility applies steps 4a to 4d below are executed; if EPS to 5GS applies the PCF for the PDU Session checks if a UE Policy association was created during 5GS to EPS mobility and in that case, it requests the termination of the UE Policy association as described in clause 4.11.0a.2a.8; otherwise step 5 in clause 4.16.5.1 is executed.

-	Step 4a: For non-roaming and Home Routed roaming the (H-)PCF for the PDU Session determines whether the UE supports provisioning of URSP Rules in EPC by checking the indication that is stored within the UE context Policy Subscription data in UDR. If the UE supports provisioning of URSP Rules in EPC the flow continues in step 4b if a UE Policy Association towards the PCF for the UE is not already established.

However, for this scenario the procedure for the PCF for the PDU session to select a PDN connection for the delivery of URSP is not specified. This is also related with LS sent by CT3 during meeting CT3#130 (C3-234513)

Observation #7: For the scenario of 5GS to EPS handover, the procedure for the PCF for the PDU session to select a PDN connection for the delivery of URSP is not specified.

Observation #8: For a network deployment that doesn’t support EPC interworking, for the scenario of EPS initial attach, after a MME change, if the new MME doesn’t support ePCO, the PCF won’t be able to deliver any URSP update to the UE while the UE is served by the new MME.

Observation #9: For a network deployment that doesn’t support EPC interworking, for the scenario of EPS initial attach with a MME not supporting ePCO, after a MME change, if the new MME supports ePCO but the previous didn’t support it the UE will not initiate any procedure for the negotiation of URSP provisioning support in EPS with the SMF+PGW-C.

.

# Proposal

Proposal#0: It is considered that the provisioning of URSP rules in EPC is mainly required for those network deployments supporting EPC interworking, with the purpose of having consistent URSP across 5GC and EPC. Therefore, and as consequence of Observation#6 it is considered the support of URSP provisioning in EPS only applies to those network deployments supporting EPC interworking, so for both scenarios EPS initial attach in EPC and 5GS to EPS handover it can be assumed the MME always support ePCO.
With that assumption, Observations#8-9 and the scenario where a change of MME implies a change in the support of ePCO in the MME don’t apply for the delivery of URSP in EPS.

Proposal #1: To overcome ISSUE#1 it is proposed that if the negotiation of URSP Provisioning Support in EPS with the SMF+PGW-C fails for the first PDN connection the UE successively retries the procedure for the next PDN connections until the negotiation succeeds for one of the PDN connections. Then, when the UE receives the Indication of URSP Provisioning Support in EPS ePCO from SMF+PGW-C in the PDN Connectivity Accept message for that PDN connection, the UE initiates the UE requested bearer resource modification procedure over that PDN connection and includes the UE Policy Container ePCO in the Request Bearer Resource Modification message.

Proposals#2-#3, are as consequence of Observations#5-#6 and ISSUE#2:

Proposal #2:  It is proposed that during the establishment of every PDN connection, the SMF+PGW-C provides and indication to the PCF about whether that PDN connection can be used or not by the PCF for the PDU session for the delivery of URSP rules in EPS. Taking into accunt Proposal#0 and assuming ePCO support in the MME, the SMF+PGW-C indicates that URSP provisioning is supported in the PDN connection if the SMF+PGW-C supports both ePCO and URSP provisioning support in EPS (it is up to CT3 to determine whether the corresponding supportedFeature can be used for this purpose or a new indication in the Npcf_UEPolicyControl_Create message is needed).

Proposal #3: To overcome ISSUE#2 it is proposed that, for the delivery of UE Policy Containers to the UE, the PCF for the PDU session may use any of the established PDN connections for which the corresponding SMF+PGW-C has indicated that URSP provisioning is supported (in addition to the PDN connection in which the first UE Policy container is received). So, for the scenario of initial attach in EPS, if the initial PDN connection in which the first UE Policy Container (UE STATE INDICATION) is lost or terminated, the PCF for the PDU session still may use any of the remaining PDN connections for the URSP provisioning in EPS.

Proposals#4-#5, are as consequence of Observations#7:

Proposal #4: It is proposed that the SMF+PGW-C indicates at PDU session establishment in 5GS whether, upon 5GS to EPS mobility, the transferred PDN connection can be used or not by the PCF for the PDU session for the delivery of URSP rules in EPS. Considering Observation#6, the SMF+PGW-C indicates that URSP provisioning is supported in the transferred PDN connection if the SMF+PGW-C supports both ePCO and URSP provisioning support in EPS.

Proposal #5: It is proposed that for the delivery of UE Policy Containers to the UE upon 5GS to EPS mobility, the PCF for the PDU session may use any of the transferred PDN connections for which the corresponding SMF+PGW-C has indicated that URSP provisioning in EPS is supported.

It is proposed to agree the CRs S2-2312529 and S2-2312530 based on Proposals#0-#5.