3GPP TSG-SA2 Meeting #160	S2-2312587

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Comments on RAN status for draft SID on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML).

Document for:	Discussion

Agenda Item:	30.1

Abstract: This contribution aims to summarize related RAN conclusions to enable SA2 to have a meaningful discussion of possible WT#1 updates before a reply LS is received.

# 1	Introduction

SA WG2 is discussing, in the context of the draft rel.19 "SID on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML)", the working task #1 as captured in S2-2310034 (endorsed as the baseline for further work at SA2#158):

-	WT#1: AI/ML cross-domain coordination aspects

-	Study enhancements to support AI enabled RAN based on conclusions of the RAN study. The WT will discuss whether and how to support the cross domain (i.e. UE, RAN, 5GC, OAM and AF) collaborative AI/ML mechanisms to support the UE, the RAN, the 5GC and the AF for the aspects described by the work tasks below. The WT will also discuss interaction/coordination with RAN to support the AI enabled RAN framework:

-	WT1.1 – Study enhancements to UE data collection framework. Study whether and how to enhance UE data collection framework to meet requirements for RAN AI support for air interface operation (for RAN). This includes identifying what benefit can be achieved from enhanced UE data collection for 5GC, and the potential impacts on the 5G framework, including potential enhancements to policy control. Regarding the radio related data collected from UE or RAN, e.g, channel status information and beam information, the WT will also discuss the data leakage from the operator's domain which should be avoided.

-	WT1.2 – Study 5GC support for AI/ML model and information sharing with the UE. Study whether (and how) to support model transfer/delivery to the UE according to RAN1/RAN2 considerations, including potential enhancements to policy control. Whether and what entities or functions transfer the AI/ML model or information to the UE will be studied as part of the work. This WT will also discuss the data leakage from the operator's domain which should be avoided.

-	WT1.3: Study whether and how to support the alignment of model identification and model management between SA2 and RAN. Work will be based on the possible requirements defined by RAN1 and RAN2.

-	WT1.4: Study whether and how to support interaction/coordination with RAN3 to support the AI enabled NG-RAN framework (i.e. AI/ML for NG-RAN in Rel-18). Work will be based on possible requirements from RAN3.

-	WT1.5: Study whether and how to consider enhancements to LCS to support AI/ML based Positioning.

NOTE A: The work will not modify the architectural principle that a service-based architecture only applies for 5GC.

NOTE B: Whether SA2 will study WT1 and the content of WT1 will depend on and follow RAN study and conclusions. WT1 and associated TUs will be revised to align to RAN study conclusions, when RAN reaches such conclusions.

NOTE C: Further alignment with SA5 for the AI/ML Functional framework may be required.

NOTE D: security aspects are in the scope of SA3, however architectural aspects related to security enhancements will be discussed in this WT.

NOTE E: The model management will follow the framework as defined by RAN.

As highlighted in Note B, the entire WT#1 depends on RAN study conclusions.

SA2 has sent the outgoing LS S2-2311921 to RAN groups to request feedback, but this LS will only be discussed by those groups in parallel to the present SA2 meeting, and feedback is thus to be expected only late in the week (if at all) or at SA plenary.

This contribution aims to summarize related RAN conclusions to enable SA2 to have a meaningful discussion of possible WT#1 updates before a reply LS is received.

This discussion is split into one section for each sub-WT of WT#1 defined in the SA2 SID.

# 2	Discussion

WT#1 covers aspects of AI/ML LCM that exceed the responsibility of RAN, e.g., between a UE or gNB and 5GC, OAM, or AF.

## 2.1	WT#1.1 Enhancements to UE Data Collection Framework

RAN2#121 made the following agreement about data collection frameworks to be considered, see chairman´s notes.

Proposal 6	 Consider the following existing frameworks as starting points to be considered for data collection: SON & MDT, UE assistance information, RRM measurement reports, CSI reporting framework, LPP Provide location information. FFS whether other frameworks should be discussed.

Observation 1: No mechanism currently exists to transmit non-standardized radio measurement data collected from UEs to an entity outside of RAN, e.g., an ML model training entity, and from the RAN2 perspective, the collection of standardized measurement data is limited to the collection of RSRP, RSRQ, and UE location.

Observation 2: The WT#1.1 description covers UE data collection for the purpose of UE-side or NW-side models.

RAN2#123bis agreed the following regarding the collection of data for the offline training of NW-side models, see chairman´s notes.

5.	Related to OAM-centric data collection for NW-side model training, RAN2 studies the potential impact at on the MDT for connected mode, taking into account RAN1 further inputs/progress.

Observation 3: MDT is being studied in RAN2 for the collection of data for CSI feedback enhancement and beam management use cases.

RAN2#122 made the following agreement (from the report of [AT122][001][AIML18], R2-2306783) on data collection to limit the scope to data collection in RRC_CONNECTED mode.

P6a: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement on the non-connected state can be revisited when needed.

Observation 4: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting).

RAN2#123 agreed to proposals 1 – 3 from R2-2308286, “Report of [Post122][060][AIML] Mapping of functions to physical entities (CMCC)”, which were associated with entity mapping tables for CSI (Channel State Information) compression with a two-sided model, CSI prediction and beam management with a UE-side model, and beam management with a NW-side model. Ignoring FFS items, the following entity mappings were agreed for the data collection part of model training, which is assumed to be offline training.

| Use Case | Mapped Entities |
| --- | --- |
| CSI compression with two-sided model | gNB, OAM, OTT server, UE |
| Beam management with UE-side model | OTT server, UE |
| Beam management with NW-side model | gNB, OAM |

Observation 5: The combined set of agreed termination entities for data collection for AIML-enabled CSI enhancement and beam management enhancement consists of gNB, OAM, OTT server, and UE. Of these, support for data collection at the OAM and OTT server could be relevant to SA2.

RAN2#123bis agreed on the following principles for data collection for the offline training of NW-side models.

- logging is supported

- periodic, event based reporting, on demand report

- The UE memory, processing power, energy consumption, signalling overhead should be taken into account

At least the first and second bullets is relevant to the SA2 work.

Observation 6: RAN2 agreed to support periodic, event-based reporting, and on-demand report for data collection for the offline training of NW-side models.

The draft TR 38.843 states the following about UE-side data collection for CSI feedback enhancement.

- Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc.

-The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side.

Proposal 1: WT#1.1 is required by RAN conclusions.

Data collection for offline training for the CSI feedback enhancement and beam management use cases should focus on RRC_CONNECTED state, support periodic, event-based reporting, and on-demand reporting, support the collection of standardized and non-standardized datatypes, and should support data collection to terminate at the OAM and an OTT server. Coordination with SA5 is required for OAM aspects.

## 2.2	WT#1.2 Model Transfer/Delivery

WT#1.2 is covers “AI/ML model and information sharing”, including model transfer/delivery to the UE, policy control, and “whether and what entities or functions transfer the AI/ML model or information to the UE.”

The discussion in RAN2 has identified 8 options for model transfer/delivery, including control plane and user plane solutions for RAN, 5GC, and the LMF, and user plane solutions for transfer/delivery from an OTT or OAM. draft TR 38.843 contains a table comparing these options in the, but no conclusions were reached, although it seemed in contributions to RAN2#123bis there was support for removing core network as an option for model transfer/delivery

Proposal 2: Postpone WT#1.2 to wait for more progress in RAN.

## 2.3	WT#1.3 Model Identification and Model Management

RAN2 agreements on the model ID have not gone into depth, particularly because the mechanics of model-ID-based and functionality-based LCM have not been finalized.

RAN2#121 agreed on a globally unique model ID.

RAN2 assumes that Model ID is unique “globally”, e.g. in order to manage test certification each retrained version need to be identified.

RAN2#121bis agreed using the model ID to identify a transferred/delivered model and discussed two options on how the model ID could be assigned.

* Model ID can be used to identify model or models for the following LCM purposes:

model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step).

(e.g. for so called “model ID based LCM”)

* If model transfer/delivery is supported, model ID can be used for model transfer/delivery LCM purpose.

* How to achieve globality of the Model ID is FFS.

Initial discussion in RAN2: the following global unique model ID definition directions can be considered as a starting point:

Direction1: Pre-defined/hard-coded global unique model ID

Direction3: Assigned global unique model ID via specific ID management node.

Note: Other global unique model ID definition is not precluded.

Model ID structure, if any, is FFS

Considering that RAN2 has not yet agreed on the full scope of the use of a model ID, e.g., whether it is used outside of model transfer/delivery, and how it is structured or assigned, it seems appropriate to postpone WT#1.3 until RAN2 reaches a better understanding.

Observation 9: RAN2 has not yet agreed on sufficient details of the model ID and its use to involve SA2.

Proposal 3: Postpone WT#1.3 to wait for more progress in RAN.

## 2.4	WT#1.4 Coordination with RAN3

RAN3 has been working on the AI/ML for NG-RAN work item in Rel.18. The current work assumes that no changes to the current architecture are needed. RAN3 agreed in Rel.18 that the existing MDT framework is used as baseline for data collection from the UE. In addition the current work focuses on RAN aspects and no agreement with impact on the core network was taken. Rel.19 discussions for a possible Rel.19 study or work item are still under discussion in RAN. Therefore, RAN3 has today no requirements for SA2 with respect to supporting AI/ML for NG-RAN based on Rel-18 conclusions.

Proposal 4: WT 1.4 is not required. RAN3 has not identified any requirements towards SA2 with respect to supporting AI/ML for NG-RAN.

## 2.5	WT#1.5 Enhancements to Support AI/ML Positioning

Enhancements to support AI/ML positioning are addressed separately from WT#1.1. Similar to data collection to support the training of CSI feedback enhancement and beam management models, there is a gap in transmitting UE measurement data to a training entity, whether that entity trains UE-side or gNB-side models. To that end, enhancements are needed to bring measurements that typically terminate in the LMF to other entities.

RAN2#121 made the following agreement about data collection frameworks to be considered.

Proposal 6	 Consider the following existing frameworks as starting points to be considered for data collection: SON & MDT, UE assistance information, RRM measurement reports, CSI reporting framework, LPP Provide location information. FFS whether other frameworks should be discussed.

Of these frameworks, the only one that carries positioning measurements from a UE is LPP. However, there is no mechanism to carry positioning measurements from the LMF to an entity that would train a UE-side model, which is assumed to be a “UE-side OTT server”.

Observation 10: LPP is the only framework identified for data collection that carries positioning measurements from a UE, and LPP terminates in the LMF.

Observation 11: There is no mechanism to carry UE positioning measurements from an LMF to an entity that would train UE-side models.

RAN2#123 agreed to proposals 4 – 6 from R2-2308286, “Report of [Post122][060][AIML] Mapping of functions to physical entities (CMCC)”, which were associated with entity mapping tables for AIML positioning enhancement for UE-side, LMF-side, and gNB-side models. Ignoring FFS items, the following entity mappings were agreed for the data collection part of model training, which is assumed to be offline training.

| Use Case | Mapped Entities |
| --- | --- |
| Positioning enhancement UE-side model | LMF, OTT server, UE |
| Positioning enhancement LMF-side model | LMF |
| Positioning enhancement gNB-side model | gNB, OAM, LMF |

Observation 12: The combined set of agreed termination entities for data collection for AIML-enabled positioning enhancement consists of gNB, OAM, LMF, OTT server, and UE. Of these, support for data collection at the OAM and OTT server could be relevant to SA2, and access to the data could be relevant for the LMF.

Additional requirements related to the RRC mode, types of reporting, and the support for non-standardized datatypes were covered in Section 2.1, WT#1.1.

Proposal 5: WT#1.5 is required by RAN conclusions.,

Data collection for the positioning use case should focus on RRC_CONNECTED state, support periodic, event-based reporting, and on-demand reporting, support the collection of standardized and non-standardized datatypes, and should support data collection to terminate at the OAM and an OTT server.

# 3	Conclusion

Proposal 1: WT#1.1 is required by RAN conclusions.

Data collection for offline training for the CSI feedback enhancement and beam management use cases should focus on RRC_CONNECTED state, support periodic, event-based reporting, and on-demand reporting, support the collection of standardized and non-standardized datatypes, and should support data collection to terminate at the OAM and an OTT server. Coordination with SA5 is required for OAM aspects.

Proposal 2: Postpone WT#1.2 to wait for more progress in RAN.

Proposal 3: Postpone WT#1.3 to wait for more progress in RAN.

Proposal 4: WT 1.4 is not required. RAN3 has not identified any requirements towards SA2 with respect to supporting AI/ML for NG-RAN.

Proposal 5: WT#1.5 is required by RAN conclusions.

Data collection for the positioning use case should focus on RRC_CONNECTED state, support periodic, event-based reporting, and on-demand reporting, support the collection of standardized and non-standardized datatypes, and should support data collection to terminate at the OAM and an OTT server. Coordination with SA5 is required for OAM aspects.