3GPP TSG-SA WG2 Meeting #160	S2-2312659

# Chicago, US, 13 – 17 November, 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0398 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Layer-2 IDs of End UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN2 has sent a LS to SA2 to ask SA2 whether working assumption that the RRCReconfigurationSidelink message carries both the L2 ID and the Local ID of the peer Remote UE and assumes that the association between the User Info ID and the L2 ID of the peer Remote UE is done at the Remote UE ProSe layer is feasible or not in R2-2311566. RAN2 agreed that, instead of a L2 ID, a Local ID is used as a UE ID in a SRAP header for L2 based U2U relay and two Local IDs are included in the SRAP header to identify source and target Remote UE respectively. In RAN2#123bis, RAN2 also agreed that the RRCReconfigurationSidelink message is used to indicate the Local ID pair from relay UE to source and target Remote UEs respectively. RAN2 further discussed how to link the User Info ID with the Local ID at the remote UE, based on two options that are both feasible from RAN2 perspective, and then RAN2 made a working assumption that the RRCReconfigurationSidelink message carries both the L2 ID and the Local ID of the peer Remote UE and assumes that the association between the User Info ID and the L2 ID of the peer Remote UE is done at the Remote UE ProSe layer. Therefore, RAN2 would ask SA2 if the above working assumption is feasible from SA2 perspective. RAN2 will appreciate timely response from SA2 before TSG-RAN#102. RAN2 intend to implement this working assumption if SA2 indicate it is feasible. First of all, it is considered that this working assumption is feasible in SA2. Then to address this assumption, the L2 ID of the peer End UE is required to be known another End UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The L2 ID of one End UE is sent to another End UE in the link management message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The working assumption of RAN2 is not confirmed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

6.4.3.7.2	Layer-2 link management over PC5 reference point for 5G ProSe Layer-2 UE-to-UE Relay

For the 5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay as described in clause 6.7.2, the description in clause 6.4.3.7.1 applies with the following differences and clarifications:

-	The Direct Communication Request message over the second hop PC5 reference point additionally includes the unicast source Layer-2 ID of the source 5G ProSe End UE as communicated with the 5G ProSe Layer-2 UE-to-UE Relay.

-	The Direct Communication Accept message over the first hop PC5 reference point additionally includes the unicast destiantion Layer-2 ID of the target 5G ProSe End UE as communicated with the 5G ProSe Layer-2 UE-to-UE Relay.

-	The Link Modification Request message over the second hop PC5 reference point additionally includes the unicast source Layer-2 ID of the source 5G ProSe End UE as communicated with the 5G ProSe Layer-2 UE-to-UE Relay, if an existing PC5 link is used between the 5G ProSe UE-to-UE Relay and the target 5G ProSe End UE.

-	The Link Modification Accept message over the first hop PC5 reference point additionally includes the unicast destiantion Layer-2 ID of the target 5G ProSe End UE as communicated with the 5G ProSe Layer-2 UE-to-UE Relay, if an existing PC5 link is used between the source 5G ProSe End UE and the 5G ProSe UE-to-UE Relay.

The message contents over PC5 reference point for unicast mode 5G ProSe Direct Communication as depicted from clause 6.4.3.1 to clause 6.4.3.5 are same for the end-to-end connection between peer 5G ProSe End UEs.

Editor's note:	Whether the LIU between peer Layer-2 End UEs has same message contents as direct PC5 LIU messages is FFS.

* * * * End of Changes * * * *