3GPP TSG-SA WG2 Meeting #160	S2-2312792

# Chicago, US, Nov 13 – 17, 2023		(revision of S2-231xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0400 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | U2U Relay RAN alignment for Layer-2 IDs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN2 LS (S2-2311980/R2-2311566) asks SA2 if the following working assumption is feasible from SA2 perspective. RAN2 further discussed how to link the User Info ID with the Local ID at the remote UE, based on two options that are both feasible from RAN2 perspective, and then RAN2 made a working assumption that the RRCReconfigurationSidelink message carries both the L2 ID and the Local ID of the peer Remote UE and assumes that the association between the User Info ID and the L2 ID of the peer Remote UE is done at the Remote UE ProSe layer. Therefore, RAN2 would ask SA2 if the above working assumption is feasible from SA2 perspective. RAN2 will appreciate timely response from SA2 before TSG-RAN#102. RAN2 intend to implement this working assumption if SA2 indicate it is feasible. The working assumption is feasible, and the following enhancements are to align with the assumption: - The Direct Communication Request message over the second hop PC5 additionally includes Layer-2 ID of source 5G ProSe Layer-2 End UE. - The Direct Communication Accept message over the first hop PC5 additionally includes the Layer-2 ID of target Layer-2 End UE. - Souce/Target Layer-2 End UE keeps the association between User Info ID and Layer-2 ID of the peer End UE, and its ProSe layer passes the Layer-2 ID of the peer End UE to the AS layer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the per-hop PC5-S message to include the Layer-2 ID of End UE to the peer End UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN2 working assumption not feasible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.7.2, 6.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.4.3.7.2	Layer-2 link management over PC5 reference point for 5G ProSe Layer-2 UE-to-UE Relay

For the 5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay as described in clause 6.7.2, the description in clause 6.4.3.7.1 applies with the following differences.

-	The Direct Communication Request message over the second hop PC5 reference point additionally includes the Layer-2 ID of the source 5G ProSe Layer-2 End UE which is obtained by the 5G ProSe Layer-2 UE-to-UE Relay in step 3 of clause 6.7.1.1.

-	The Direct Communication Accept message over the first hop PC5 reference point additionally includes the Layer-2 ID of the target 5G ProSe Layer-2 End UE which is obtained by the 5G ProSe Layer-2 UE-to-UE Relay in step 6 of clause 6.7.1.1.

The message contents over PC5 reference point for unicast mode 5G ProSe Direct Communication as depicted from clause 6.4.3.1 to clause 6.4.3.5 are same for the end-to-end connection between peer 5G ProSe End UEs.

Editor's note:	Whether the LIU between peer Layer-2 End UEs has same message contents as direct PC5 LIU messages is FFS.

# * * * * Second change * * * *

### 6.7.2	5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay

This procedure applies to 5G ProSe Layer-2 UE-to-UE Relay.

Figure 6.7.2-1: 5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay

Service authorization and provisioning has been performed for the 5G ProSe Layer-2 UE-to-UE Relay and the 5G ProSe End UEs as described in clause 6.2 before this procedure.

1.	Model A or Model B 5G ProSe UE-to-UE Relay Discovery as described in clause 6.3.2.4 is performed and a source 5G ProSe End UE selects a suitable 5G ProSe Layer-2 UE-to-UE Relay for the communication with a target 5G ProSe End UE.

2.	The source 5G ProSe End UE decides whether to use an existing PC5 link with the 5G ProSe UE-to-UE Relay for the required service. If an existing PC5 link is used then the Layer-2 link modification procedure as specified in clause 6.4.3.7 is used towards a 5G ProSe UE-to-UE Relay, otherwise a Layer-2 link establishment procedure is used towards a 5G ProSe UE-to-UE Relay.

This procedure is towards the selected 5G ProSe UE-to-UE Relay and for Layer-2 link establishment, the security establishment is performed before step 3 is initiated.

After the source 5G ProSe End UE receives the Layer-2 ID of the target 5G ProSe End UE, it maintains an association between the User Info ID and the Layer-2 ID of the target 5G ProSe End UE, and the ProSe layer passes the Layer-2 ID of the target 5G ProSe End UE to the AS layer.

3.	The 5G ProSe Layer-2 UE-to-UE Relay decides whether to use an existing PC5 link between the 5G ProSe UE-to-UE Relay and the target 5G ProSe End UE for the required service and initiates Layer-2 link establishment procedure or Layer-2 link modification procedure as specified in clause 6.4.3.7 with the target 5G ProSe End UE.

This procedure is performed towards the target 5G ProSe End UE using the unicast Layer-2 ID.

The 5G ProSe Layer-2 UE-to-UE Relay sends a Direct Communication Accept message or Link Modification Accept message to the the source 5G ProSe End UE after step 3 is completed.

After target 5G ProSe End UE receives Layer-2 ID of source 5G ProSe End UE, it maintains an association between the User Info ID and the Layer-2 ID of the source 5G ProSe End UE, and the ProSe layer passes the Layer-2 ID of the source 5G ProSe End UE to the AS layer.

4.	The source 5G ProSe End UE establishes an end-to-end connection for unicast mode communication with the target 5G ProSe End UE as described in clause 6.4.3.7.

The data is transferred between the source 5G ProSe End UE and the target 5G ProSe End UE via the 5G ProSe Layer-2 UE-to-UE Relay. The 5G ProSe Layer-2 UE-to-UE Relay forwards all the data messages between the source 5G ProSe End UE and the target 5G ProSe End UE, as specified in TS 38.300 [12].

# * * * * End of changes * * * *