3GPP TSG-WG SA2 Meeting #160	S2-2312794

Chicago, US, Nov 13 – 17, 2023	(revision of S2-231xxxx)

# Title:	[DRAFT] LS on 5G ProSe UE-to-UE relay discovery with security aspects

# Release:	Rel-18

# Work Item:	5G_ProSe_Ph2

Source:	[Huawei to be] SA2

To:	SA3

Cc:	CT1

Contact Person:

Name:	Steven Wenham

E-mail Address:	steven DOT wenham AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None.

1. Overall Description:

During the 5G ProSe UE-to-UE relay discovery procedure, the 5G ProSe UE-to-UE Relay UE sends the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE to the corresponding discoverer 5G ProSe End UE.

Considering that a) the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE are in the protected direct discovery set that is transparent to the 5G ProSe UE-to-UE Relay UE as specified in TS 33.503 clause 6.1.3.3.3.2, and b) the 5G ProSe UE-to-UE Relay UE may serve more than one discoverer 5G ProSe End UE simultaneously. According to the LS from CT1 (C1-237897), the 5G ProSe UE-to-UE Relay UE can NOT decide the Layer-2 ID of the discoverer 5G ProSe End UE as the User Info ID of the discoverer can NOT be obtained by the Relay.

Meanwhile, the User Info IDs in the Direct Communication Request during the 5G ProSe UE-to-UE Relay Communication with integrated Discovery procedure are not encrypted. Applying a similar mechanism in the UE-to-UE relay discovery, i.e. not encrypting the User Info IDs, will be a feasible and convenient solution with unified behaviour on the Relay.

Question: Is there any security risk if the User Info IDs of discoverer 5G ProSe End UE and discoveree 5G ProSe End UE are not encrypted in the direct discovery set within the UE-to-UE Relay Discovery Solicitation message and the 5G ProSe UE-to-UE Relay Discovery Response message? i.e., taking the User Info IDs out of the protected direct discovery set.

2. Actions:

To SA3 group.

ACTION: 	SA2 kindly asks SA3 to provide answer to the above question.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #160-Ad Hoc-e	22nd – 29th January, 2024			Online

TSG-SA2 Meeting #161		26th February – 1st March, 2024		Athens, GR