3GPP TSG-WG SA2 Meeting #160	S2-2312797

Chicago, US, Nov 13 – 17, 2023	(revision of S2-231xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on comparison between Rel-17 REDCAP UE and Rel-18 eREDCAP UE

Document for:	Discussion

Agenda Item:	9.37

Work Item / Release:	REDCAP_Ph2 / NR_redcap_enh-Core / Rel-18

Abstract: Discussion and analysis for the gap between a Rel-17 REDCAP UE and a Rel-18 eREDCAP UE.

# 1. Introduction

In the LS sent form RAN3, R3-234725/S2-2311938, SA2 is asked to clarify whether there is need to include the ‘eRedCap indication’ in the NGAP INITIAL UE MESSAGE, to enable possible charging and policy control and differentiation of eRedCap devices, as was done in the past releases for Rel-17 RedCap UEs. This paper analyses the capability gap between a Rel-17 REDCAP UE and Rel-18 eREDCAP UE, and whether it is necessary for Core Network to be aware of the difference between them.

# 2. Discussion

For Rel-17 REDCAP, an intention of introducing the REDCAP UE category was to support industrial sensors, video surveillance and wearables scenarios, enabling reduced capability NR devices suitable for these use cases with requirements on low UE complexity and sometimes also on low UE power consumption. The capability of REDCAP UE is as following as defined in TS 38.306 [1]:

-	The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs;

-	The maximum mandatory supported DRB number is 8;

-	The mandatory supported PDCP SN length is 12 bits while 18 bits being optional;

-	The mandatory supported RLC AM SN length is 12 bits while 18 bits being optional;

-	For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported; for FR2, either 1 or 2 DL MIMO layers can be supported, while 2 Rx branches are always supported. For FR1 and FR2, UE features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branch or more than 1 UL MIMO layer are not supported by RedCap UEs;

-	CA, MR-DC, DAPS, CPAC and IAB (i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs.

To further expand the market for RedCap use cases with relatively low cost, low energy consumption, and low data rate requirements, e.g., industrial wireless sensor network use cases, some further complexity reduction enhancements were considered. The objectives of Rel-18 eREDCAP WI are as following [2]:

- Power saving/energy efficiency enhancements

- Enhanced eDRX in RRC_INACTIVE (>10.24s)

- Complexity/cost reduction

- Further reduced UE complexity in FR1

  - UE BB bandwidth reduction

    - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL

    - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth.

    - Support additional separate early indication(s)

  - UE peak data rate reduction

    - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction

    - The relaxed constraint is, e.g., 1 (instead of 4).

    - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap.

  - Both 15 kHz SCS and 30 kHz SCS are supported.

Except for the enhanced eDRX longer than 10.24s, the other features are not visible to the Core Network without and don’t have any impact on SA2. For the enhanced eDRX longer than 10.24s, the CN based MT Communication handling is triggered by the request from the gNB and there is no need for the Core Network to understand whether this is for REDCAP UE or eREDCAP UE.

A further gap analysis for the capabilities of REDCAP and eREDCAP is presented in the endorsed RAN1 UE feature list for Rel-18 NR eREDCAP after RAN1#114bis as following [3]:

-	The following components are the same as for Rel-17 REDCAP UE:

1. Maximum FR1 RedCap UE bandwidth is 20 MHz.

3. Early indication of RedCap UE in Msg.1 for 4-step RACH

4. Separate initial UL BWP for RedCap UEs

- It includes the configuration(s) needed for RedCap UE to perform random access

- Enabling/disabling of frequency hopping for common PUCCH resources

5. Separate initial DL BWP for RedCap UEs

- It includes CSS/CORESET for random access

- For separate initial DL BWP used for paging, CD-SSB is included

- For separate initial DL BWP only used for RACH, SSB may or may not be included

- For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included

6. 1 UE-specific RRC configured DL BWP per carrier

7. 1 UE-specific RRC configured UL BWP per carrier

8. RRC reconfiguration of any parameters related to BWP

9. UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB

10. NCD-SSB based measurements in RRC-configured DL BWP

-	The following components are new compared to Rel-17 REDCAP UE for a Rel-18 eREDCAP UE with reduced baseband bandwidth in FR1:

1. DL/UL peak data rate target of 10 Mbps corresponding to vLayers·Qm·f = 3.2

2. Maximum number of PDSCH/PUSCH PRBs that can be scheduled/configured for unicast is 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS

3. Relaxed processing timeline of 1/0.5 ms for 15/30 kHz SCS when the RAR PDSCH and MsgB PDSCH (if supported) is larger than 25/12 PRBs for 15/30 kHz SCS

4. Network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs

5. Maximum number of Msg4 PDSCH PRBs that can be [decoded/scheduled] and maximum number of Msg 3 PUSCH PRBs and Msg A PUSCH PRBs (if supported) that can be [transmitted/scheduled] is 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS

Editor’s Note: FFS whether to add additional components

-	For Rel-18 eREDCAP UE without reduced baseband bandwidth in FR1, the difference compared to the above is as following:

1. The 5th capability in the above does not apply any more.

2. For the DL/UL peak data rate target of 10Mbps in the 1st capability in the above should be corresponding to vLayers·Qm·f = 0.75 when vLayers = 1 and vLayers·Qm·f = 0.8 when vLayers = 2.

In summary, based on the analysis and the capability comparison between the Rel-17 REDCAP UE and the Rel-18 eREDCAP UE. There is no need for 5GC to recognize the difference between Rel-17 REDCAP UE and Rel-18 eREDCAP.It’s proposed to not introduce the eREDCAP indication in the NG interface.

# 3. Conclusion and proposal(s)

It’s proposed to not introduce the eREDCAP indication in the NG interface and agree the LS Reply in S2-2312796.

# 4. References

[1] 3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[2] RP-223544, Revised WID on Enhanced support of reduced capability NR devices.

[3] R1-2310635, Updated RAN1 UE features list for Rel-18 NR after RAN1#114bis.