3GPP TSG-SA WG2#160	            S2-2312893

Chicago, USA, 13-17 November, 2023

Source:	Nokia, Nokia Shanghai Bell

Title: 	KI#3, KI#5, Issues with AoI reporting to enforce DL data restrictions

Agenda item:	9.11.2

WI/Rel:	eNS_Ph3/Rel-18

Document for:	Discussion

# Introduction

At SA2#158 the following CR was agreed, including several Editor's notes that need resolution.

| S2-2309754 | 23.501 CR4857 (Rel-18, 'F'): Downlink packet handling for UE when outside of slice support area |
| --- | --- |

This paper addresses these editor's notes and also identifies issues with the proposed approach.

It is our view the CR has conflated two separate issues:

- How to control DL data transmission to minimize failed SMF initiated Service Requests (the original scope of the CR as per title and reason for change).

- How to decide at SMF whether to release or deactivate a PDU session (not really the original intention or the CR) when the S-NSSAI is partially allowed or supported in a set of cells not matching deployed TAs, as per the text here below in the CR quoted above:

If the AMF determines a PDU Session is associated with S-NSSAI present in the Partially Allowed NSSAI, the AMF indicates to the SMF that the PDU Session is subject to area restrictions for the S-NSSAI. As a result, the SMF subscribes to "UE mobility event notification" for reporting UE presence in Area of Interest by providing S-NSSAI to the AMF as described in clauses 5.6.11 and 5.3.4.4.

For the second aspect we believe the proposed approach is unnecessary and actually not correct as the simple reporting of IN/OUT of a AoI cannot be the basis of decision to release a PDU session or to deactivate it, as what really matters is knowing the S-NSSAI is Allowed or Partially allowed and this is a RM condition which is time varying, or even the fact a S-NSSAI is associated to a NS-AoS in the RA that is at sub TA level: in both cases the condition may change when the UE moves to the next RA and in the next RA the UE may still be inside the AoI and actually become fully Allowed if it was partially Allowed. Hence, we cannot assume that a S-NSSAI is permanently Partially Allowed or supported partially is some cells of TAs of the RA other than within the same RA.

Let's consider for instance S-NSSAI 1 in figure 1:

Initially S-NSSAI is partially allowed in RA 1(TA 1,TA 2), so the SMF receives indication from the AMF it should be subject to AoI reporting as it is subject to Area Restriction. Then, as the UE moves in connected mode to TA3, S-NSSAI 1 becomes fully Allowed in RA 2 = TA3, but the SMF is unaware of that. Then, upon continuing the connected mode mobility, S-NSSAI 1 is Rejected in RA in RA3 =TA4 and again the SMF does not know the RM status of the S-NSSAI. If the SMF based on the text above decides to retain the PDU session even out of NS-AoS as it still believes it is Partially Allowed, it will not release the PDU session when the NG-RAN indicates "S-NSSAI not supported" in the cause of PDU sessions resources release in PDU SESSION RESOURCE NOTIFY SM container or PATH SWITCH REQUEST SM container. Hence the SMF does not release the PDU session for a rejected S-NSSAI despite receiving a cause code in Nsmf_PDUSession_UpdateSMContext from gNB indicating S-NSSAI is not supported. Note that today the AMF is not required to release PDU sessions during Handover when the MRU happens after handover execution as the handover procedure itself takes care of releasing the not retained PDU sessions. This new behavior would require the additional effort at AMF even for UEs in connected mode. This is not efficient and not backwards compatible.

OBSERVATION 1: the knowledge of whether the S-NSSAI is IN or OUT is not sufficient for the SMF to take the correct PDU session handling decision.

So, we propose that it is required to indicate to SMF from gNB a cause code "Partially allowed S-NSSAI not supported" when it releases the resources of the PDU session so that the SMF can have a different behaviour than releasing the PDU session when it receives it. The AoI reporting is not sufficient for this purpose and in any case it is wasteful as the SMF can receive a suitable cause code from the NG-RAN without activation of AoI reporting.

Proposal 1: if the PDU session is part of a Partially Allowed NSSAI when the NG-RAN releases the PDU session resources by either a PDU SESSION NOTIFY or PATH SWITH REQUEST message it provides an indication "Partially allowed S-NSSAI is not supported".

The CR initial scope should therefore be retained and is only about restriction of DL data for established PDU sessions. It is our view that for this purpose we need to enable the DL data restriction feature only if there is evidence a PDU sessions does have DL data or the operator configures the SMF to request this behavior. But this decision cannot be AMF-based as it is specific to the particular PDU session rather than the slice (for the same slice there can be DNNs experiencing MT data and some DNNs not experiencing MT data). Furthermore, this should be a feature independent from KI#5,3 and apply to any PDU session that may be restricted. For this reason, the trigger for the SMF to require AoI reporting should be relying on either a local trigger at SMF based on e.g. detection of a first DL data packed causing a MT-SR, or by a static configuration per DNN if the operator knows there can be a significant MT-SR transactions for a DNN (e.g. the operator collects statistics and detects that a specific DNN causes issues, then it decides to resort to this feature). So it should be a separate feature which of course can be leveraged also to restrict DL sessions that may be associated (at some point in time) with a S-NSSAI in a Partially Allowed NSSAI or be in a RA where the NS-AoS is a subset of the cells of the RA.

# Conclusion

It is recommended that the changes in CR S2-2309754 are modified to define a generic features for DL data restriction based on AoI and CR S2-2312949 is approved. Additionally, separate CR in S2-2312919 and S2-2312896 shall be approved introducing a cause code allowing the SMF to take informed decision for the use cases in KI#3 and KI#5.