3GPP TSG-WG SA2 Meeting #160	S2-2312894

November 13 - 17, 2023, Chicago, US	(revision of S2-231xxxx)

Title:	[DRAFT] Reply LS on Provisioning separate DL and UL PDU Set QoS Parameters to NG-RAN

Response to:	R3-235890 / S2-2311967

Release:	Release 18

Work Item:	NR_XR_enh-Core

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact Person:

#### Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks RAN3 for their LS (R3-235890 / S2-2311967) on provisioning separate DL and UL PDU Set QoS Parameters to NG-RAN.

SA2 has would like to provide the following feedback:

-	SA2 is of the opinion that the UL and DL PDU Set-level QoS parameters should be the same.

-	At the same time, SA2 is aware that the PDU Set-level QoS in the UL may not be met if the UE does not support UL PDU Set handling.

-	SA2 understands (in reference to the running draft CRs to 38.306 and 38.331 in R2-2310149 and R2-2310150, respectively) that the UE can implicitly indicate overall support for UL PDU Set handling via the three new indications (pdu-SetDiscard-r18, psi-BasedDiscard-r18, xr-AssistanceInfo-r18), as well as dynamically indicate UE’s awareness/ visibility of PDU Sets via a new indication provided in the RRC UE Assistance Information message.

-	For a QoS Flow that requires PDU Set-level handling, SA2 is of the opinion that NG-RAN should report to 5GC when it becomes aware that the UE is not able to identify PDU Sets in the uplink, either because the UE does not support UL PDU Set-level QoS handling at all (as per the three new indication) or because the UE has no visibility of PDU Sets for other reasons (as per the new indication in the RRC UE Assistance Information message).

2. Actions:

To RAN3:

ACTION: 	SA2 kindly asks RAN3 to take the information above into account.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#160-AH			22 - 29 Jan 2024			Electronic