3GPP TSG-WG SA2 Meeting #160	S2-2312977

# 13 – 17 Nov, 2023, Chicago, US		(revision of S2-2309347 rev of 7339)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4677 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Non-3GPP QoS Assistance Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - As in TS 24.501, when SMF provides the QoS flow descritions to UE which contains the following information: - 6.2.5.1.1.4 QoS flow descriptions The network can also provide the UE with one or more QoS flow descriptions associated with a PDU session at the PDU session establishment or at the PDU session modification. Each QoS flow description contains: a) a QoS flow identifier (QFI); b) if the flow is a GBR QoS flow: 1) Guaranteed flow bit rate (GFBR) for UL; 2) Guaranteed flow bit rate (GFBR) for DL; 3) Maximum flow bit rate (MFBR) for UL; 4) Maximum flow bit rate (MFBR) for DL; and 5) optionally averaging window, applicable for both UL and DL; c) 5QI, if the QFI is not the same as the 5QI of the QoS flow identified by the QFI; and d) optionally, an EPS bearer identity (EBI) if the QoS flow can be mapped to an EPS bearer as specified in subclause 4.11.1 of 3GPP TS 23.502 [9]. Proposal 1, to remove the duplicated parameter GFBR/MFBR. Because the QoS Flow description above already includes the GFBR/MFBR, so no necessary to include these two parameters in the N3QAI. Proposal 2, to remove the duplicated parameter QoS characteristics. Because when 5QI is provided to UE, the UE knows the QoS characteristics regarding to this 5QI, also not necessary to include the QoS characteristics in N3QAI Proposal 3: to remove the Noticiation control Because Notification control is required/used by NG-RAN, when NG-RAN decides that the GFBR is no longer guaranteed, then notify to SMF. It doesn’t know how to do with UE, and currently there is no mechanism for UE to use such Notification control. #### 5.7.2.4 Notification control ##### 5.7.2.4.1 General The QoS Parameter Notification control indicates whether notifications are requested from the NG-RAN when the "GFBR can no longer (or can again) be guaranteed" for a QoS Flow during the lifetime of the QoS Flow. Notification control may be used for a GBR QoS Flow if the application traffic is able to adapt to the change in the QoS (e.g. if the AF is capable to trigger rate adaptation). The SMF shall only enable Notification control when the QoS Notification Control parameter is set in the PCC rule (received from the PCF) that is bound to the QoS Flow. The Notification control parameter is signalled to the NG-RAN as part of the QoS profile. Proposal 4: add clarification for “Maximum Packet Loss Rate” as following “…Maximum Packet Loss Rate (in this release, only for a GBR QoS Flow belonging to voice media as defined in clause 5.7.2.8)” Because, As in TS 23.501, Maximum Packet Loss Rate is only defined for GBR QoS Flow belonging to voice media. #### 5.7.2.8 Maximum Packet Loss Rate The Maximum Packet Loss Rate (UL, DL) indicates the maximum rate for lost packets of the QoS Flow that can be tolerated in the uplink and downlink direction. This is provided to the QoS Flow if it is compliant to the GFBR NOTE: In this Release of the specification, the Maximum Packet Loss Rate (UL, DL) can only be provided for a GBR QoS Flow belonging to voice media. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To remove the duplicated parameters “QoS characteristics” and “GFBR/MFBR” - To clarify that Maximum Packet Loss Rate with “in this release, only for a GBR QoS Flow belonging to voice media as defined in clause 5.7.2.8) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Non-3GPP QoS Assistance Information doesn’t work correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.44.3.3	Non-3GPP QoS Assistance Information

QoS experienced by PINEs connected behind a PEGC depends on the end-to-end path between a PINE and the application server, i.e. depends on the QoS differentiation in both the 3GPP network and the non-3GPP network attached to the PEGC. Non-3GPP QoS Assistance Information (N3QAI) enables the PEGC to perform QoS differentiation for the PINEs in the non-3GPP network behind the PEGC.

During PDU session establishment and PDU session modification, if the SMF provides the PEGC with QoS flow descriptions, the SMF may additionally signal N3QAI for each QoS flow to the PEGC based on the (DNN, S-NSSAI) combination of the PDU Session. Based on the N3QAI together with QoS rule information, the PEGC may reserve resources in the non-3GPP network. N3QAI consists of the following QoS information:, Maximum Packet Loss Rate (in this release, only for a GBR QoS Flow belonging to voice media as defined in clause 5.7.2.8).

How to enforce QoS based on the N3QAI in the non-3GPP network is considered outside the scope of 3GPP.

# * * * * End of changes * * * *