3GPP TSG-SA2 Meeting #160	S2-2313014

Chicago, US, October 13 – 17, 2023	(revision of S2-2311015)

# Source:	Ericsson

# Title:	New WID on TEI19_TPlmnNfSel; (Re)selection of target NF in target PLMN

# Document for:	Information

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: (Re)selection of target NF in target PLMN

Acronym: TEI19_TPlmnNfSel

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a Feature

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  | {optional free text} |

# 3	Justification

3GPP has defined indirect communication via the SCP, allowing for different levels of delegation from the source NF (acting as client) to the SCP, of the selection and reselection of the target NF (acting as a server).

NOTE: NF consumer is a client when sending requests or subscriptions and server when receiving notifications, NF producer is a server when receiving requests or subscriptions and client when sending notifications.

That is, the selection or reselection of the target NF (server) may be performed in the SCP instead of in the source NF (client).

Delegation of (re)selection functionality from NF to SCP provides some advantages as follows:

- It allows that selection is performed closer to the target destination, which provides in some cases a more up to date information about target health and status.

- It allows that reselection is performed closer to the target destination, that minimizes signalling path for the retry.

- Selection and/or reselection is centralized in the SCP, which may enables more consistent (re)selection policies.

In current specifications, indirect communication is defined only for intra-PLMN interactions, this implies that delegation of (re)selection functionality  is not possible when the target NF (server) is in a different PLMN than the source NF (client). Therefore, the advantages of delegation of (re)selection functionality is missing for inter-PLMN interaction, precisely when the proximity to the target NF (server)  is crucial to have health/status accurate information and to avoid long signalling paths (inter-PLMN paths) when reselection is required (e.g. in case of target NF (server) failure or overload).

Therefore, if (re)selection of target NF (server) is enabled to be performed in the target PLMN, the advantages listed above are applicable as long as the (re)selection is done in the target PLMN, and in addition: to enable the operator of the target PLMN to deploy its own (re)selection policies, independently from NF (re)selection policies in other PLMN, including the possibility to use operator specific attributes

# 4	Objective

The objective is to enable the (re)selection of the target NF (acting as a server) in the target PLMN indepently of the communication model (see TS 23.501, Annex E) used in source PLMN. For example in a cross PLMN, a service request by source PLMN knowing the target PLMN communication model is able to  adapt the service request to meet the communicaiton model of the target PLMN.

This work item will require 0.5 TU(s) to discuss and agree on the needed CRs.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Updates for NF discovery and selection by target PLMN. | TSG#104 |  |
| TS 23.502 | Update for NF discovery and selection by target PLMN. | TSG#104 |  |

# 6	Work item Rapporteur(s)

Hallenstål, Magnus, Ericsson, Magnus (dot) L (dot) Hallenstal (at) ericsson (dot) com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Ericsson