3GPP SA WG2 Meeting #160	S2-2313028

# 13-17 November 2023, Chicago, IL, USA	(was S2-230xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4653 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction the procedure for revoking a Multi-member AF session with required QoS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the Multi-member AF session revoking procedure, NEF sends the Npcf_PolicyAuthorization_Delete only include the Identification of the application session context, there is no need to include the UE address to PCF, the current description is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - remove NEF provides the UE address and the received parameters to PCF in the Npcf_PolicyAuthorization_Delete request in clause 4.15.6.13.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.13.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

##### 4.15.6.13.4	Procedure for Revoking a Multi-member AF session with required QoS

Figure 4.15.6.13.4-1: Procedure for revoking a Multi-member AF session with required QoS

1.	The AF sends a request to revoke the allocated resources for the traffic flows for the communication between a set of UEs and an AF, using Nnef_AFsessionWithQoS_Revoke request message (Transaction Reference ID).

2.	The NEF authorizes the AF request.

3.	The NEF sends the Npcf_PolicyAuthorization_Delete request to PCF as described in clause 5.2.5.3.4 excluding TSCTSF related info for the corresponding UE.

4.	The PCF requests SMF to proceed the SM Policy Association Modifications procedure as defined in clause 4.16.5.2.

5.	The PCF sends the Npcf_PolicyAuthorization_Delete response message to the NEF.

6.	The NEF sends the Nnef_AFsessionWithQoS_Revoke response message (Transaction Reference ID) to the AF.

# >>>>END OF CHANGES<<<<