SA WG2 Meeting #S2-160	S2-2313056

13 - 17 November, 2023, Chicago, USA

Source:	Nokia, Nokia Shanghai Bell (Rapporteur)

Title:	New WID: TEI19_MPQUIC_Enh; Adding support for IP and Ethernet traffic to MPQUIC steering function

Document for:	Information

Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Draft TEI19 WID on adding support for IP and Ethernet traffic to MPQUIC steering function

Acronym:	TEI19_MPQUIC_Enh

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x |  | x |  |
| No | x |  | x |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| x | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| ATSSS_Ph33 | Access Traffic Steering, Switching and Splitting support in the 5G system architecture; Phase 3 | Introduced MPQUIC as new ATSSS steering function in Rel-18. |

# 3	Justification

ATSSS supports traffic steering, splitting, and switching across one 3GPP and one non-3GPP access of the same or different PLMN. In Rel-18 support of MPQUIC as additional steering function was introduced but MPQUIC can only be used for UDP traffic.

IP support for MPQUIC was proposed and studied for Rel-18, see TR 23.700-93 clause 6.14 Solution #14: Proxy based solution using MP-QUIC. ATSSS-LL supports in principle IP and Ethernet traffic but does not provide means for packet reordering and detection of packet out-of-order delivery. For use cases like factory applications having one single ATSSS steering function allowing to steer, split, switch and duplicate IP and Ethernet frames while providing in-built packet reordering and out-of-order delivery detection is beneficial.

Proxying IP in HTTP is described in RFC9484, while proxying Ethernet in HTTP is described in draft-ietf-masque-connect-ethernet-01 which is a MASQUE WG draft.

# 4	Objective

Enhance the existing MPQUIC steering function to enable support for IP and Ethernet traffic. TR 23.700-93 clause 6.14 Solution #14 will serve as baseline.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Enable support for IP and Ethernet traffic in case of MPQUIC steering function. | TSG#104 |  |
| TS 23.502 | Enable support for IP and Ethernet traffic in case of MPQUIC steering function. | TSG#104 |  |

# 6	Work item Rapporteur(s)

Rainer Liebhart, Nokia, rainer(dot)liebhart(at)nokia(dot)com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Not applicable

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai-Bell