3GPP SA WG2 Meeting #160	S2-2313120

# Chicago, U.S., November 13 – 17, 2023

Source: 	Samsung

Title: 	KI#2 : New Solution to Interaction with the DC Application Server for media resource and DC application handling

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This paper proposes solution for KI#2.

1 Introduction

This paper proposes solution for KI#2 in TR 23.700-77 for the FS_NG_RTC_Ph2, how to Interact with the DC Application Server to handle DC media resource and DC application.

2 Proposal

It is proposed to approve following content in TR 23.700-77.

# * * * * First change * * * *(all new texts)

6.X	Solution #X: Interaction with the DC Application Server for media resource and DC application handling

6.X.1	Description

This solution is to enhance the existing application data channel setup signalling procedure and bootstrap data channel setup signalling procedure for considering DC Application Server.

6.X.2	Procedures

6.X.2.1 Bootstrap Data Channel Setup Signalling Procedure

In steps 20-23 corresponding to DC Application download during the Bootstrap Data Channel Setup Signalling procedure, the UEs send application request messages to DCSF through MF or MRF to request a data channel application or an application list, via the established bootstrap data channel with its data channel capabilities.

If DCSF fails to provide proper data channel application or an application list in steps 20-23 during the Bootstrap Data Channel Setup Signalling procedure, additional steps are required as follows.

If DCSF cannot provide proper data channel applications requested by UE #1 and UE #2, the DCSF can request data channel applications from the Data Channel Application Server via MDC3. It is assumed that Data Channel Application Server related information (e.g. URL) has already been provisioned to DCSF when Data Channel Application Server requests subscription to DCSF.

DCSF can download the proper DC application from the Data Channel Application Server and forward it to UE #1 and UE #2 through MF or MRF.

6.X.2.2 Application Data Channel Setup Signalling Procedure

In steps 4 in P2A and P2A2P procedure, after receiving the session event notification, the DCSF determines the policy about how to process the application data channel establishment request based on the related parameters (i.e. associated DC application binding information) in the notification and/or DCSF service specific policy.

If DCSF fails to create appropriate media resources in step 4 in P2A and P2A2P procedure, additional steps are required as follows.

If DCSF cannot create proper media resources in the MF or MRF based on receiving the session event notification from IMS AS, DCSF can request media resources from Data Channel Application Server via DC3/DC4.

DCSF can receive DC media resources related to specific applications that DCSF does not support from the Data Channel Application Server.

After receiving media resources from DC AS, DCSF requests IMS AS to update the MF resource.

6. X.3	Impacts on Existing Nodes and Functionality

This solution may have the following impacts to existing entities and interfaces:

DCSF:

- Support forward HTTP traffic between the MF and DC Application Server via MDC3

- Support downloads specific data channel application from DC Application Server via MDC3.

# * * * * End of change * * * *