SA WG2 Meeting #S2-160	S2-2313178

Chicago, USA, November 13 – 17, 2023

Source:	Meta USA

Title:	New KI: WT#2.2: High Level KI Description for dynamic change (via user plane) in traffic characteristics.

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract of the contribution: resubmit of S2-2311099 which was not opened. This contribution proposes texts for high level description related to WT#2.2 to TR 23.700-70 v0.0.0

# 1.	Discussion

Based on the approved FS_XRM_Ph2 SID, WT#2.2 includes the following highlighted text as description for the study. This pCR proposes some high-level description texts for the TR.

---

WT#2 QoS handling enhancement for XRM services. WT#2.1 Study whether and what enhancements are needed for traffic detection and QoS Flow mapping for different media types multiplexed data flows within a single end-to-end transport connection. WT#2.2 Study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters), provided by the application in the DN. NOTE 2: This will require close coordination between SA4 and SA2. ….

---

# 2.	Text proposal

It is proposed to agree the following changes to TR 23.700-70:

<<all new texts>>

References

[xx]	Authentic Volumetric Avatars from a Phone Scan;  https://dl.acm.org/doi/pdf/10.1145/3528223.3530143.

## 5.x	Key Issue #x: Handling of dynamic change of traffic characteristics of user data

### 5.X.1	Description

Editor’s note: This clause provides a description of the key issue. It's recommended to provide Use cases/scenarios here to support the key issue.

In traditional multimedia communications with 2D video, both the audio and video medias are sent from sender to the receiver. This results in two different QoS flows where the video stream could be in the range of ~1Mb/s while audio in ~200Kb/s. To support good QoE, GBR resource type can be used for these type of media streams since the bit rate is quite predictable and stable over the course of the session.

With the advancement in applying ML in communications (e.g like Volumetric Avatars see [REF xx]), the rendering of the sender image is based on applying e.g., identity and expression code to a universal prior data model at the decoder side. In other words, the downloading of data model to the decider side could be seen as a burst of user data (e.g., 100MB) that happens once in a session while the expression code/identity code (e.g., 100kB) could be seen as low constant data bit rate of user data.

This type of communication data model can also be applied to other AI related application like contextual AI when the downloading of data model could be seen as data burst while contextual related code point (e.g., location, pose) is much smaller.

Using GBR resource type based on data burst size is overkill for the system as most of the data rate requirement for the session is relatively low. Furthermore, commercial application using GBR has not been proven to be successful in the industry so far.

Using non GBR will introduce extra delay while data model is being downloaded during the “data burst” period as this is required to render any output to the user, hence, this type of delay can greatly affect the QoE of the xR application.

Overall, 5GS QoS Framework does not fit well to support dynamic change of traffic characteristics of user data like short term data boost requirement.

The objective of this Key Issue is to study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g. burst related parameters), provided by the application in the DN.

<< End of proposal >>