3GPP TSG-SA WG2 Meeting #160	S2-2313212

# Chicago, USA, November 13 – 17, 2023							      (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0407 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Location Reporting for multi-path L2 U2N Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN3 asked SA2 the following question in their LS S2-2311965/R3-235761: For the Multi-path operation in SL Relay Enhancement WI, RAN3 discussed the aspect on whether the NG-RAN node reports both the cell ID of the direct path and the cell ID of the indirect path to the AMF, in case the direct path and the indirect path connect with different cells of same gNB. RAN3 observed that the NG-RAN node can also report to the AMF the cell identity of PSCell in the dual connectivity. Because it is the 5GC to use the User Location Information, RAN3 would like to request SA2 answer the following question: Question: Whether it is required for NG-RAN node to report both cell IDs of direct path and indirect path to AMF, in case the direct path and the indirect path connect with different cells of same gNB? Similar to dual connectivity, it is proposed to let NG-RAN report both cell IDs of direct path and indirect path to the AMF, in case of multi-path L2 U2N relay communication. In addition, it is proposed to also report an identifier of the Layer-2 U2N Relay UE of the indirect path to further improve the location reporting of the Remote UE. This is also in line with location determination for emergency calls which uses the location of the U2N Relay UE in case Remote UE makes an emergency call via U2N Relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding specification text to clause 6.9.3 about NG-RAN reporting both cell IDs of direct path and indirect path, and an identifier of the Layer-2 U2N Relay UE to AMF, in case of multi-path L2 U2N Relay communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Location Information about the Remote UE that uses multi-path L2 U2N Relay Communication would not be reported properly to the CN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.9.3	Multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay

Editor's note:	Alignment with RAN WGs is to be done.

When a 5G ProSe Layer-2 Remote UE accesses the network via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay for multi-path communication, then NG-RAN shall include the Cell ID information for the direct Uu path, and the Cell ID information for the indirect path as well as an identifier of the 5G ProSe Layer-2 UE-to-Network Relay used for the indirect path, in the NG-RAN Location reporting procedures defined in clause 4.10 in TS 23.502 [5].