SA WG2 Meeting #SA2-160	S2-2313217

Chicago, USA, November 13 – 17, 2023

# Title:	[Draft] Reply LS on Trigger for secure user plane establishment via user plane

# Response to:	LS on Trigger for secure user plane establishment via user plane (S2-2311948/C1-237891)

# Release:	Rel-18

# Work Item:	eLCS_Ph3

Source:	SA2

To:	CT1

Cc:	SA3, CT4

Contact Person:

Name:	Mao Cai

E-mail Address:	simon.cai@nokia-sbell.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR     TS23.273, TS23.502

# Overall description

SA2 thanks CT1 for their LS on Trigger for secure user plane establishment via user plane (S2-2311948/C1-237891).

SA2 has discussed the questions from CT1 and provides the following responses:

For the issue1: LMF knowledge of the UE address is not specified as part of the UP positioning functionality

[SA2 answer] The UE address can be available from parameters or queried by the LMF from the serving SMF. The attached CR contains the details.

For the issue2: It is not clear what method/protocol to use for such LMF to UE user plane trigger/configuration and whether enabling of such trigger requires additional functionality, e.g. control plane signalling.

[SA2 answer]: SA2 expects CT1 to further define the protocols and detailed triggers. SA2 agrees to align with CT1 for the future update.

For the issue3: There is no security protection of LMF to UE signalling via user plane in the UP positioning functionality at this point of the signalling flow.

[SA2 answer]: Step 6 is a notification from LMF to UE, used to establish the user plane connection for user plane positioning. TLS security information and other information needed for secure user plane connection can be preconfigured or cached in the UE. SA2 has an editors’ note for security mechanism and security information provided by LMF to UE in clause 6.18 of TS 23.273. Any security requirements for step 6, if needed, would be handled as part of the resolution of this editors’ note.

2. Actions

To CT1:

ACTION: 	SA2 kindly asks CT1 to take the above information into account.

3. Date of Next SA2 Meetings:

3GPP TSG-SA2 Meeting #160 Ad hoc	22 January- 29 January 2024	Elbonia

3GPP TSG-SA2 Meeting #161	26 February- 1 March 2024	Athens, GR