3GPP TSG-WG SA2 Meeting #160	S2-2313358

# Chicago, USA, November 13 – 17, 2023		Was S2-2312983

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1019 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve EN for end-to-end data volume transfer time analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Verizon, Samsung, Huawei, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The reply LS (S5-235454) from SA5 to SA2’s LS S2-2304067 confirms that SA5 would like to develop and provide within Rel 18 timeframe support for the per UE measurements for the RAN part UL/DL packet delay and UL/DL packet delay between PSA_UPF and UE. Based on that SA5 ongoing work, the following EN for end-to-end data volume transfer time analytics: Editor's note: The inclusion of the following OAM inputs: RAN part delay and Average UL/DL packed delay between PSA UPF and UE is subject to SA WG5 verification. can be replaced with a general note: Note x: The inclusion of the following OAM inputs: per UE RAN part delay and per UE Average UL/DL packet delay between UPF and UE is subject to SA WG5 work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that measurements for the RAN part UL/DL packet delay and UL/DL packet delay between PSA_UPF and UE can be collected from OAM as average input data across all UEs within the available category of choice (e.g. per S-NSSAI) for the end-to-end data volume transfer time analytics. - Replace the Editor’s Note related to SA WG5 verification on RAN part delay and Average UL/DL packed delay between PSA UPF and UE with a general note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Input data collected from OAM for end-to-end data volume transfer time analytics are unclear and Editor’s Note remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

### 6.18.1	General

Clause 6.18 describes how NWDAF can provide E2E data volume transfer time analytics, in the form of statistics or predictions or both, to a service consumer. NWDAF collects E2E data volume transfer time related input data from 5GC NFs, OAM and AF. The consumer can either subscribe to analytics notifications (i.e. a Subscribe-Notify model) or request a single notification (i.e. a Request-Response model).

The E2E data volume transfer time refers to a time delay for completing the transmission of a specific data volume from UE to AF, or from AF to UE. If a target number of repeating data transmissions or a target time interval between data transmissions is given, the E2E data volume transfer time can be provided as an average value of every data volume transfer time within the Analytics target period. The E2E data volume transfer time analytics may be used to assist an AF hosting AI/ML-based services, e.g. for member selection of federated learning.

The E2E data volume transfer time analytics may be provided as defined in clause 6.18.3 for a UE individually or a list of UEs.

More than one E2E data volume transfer time classes might be assigned by operator or AF for a group of UEs. The UEs might be classified to high-, medium- and low-transfer time classes with respect to the threshold(s) of the corresponding class.

The service consumer may be an NF (e.g. AF, or NEF).

The consumer of these analytics  indicates in the request:

-	Analytics ID = "E2E data volume transfer time ".

-	Target of Analytics Reporting: a single UE (SUPI/GPSI) or a group of UEs (a list of SUPIs/GPSIs).

-	Analytics Filter Information optionally including:

-	DNN;

-	S-NSSAI;

-	Application ID;

-	Area of Interest (AOI(s)): restricts the scope of the E2E data volume transfer time analytics to the provided area;

-	An optional list of analytics subsets that are requested (see clause 6.18.3);

-	Optionally, either a target number of repeating data transmissions or a target time interval between data transmissions within the Analytics target period;

-	Optionally, a request for geographical distribution (i.e. the AoIs) of the UEs.

-	 Data Volume UL/DL: indicates a specific data volume transmitted once from UE to AF and/or from AF to UE;

- QoS requirements (e.g. 5QI, QoS Characteristics);

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, preferred level of accuracy of the analytics.

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.18.3).

-	Optional preferred order of results for the list of E2E data volume transfer time:

-	ordering criterion: "E2E data volume transfer time",

-	order: ascending or descending.

-	Optionally, Reporting Thresholds, which indicate conditions on the level to be reached for respective analytics subsets (see clause 6.18.3)

-	Optionally, maximum number of UEs.

* * * Next Change * * * *

### 6.18.2	Input Data

The NWDAF supporting analytics on E2E data volume transfer time shall be able to collect information from AF, OAM and 5GC NFs.

The information collected by the NWDAF from the OAM is defined in the Table 6.18.2-1, from relevant 5GC NFs (i.e. UPF, SMF, AMF) is defined in Table 6.18.2-2 and from AF is defined in Table 6.18.2-3.

Table 6.18.2-1: Input data from OAM related to E2E data volume transfer time

| Information | Source | Description |
| --- | --- | --- |
| RAN part delay for DL and UL | OAM | Average packet transmission delay through the RAN part to the UE per 5QI and per S-NSSAI as captured in clauses 6.3.1.2.1 and 6.3.1.7.1 in TS 28.554 [10]. |
| Timestamp | OAM | A time stamp associated with the collected information. |
| RAN Throughput for DL and UL | OAM (see NOTE 1) | The per UE measurement of the throughput for DL and UL as specified in clauses 5.2.1.1 and 5.4.1.1 of TS 37.320 [20]. |
| RAN Packet delay for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet delay for DL and UL, including per DRB per UE packet delay as specified in clause 5.4.1.1 of TS 37.320 [20]. |
| RAN Packet loss rate for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet loss rate for DL and UL, including the per DRB per UE packet loss rate as specified in clause 5.4.1.1 of TS 37.320 [20]. |
| Average UL/DL packet delay between PSA UPF and UE | OAM | The average of UL/DL packet delay between PSA UPF and UE per S-NSSAI as captured in clauses 5.4.9.1.1 and 5.4.9.2.1 in TS 28.552 [8]. |
| Average DL/UL UE throughput in gNB | OAM | Average DL/UL UE throughput in the gNB per QoS level (mapped 5QI) and per S-NSSAI as specified in clause 5.1.1.3.1 and 5.1.1.3.3 of TS 28.552 [8]. |
| NOTE 1: Per UE measurement for a specific UE from OAM (via MDT) is as captured in clause 6.2.3.1. |  |  |

NOTE x: The inclusion of the following OAM inputs: per UE RAN part delay and per UE Average UL/DL packet delay between UPF and UE is subject to SA WG5 work.

NWDAF subscribes the network data from OAM in the Table 6.18.2-1 by using the services provided by OAM as described in clause 6.2.3.

NOTE 1:	Whether the UE(s) is supporting a Slice or not can be checked by retrieving the registered AMF details from UDM or by asking AMF about what Slice is used by the UE(s) at the current registration (Alternatively, if NSACF is deployed, NSACF can provide a report on what slices are used by the UE(s)).

NOTE 2:	User consent checking from UDM can apply for these analytics.

Table 6.18.2-2: Service Data from 5GC NFs for E2E data volume transfer time analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| UE location | AMF, GMLC | Location of the UE(s) needs to be selected via AMF if the application needs to be started at the same time. If the AoI indicated by the AF is a finer granularity area than the Cell level, the current location of the UE(s) needs to be selected via GMLC instead. |
| UE ID | AMF | (list of) SUPI(s). |
| >5QI | SMF | A reference to 5G QoS characteristics. |
| >QoS flow Packet Delay | SMF, UPF | The observed Packet delay for UL/DL/round trip directions between UE and PSA_UPF. |
| >RAT Type | SMF | The RAT types the UE camps on. |
| >Access Type | SMF | The list of Access Type(s) used for the PDU Session. |

Table 6.18.2-3: Service Data from AF for E2E data volume transfer time analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | AF | Timestamp of the collected information. |
| Application ID | AF | Identifier of the application at the AF. |
| UE ID(s) | AF | Internal or External UE IDs (i.e. SUPI or GPSI, respectively). |
| Transmitted UL/DL data volume | AF | The volume of the transmitted UL/DL data. |
| UL/DL transmission time duration | AF | The time duration (start and end time) needed for sending the volume of UL/DL data. |
| Application Server Instance information | AF | The IP address/FQDN of the Application Server. |

NOTE 3:	How to derive the time delay for sending a specific data volume between UPF and AF on N6 interface is out of scope of the present specification.

For the calculation of the “E2E data volume transfer time” the NWDAF uses mainly the DL/UL Data Volume provided by the consumer and the “Average UE throughput for DL and ULin gNB” of the gNB serving the UE, or the gNBs serving the UEs in the AoI provided by the consumer. Per UE input parameters may also be taken into account by the NWDAF.

* * * End of Changes * * * *