3GPP TSG-WG SA2 Meeting #160	S2-2313481

(revision of  2312543)

# Chicago, USA, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4609 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add missing Npcf_BDTPolicyControl_Get service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Amdocs Software Systems Ltd, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, TEL15 |  |  |  |  |  | Date: |  |  | 2023-11-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Npcf_BDTPolicyControl_Get service operation missing in Section 5.2.5.5. Service operation is missing to retrieve BDT policy from the PCF. AF may want to get latest set of bdt policies stored in PCF via NEF. It can also be useful for operational cases, restoration of session for consumer NF, stale session and similar cases where bdt policy can be retrieved using bdtPolicyId. PCF is the main source of bdt session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Get Operation to Table 5.2.5.1-1 AddingSection 5.2.5.5.5 Npcf_BDTPolicyControl_Get service operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment of BDT Get service operation with TS29554_Npcf_BDTPolicyControl.yaml. If Npcf_BDTPolicyControl_Get service operation procedure is missing, it leads to incomplete spec procedures. Also, it does not allow to define Get operation functionality and usability in th spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

*****FIRST CHANGE *****

#### 5.2.5.1	General

The following table illustrates the PCF Services.

Table 5.2.5.1-1: NF services provided by PCF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Npcf_AMPolicyControl | Create | Request/Response | AMF |
|  | Update | Request/Response | AMF |
|  | UpdateNotify | Subscribe/Notify | AMF |
|  | Delete | Request/Response | AMF |
| Npcf_Policy Authorization | Create | Request/Response | AF, NEF, TSCTSF |
| (NOTE) | Update | Request/Response | AF, NEF, TSCTSF |
|  | Delete | Request/Response | AF, NEF, TSCTSF |
|  | Notify | Subscribe/Notify | AF, NEF, NWDAF, PCF, TSCTSF |
|  | Subscribe |  | AF, NEF, NWDAF, PCF, TSCTSF |
|  | Unsubscribe |  | AF, NEF, NWDAF, PCF, TSCTSF |
| Npcf_SMPolicyControl | Create | Request/Response | SMF |
|  | UpdateNotify | Subscribe/Notify | SMF |
|  | Update | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
| Npcf_BDTPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
|  | Get | Request/Response | NEF |
| Npcf_UEPolicyControl | Create | Request/Response | AMF, PCF |
|  | Update | Request/Response | AMF, PCF |
|  | UpdateNotify | Subscribe/Notify | AMF, PCF |
|  | Delete | Request/Response | AMF, PCF |
| Npcf_EventExposure | Subscribe | Subscribe/Notify | AF, NEF, NWDAF |
|  | Unsubscribe |  |  |
|  | Notify |  |  |
| Npcf_AMPolicyAuthorization | Create | Request/Response | AF, NEF, TSCTSF |
|  | Update | Request/Response | AF, NEF, TSCTSF |
|  | Delete | Request/Response | AF, NEF, TSCTSF |
|  | Notify | Subscribe/Notify | AF, NEF, 5G DDNMF |
|  | Subscribe |  | AF, NEF, 5G DDNMF |
|  | Unsubscribe |  | AF, NEF, 5G DDNMF |
| Npcf_PDTQPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
| NOTE: In the Npcf_PolicyAuthorization operations, PCF is a consumer when the PCF for the UE and the PCF for the PDU session are different. |  |  |  |

***** END OF CHANGES ***

***** SECOND CHANGE *****

##### 5.2.5.5.5	Npcf_BDTPolicyControl_Get service operation

Service operation name: Npcf_BDTPolicyControl_Get

Description: The service operation to retrieve individual BDT policy from the PCF. AF can utilize this API to get current set of bdt policies stored in PCF via NEF. NEF can get one bdtPolicy at a given time. To get set of bdt policies, NEF can loop through the multiple bdt policy ids. Get API can be useful to retrieve bdtPolicy for operational cases, restoration of session for consumer NF, stale session and similar cases. PCF is the main source of bdt session.

Trusted AF can also directly fetch bdtPolicies from PCF.

Inputs, Required: bdtPolicyId (Background data transfer Policy ID string identifying the individual BDT policy resource in the PCF)

Inputs, Optional: None

Outputs, Required: BdtPolicy (Individual BDT Policy Information)

Outputs, Optional: None.

***** END OF CHANGES ***