3GPP TSG-SA WG2 Meeting #160	 S2-2313602

# Chicago, USA, November 13 – 17, 2023												(rev of S2-2308487)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4269 | rev | 3 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | URSP rule delivery via EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | #1 Corrections needed in clause 4.11.0a.2a (Interaction with PCC for URSP delivery via EPS) 1-1: The following sub-clauses do not go together, because PDN connection establishment in EPS will result in SM Policy Association Establishment instead of Modification. 4.11.0a.2a.2 SMF initiated SM Policy Association Modification 4.11.0a.2a.2.1 PDN connection establishment in EPS 1-2 Empty clauses: Clauses 4.11.0a.2a.1 and 4.11.0a.2a.4 do not have any heading and content. 1-3 Misalignment with 23.501 Per clause 5.17.8 of TS 23.501 (see below), the capability of URSP provisioning in EPS is negotiated during Initial Attach and PDN connection establishment, and UE provides UE policy container in UE requested bearer resource modification procedure. ### 5.17.8 URSP Provisioning in EPS When the UE first registers in 5GS, the UE includes the Indication of URSP Provisioning Support in EPS in the UE Policy Container carried in Registration Request. Only when receiving this indication in the UE Policy Container, the PCF will provision the URSP to UE in EPS. During Initial Attach with default PDN connection establishment procedure in EPS, the UE provides the Indication of URSP Provisioning Support in EPS PCO in the PDN connectivity request to SMF+PGW-C. If the SMF+PGW-C supports URSP provisioning in EPS, it provides the Indication of URSP Provisioning Support in EPS PCO to UE in the PDN Connectivity Accept message. NOTE 1: The ePCO capability negotiation between UE and network is done during Attach procedure as defined in TS 24.301 [13]. When the UE receives the URSP Provisioning Support in EPS PCO and ePCO support indication from EPC, then the UE initiates the UE requested bearer resource modification procedure and includes the UE Policy Container ePCO in the Request Bearer Resource Modification message, the UE Policy Container ePCO will be further forwarded by MME to SMF+PGW-C. When the UE Policy Container ePCO is received by SMF+PGW-C, it forwards transparently the UE Policy Container to PCF for the PDU Session, then the PCF for the PDU Session establishes the UE Policy Association with PCF for the UE. The PCF for the UE generates the corresponding URSP rules in a similar way as it is done in 5GS and sends the URSP rules to UE in the UE Policy Container as described in clause 4.11.0a.5 of TS 23.502 [3]. 1-4 Text in 4.11.0a.2a.2.2 (for 5GS and EPS mobility) for UE Policy Association Establishment duplicated with clause 4.11.0a.2a.5 1-5 Duplicated heading in clause 4.11.0a.2a.8 and 4.11.0a.2a.9 4.11.0a.2a.8 UE Policy Association Termination initiated by PCF for PDU session 4.11.0a.2a.9 UE Policy Association termination initiated by the PCF for a PDU Session It is assumed that clause 4.11.0a.2a.9 is intended for the UE Policy Association termination initiated by the PCF for the UE and correction is made accordingly. #2 Impact to EPS procedure “UE requested bearer modification procedure” should be separated from the impact to EPS procedure “PDN Connection Establishment”. #3 Impact to EPS procedure “SMF+PGW-C initiated bearer modification without bearer QoS update” is missing. #4 Miscellaneous corrections and update |  |  |  |  |  |  |  |  |
| Summary of change: |  | General update: Include “NOTE: If the PCF for the PDU Session is the same as the PCF for the UE, the interactions between them are internal to the PCF” in this general sub-clause to avoid the same note in other sub-clauses, e.g., 4.11.0a.2a.2.1, 4.11.0a.2a.2.2, 4.11.0a.2a.2.3, 4.11.0a.2a.9. The enhancement to support URSP deliver in EPS should apply to clause 4.11.0a.2 which refers to clause 4.16.5, therefore add reference to clause 4.16.5. Change “EPC” to “EPS” as the latter refers to EUTRAN+EPC. Other update in different sub-subclauses: 4.11.0a.2a.0: Alignment with TS 23.501 Reshuffle text to improve readability. 4.11.0a.2a.1 voided 4.11.0a.2a.1a, new sub-clause for SM Policy Association Establishment in EPS Capture the impact to support URPS rule delivery in EPS. 4.11.0a.2a.2.1, Change the heading to “SMF initiated SM Policy Association Modification due to UE requested bearer resource modification procedure”. Wording improvement 4.11.0a.2a.2.2 Change the heading to “SMF initiated SM Policy Association Modification at mobility between 5GS and EPS”. For 5GS to EPS mobility, remove duplicated text of UE Policy Association Establishment and provide a reference to the sub-clause “UE Policy Association Establishment”. 4.11.0a.2a.2.3 change the adding to “SMF initiated SM Policy Association Modification for UE reporting URSP update result”. 4.11.0a.2a.3 wording update 4.11.0a.2a.4 voided 4.11.0a.2a.5 Add “from the PCF for the PDU Session” in the heading. Replace “Initial Attach to EPC” and “PDN connectivity” with “UE requested bearer resource modification” to align with TS 23.501 4.11.0a.2a.6, 4.11.0a.2a.7: Mainly editorial 4.11.0a.2a.8 clarify that AMF is replaced with (V-)PCF in step 3 4.11.0a.2a.9 In the heading, change “for a PDU Session” to “for a UE”. Remove the cases of PDN connection termination and EPS to 5GS mobility as they are not applicable. 4.11.0a.5 Move the text about UE requested bearer resource modification to a new sub-clause 4.11.0a. X (new). 4.11.0a.X+1 Add the missing impacted in EPS procedure “SMF+PGW-C initiated bearer modification without bearer QoS update”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete/incorrect specification Misalignment between specifications, unclear input to stage 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.2a.0, 4.11.0a.2a.1 (void), 4.11.0a.2a.1a (new), 4.11.0a.2a.2, 4.11.0a.2a.2.1, 4.11.0a.2a.2.2, 4.11.0a.2a.2.3, 4.11.0a.2a.3, 4.11.0a.2a.4 (void), 4.11.0a.2a.5, 4.11.0a.2a.6, 4.11.0a.2a.7, 4.11.0a.2a.8, 4.11.0a.2a.9, 4.11.0a.5, 4.11.0a.X (new), 4.11.0a.X+1 (new), 4.11.5.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The PCF discovery, using the URSP delivery in EPS capability of the PCF, is defined in 23.501 CR4724. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 4.11.0a.2a	Interaction with PCC for URSP delivery via EPS

##### 4.11.0a.2a.0	General

This clause captures the enhancement to the interaction with PCC to support URSP delivery via EPS.

To Support URSP provisioning in EPS, the SMF+PGW-C receives the Indication of URSP Provisioning Support in EPS PCO and ePCO support indication from the UE, if the SMF+PGW-C also supports URSP Provisioning Support in EPS and ePCO, it selects a PCF which also supports URSP Provisioning in EPS to establish the SM Policy Association based on the received Indication of URSP Provisioning Support in EPS PCO and local configuration.

When the SMF+PGW-C receives the UE Policy Container ePCO in Bearer Resource Command during UE requested bearer resource modification procedure, it forwards transparently the UE Policy Container to PCF for the PDU Session in the Npcf_SMPolicyControl_Update Request including indication that "UE Policy Container received" PCRT was met. When the PCF for Session Management receives UE Policy Container from PCF for the UE, it forwards the UE Policy Container to SMF+PGW-C in Npcf_SMPolicyControl_UpdateNotify Request. The PCF for the PDU Session retrieves the PCRTs for UE Policy from PCF for the UE and subscribe to the applicable PCRTs in EPC to SMF+PGW-C.

When URSP Provisioning is supported in EPS, the PCF for the PDU Session may establish the UE Policy Association with PCF for the UE:

-	During EPS initial attach with default PDN connection establishment procedure or during the first request for PDN connectivity in EPS for a UE when the first UE Policy Container is received from the UE for an SM Policy association and forwards the UE Policy Container to PCF for the UE in Npcf_UEPolicyControl_Create Request.

-	During 5GS to EPS mobility with N26 when the SMF+PGW-C notifies the PCF for the PDU Session about a RAT-Type change via Npcf_SMPolicyControl_Update.

The PCF for the UE triggers the re-evaluation of applicable URSPs in the cases as described in clause 6.1.2.2.3 of TS 23.503 [20]  and determines whether an update of URSP is needed for the UE. The PCF for the UE generates the URSP and sends it to the PCF for the PDU Session in the UE Policy Container via Npcf_UEPolicyControl_UpdateNotify Request.

The procedures for the UE Policy association defined in clauses 4.16.11, 4.16.12 and 4.16.13 apply in EPS with the main differences below:

-	AMF is replaced by the PCF for the PDU Session.

-	Delivery of UE Policy Containers from the PCF for the UE to the PCF for the PDU Session is done via Npcf_UEPolicyControl service instead of via UE Configuration Update procedure.

URSP delivery via EPS is also supported in roaming scenarios, both for a PDN connection established in Home Routed and for a PDN connection established in LBO.

NOTE:	If the PCF for the PDU Session is the same as the PCF for the UE, the interactions between them are internal to the PCF.

##### 4.11.0a.2a.1	Void

##### 4.11.0a.2a.1a	SM Policy Association Establishment in EPS

The following enhancement is applicable to clause 4.11.0a.2 which refers to clause 4.16.4 (SM Policy Association Establishment):

-	Step 1 of clause 4.16.4: If the SMF+PGW-C receives URSP Provisioning Support indication, the SMF+PGW-C selects the PCF based on the URSP delivery in EPS capability of the PCF, as defined in TS 23.501 [2] clause 6.3.7.1.

##### 4.11.0a.2a.2	SMF initiated SM Policy Association Modification

4.11.0a.2a.2.1	 SMF initiated SM Policy Association Modification due to UE requested bearer resource modification procedure

When SMF+PGW-C receives the UE Policy Container in the UE requested requested bearer resource modification procedure, the SMF+PGW-C provides the UE Policy Container to the PCF.

The SM Policy Association Modification procedure in clause 4.16.5.1 applies with the following enhancements is executed with the following differences, applicable between step 4 and step 5:

If the PCF receives both the indication that the PCRT "UE Policy Container received" was met and the UE Policy Container, steps 4a to 4d below are executed

-	Step 4a: The PCF determines that the SM Policy Association is used to provision URSP rules to the UE by checking the UE Policy Container received from SMF+PGW-C..

-	Step 4b: The PCF for the PDU Session discovers the PCF for the UE by querying the BSF, using Nbsf_Management_Discovery for the SUPI . If no result is obtained from the BSF the PCF for the PDU Session selects the PCF for the UE by querying NRF, using the URSP delivery in EPS capability of the PCF, as defined in TS 23.501 [2] clause 6.3.7.1.

-	Step 4c: The PCF for the PDU Session establishes a UE Policy Association towards the PCF for the UE as described in clause 4.11.0a.2a.5.

-	Step 4d: If the PCF for the UE provides any Policy Control Request Trigger parameters in the Npcf_UEPolicyControl Create Response the PCF for the PDU Session takes them into account for the generation of Policy Control Request Triggers to the SMF+PGW-C.

4.11.0a.2a.2.2	SMF initiated SM Policy Association Modification at mobility between 5GS and EPS

During 5GS to EPS mobility procedure, the PCF for the PDU Session establishes a  UE Policy Association towards the PCF for the UE if it exists otherwise, it performs PCF discovery and selection then establishes a UE Policy Association.. During EPS to 5GS mobility procedure the PCF for the PDU Session terminates the UE Policy Association, if established during 5GS to EPS mobility.

The following enhancements are applicable to clause 4.16.5.1 (SMF initiated SM Policy Association Modification procedure):

In addition to step 4 in clause 4.16.5.1, the PCF determines that 5GS to EPS or EPS to 5GS mobility applies by checking a change in RAT and Access-Type.

-	If 5GS to EPS mobility applies,  , in non-roaming and Home Routed roaming the (H-)PCF for the PDU Session determines whether the UE supports provisioning of URSP Rules in EPS by checking indication of support of URSP delivery in EPS in UDR and in LBO roaming the V-PCF for the PDU Session determine whether the UE supports provisioning of URSP rules in EPS based on local configuration, e.g. using the PEI, given that the V-PCF has no access to UDR at the HPLMN. The (H-)PCF or the V-PCF determines if the SMF+PGW-C supports delivery of URSP Rules in EPS as reported by the SMF+PGW-C. If the PCF determines that the UE and the SMF+PGW-C support provisioning of URSP Rules in EPS, the PCF for the PDU Session finds the PCF for the UE using Nbsf_Management_Discovery for the SUPI and then performs UE Policy Association Establishment procedure as specified in clause 4.11.0a.2a.5. If no result is obtained from the BSF, the PCF for the PDU Session selects the PCF for the UE by querying NRF, using the URSP delivery in EPS capability of the PCF, as defined in TS 23.501 [2] clause 6.3.7.1.

-	If EPS to 5GS mobility applies, the PCF for the PDU Session requests the termination of the UE Policy association as described in clause 4.11.0a.2a.8

4.11.0a.2a.2.3	SMF initiated SM Policy Association Modification for UE reporting URSP update result

When the UE reports the result of URSP rule update, the following enhancements are applicable to clause 4.16.5.1 (SM Policy Association Modification procedure):

In addition to step 4 in clause 4.16.5.1 if the PCF for the PDU Session receives the "UE Policy" Policy Control Request Trigger and associated UE Policy Container, steps 4a to 4b below are executed:

-	Step 4a: The PCF for the PDU Session determines that a UE Policy association has been already established towards the PCF for the UE for the SM Policy Association as described in clause 4.11.0a.2a.7.

-	Step 4b: The PCF for the PDU Session forwards the UE Policy Container to the PCF for the UE in Npcf_UEPolicyControl_Update Request.

##### 4.11.0a.2a.3	PCF initiated SM Policy Association Modification

For the purpose of URSP delivery in EPS, PCF initiated SM Policy Association Modification procedure is triggered by an interaction from the PCF for the UE.

The following enhancements are applicable to clause 4.16.5.2 (PCF initiated SM Policy Association Modification procedure):

-	Step 4: The PCF for the PDU Session may receive a UE Policy Container from the PCF for the UE and/or an update in the Policy Control Request Triggers applicable to the UE as described in clause 4.11.0a.2a.6.

-	If a UE Policy Container is received from the PCF for the UE, the PCF for the PDU Session selects one of the ongoing PDU Sessions for this UE that supports URSP Rule delivery in EPS and includes that UE Policy Container in Npcf_SMPolicyControl_UpdateNotify request.

-		If an update in the Policy Control Request Triggers applicable to the UE is received, the PCF for the PDU Session determines whether an update on the current Policy Control Request Triggers needs to be sent to the SMF+PGW-C in Npcf_SMPolicyControl_UpdateNotify request.

##### 4.11.0a.2a.4	Void

##### 4.11.0a.2a.5	UE Policy Association establishment

To support the delivery of URSP rule in EPS as described in clause 6.1.2.2.3 of TS 23.503 [20], the PCF for the PDU Session may establish a UE Policy Association towards the PCF for the UE.

This procedure may be triggered by the following procedures:

1.	UE requested bearer resource modification .

2.	5GS to EPS handover or 5GS to EPS Idle Mode mobility.

Figure 4.11.0a.2a.5-1: UE Policy Association establishment procedure

In the non-roaming case, the V-PCF for the UE is not involved, and the PCF for the PDU Session interacts with the PCF for the UE. For the Home Routed roaming scenario, the V-PCF for the UE is not involved and the H-PCF for the PDU Session interacts with the H-PCF for the UE. For the LBO roaming scenario, the V-PCF for the PDU Session interacts with the V-PCF for the UE and the V-PCF for the UE interacts with the H-PCF for the UE.

The procedure is based on clause 4.16.11.1 for a UE in 5GC with the following differences:

1.	The PCF for the PDU Session determines the need of establishment of a UE Policy Association when:

-	A UE Policy Container is received from the UE during SM Policy Association Update at UE requested bearer resource modification as described in clause 4.11.0a.2a.2.1; or

-	During 5GS to EPS mobility as described in clause 4.11.0a.2a.2.2.

2.	Same as step 2 in clause 4.16.11, with the following exceptions:

The PCF for the PDU Session includes the UE Policy Container in case of SM Policy Association update at UE requested bearer resource modification or 5GS to EPS Mobility indication in case of SM Policy Association update at 5GS to EPS mobility.

For LBO roaming the V-PCF for the PDU Session discovers and selects an instance of the V-PCF for the UE based on the available PCF instances obtained from the NRF or locally configured information in the V-PCF, depending on operator's policies, as described in clause 6.3.7.1 of TS 23.501 [2]. The V-PCF for the PDU Session contacts the V-PCF for the UE and steps 3 and 4 are executed.

For non-roaming or Home Routed roaming the (H-)PCF for the PDU Session contacts the (H-)PCF for the UE and step5 follows.

3.	Same as step 3 in clause 4.16.11.1, with the following exceptions:

The UE Policy Container is received at SM Policy Association instead of initial registration. When the UE Policy Association is being established due to 5GS to EPS mobility as described in clause 4.11.0a.2a.2.2, the V-PCF for a UE in the VPLMN needs to discover and select the same H-PCF for a UE that is serving the UE in the H-PLMN for 5GS by querying the BSF in the HPLMN. This is to be able to recover the information about the list of PSIs in the UE and the subscribed PCRTs from former UE Policy Association for the UE in 5GS as described in step 6. Therefore, the V-PCF for a UE in the VPLMN needs first to discover the BSF in the HPLMN.

NOTE 1:	For Home Routed roaming, the V-PCF for a UE is not involved, therefore in this roaming scenario it is not possible to retrieve the Application guidance on URSP Rule for inbound roamers of the PLMN and provide it to the H-PCF for the UE to be used as input for the generation of the URSP rules for the UE as described in clause 6.6.2.2.3 of TS 23.503 [20].

4.	Same as in clause 4.16.11.1.

5.	Same as in clause 4.16.11.1 step 5, with the following exceptions:

The subscription to notification of N1 message delivery of policy information to the UE using Namf_Communication_N1N2MessageSubscribe service is not applicable.

6.	The Same as step 6 in clause 4.16.11.1, with the following difference:

If "5GS to EPS Mobility" indication was received in Npcf_UEPolicyControl_Create in step 2 instead, the (H-)PCF recovers the information about the list of PSIs for the UE, the subscribed PCRTs in 5GS from former UE Policy Association for the UE and the indication about the support of provisioning of URSP rules in EPS. If the UE does not support the provisioning of URSPs in EPS, the H-PCF rejects the establishment of the UE Policy Association.

NOTE 2:	In LBO roaming the V-PCF for a UE may trigger the establishment of the UE Policy Association for a UE that does not support the provisioning of URSP in EPS, as described in step 4a in clause 4.11.0a.2a.2.2.

The (H-)PCF provides the UE Policy Container in the Npcf_UEPolicyControl_UpdateNotify Request.

7.	Same as in clause 4.16.11.

NOTE 3:	Steps 8 and 9 are executed only in LBO roaming scenario.

8.	The V-PCF for the UE forwards the UE Policy Container to the V-PCF for the PDU Session by invoking Npcf_UEPolicyControl UpdateNotify Request. The V-PCF for the UE checks the size limit as described in clause 6.1.2.2.2 of TS 23.503 [20].

9.	The V-PCF for the PDU Session sends a response to the V-PCF for the UE.

10.	If the UE provides a UE Policy Container with the result of the URSP update to the PCF for the PDU Session as described in clause 4.11.0a.2a.2.3 the PCF for the PDU Session forwards the UE Policy Container to the (V-) PCF for the UE invoking Npcf_UEPolicyControl_Update Request.

11.	The (V-) PCF for the UE sends a response to the (V-) PCF for the PDU Session.

NOTE 4:	Steps 12 and 13 are executed only in LBO roaming.

12.	The V-PCF for the UE forwards the notification response to the H-PCF by invoking Npcf_UEPolicyControl_Update Request.

13.	The H-PCF sends a response to the V-PCF for the UE.

##### 4.11.0a.2a.6	UE Policy Association Modification initiated by the PCF for the UE

The following impacts are applicable to clause 4.16.12.2 (UE Policy Association Modification initiated by the PCF procedure):

In the non-roaming case, the V-PCF is not involved, the AMF is replaced by the PCF for the PDU Session, and the role of the H-PCF is performed by the PCF for the UE.

For the Home Routed roaming scenarios, the V-PCF is not involved, the AMF is replaced by the H-PCF for the PDU Session, and the role of the H-PCF is performed by the H-PCF for the UE.

For the LBO roaming scenarios, the AMF is replaced by the V-PCF for the PDU Session, the role of the V-PCF is performed by the V-PCF for the UE and the role of the H-PCF is performed by the H-PCF for the UE.

-	Steps 1a, 1c, 2a, 2c and 2d: These steps are not applicable for Home Routed roaming, since the V-PCF is not involved.

-	Step 3: The (H-)PCF may create the UE policy container including UE policy information as defined in clause 6.1.2.2.2 of TS 23.503 [20]. The (H-) PCF checks whether the UE Policy Association for the UE is established for a UE in EPS by checking the RAT-Type of the UE Policy Association and in this case the (H-)PCF may send the UE Policy Container in the Npcf_UEPolicyControl_UpdateNotify Request. The (H-)PCF may provide updated PCRTs over the UE policy association.

For Home Routed roaming, the V-PCF for a UE is not involved, therefore the Service Parameters provided by the AF from the VPLMN are not used by the H-PCF as input for the generation of the URSP rules.

NOTE:	Steps 5 and 6 are executed only in LBO roaming.

-	Step 5: In case the UE Policy Association is for a UE in EPS, the (V-) PCF for the UE provides the UE Policy Container and/or policy control triggers to the (V-) PCF for the PDU Session.

-	Step 6: The (V-) PCF for the PDU session sends a response to the (V-) PCF for the UE.

-	Steps 7, 8 and 9 are replaced by steps 10-13 of procedure UE Policy Association Establishment in EPS from clause 4.11.0a.2a.5.

##### 4.11.0a.2a.7	UE Policy Association Modification initiated by the PCF for a PDU Session

This procedure addresses the scenario where a Policy Control Request Trigger condition is met by the PCF for a PDU Session.

The procedure is based on the one defined in clause 4.16.12.1.1 with the following differences:

In the non-roaming case, the V-PCF is not involved, the AMF is replaced by the PCF for the PDU Session, and the role of the H-PCF is performed by the PCF for the UE.

For the Home Routed roaming scenarios, the V-PCF is not involved, the AMF is replaced by the H-PCF for the PDU Session, and the role of the H-PCF is performed by the H-PCF for the UE.

For the LBO roaming scenarios, the AMF is replaced by the V-PCF for the PDU Session, the role of the V-PCF is performed by the V-PCF for the UE and the role of the H-PCF is performed by the H-PCF for the UE.

-	Step 1: Same as in clause 4.16.12.1.1 step 1, with the following exceptions:

In LBO roaming the V-PCF for the PDU Session contacts the V-PCF for the UE and steps 2 and 3 are executed. The V-PCF for the UE contacts with the H-PCF for the UE discovered during the establishment of the UE Policy association as described in clause 4.11.0a.2a.5.

For non-roaming or Home Routed roaming the (H-)PCF for the PDU Session contacts the (H-)PCF for the UE and step 4 follows.

-	Steps 2, 3,4: Same as in clause 4.16.12.1.1.

-	Step 5: The (H-)PCF may create the UE policy container including UE policy information as defined in clause 6.6 of TS 23.503 [20]. The (H-) PCF checks whether the UE Policy Association for the UE is established for a UE in EPS by checking the RAT-Type of the UE Policy Association and in this case the (H-)PCF may send the UE Policy Container in the Npcf_UEPolicyControl UpdateNotify Request.

-	Step 6: Same as in clause 4.16.12.1.1.

-	Steps 7, 8 and 9 are replaced by steps 10-13 of procedure UE Policy Association Establishment in EPS from clause 4.11.0a.2a.5.

##### 4.11.0a.2a.8	UE Policy Association Termination initiated by PCF for PDU session

The following case is considered for UE Policy Association Termination:

1.	UE Detach from the EPS.

2.	PDN connection for UE which is used for delivery of UE Policy Container in EPS is released.

3.	Changing of RAT type from EPS to 5GS, notified to PCF for the PDU Session by SMF+P-GW-C.

Figure 4.11.0a.2a.8-1: UE Policy Association termination in EPS

This procedure applies for non-roaming scenarios, LBO roaming scenarios and Home Routed roaming scenario.

In the non-roaming case, the V-PCF for the UE is not involved, the role of the V-PCF for the PDU session is performed by the PCF for the PDU session and the role of the H-PCF for the UE is performed by the PCF for the UE. For the LBO roaming scenarios, the V-PCF for the PDU session interacts with the V-PCF for the UE and the V-PCF for the UE interacts with the H-PCF for the UE.

In the Home Routed roaming scenario, the V-PCF for the UE is not involved, the role of the V-PCF for the PDU session is performed by the PCF for the PDU session in HPLMN and the role of the H-PCF for the UE is performed by the PCF for the UE in HPLMN.

1.	The (V-)PCF for the PDU session terminates a UE Policy Association with the (V-)PCF for the UE when:

-	The SMF+P-GW-C notifies the changing of RAT type from EPS to 5GS; or

-	The SMF+P-GW-C terminates the SM policy association with (V-)PCF for the PDU session due to UE detached from EPS or the PDN connection for UE which is used for delivery of UE Policy Container in EPS is released.

2.	Same as in clause 4.16.13.1 step 2, replace AMF with (V-)PCF for the PDU session.

3.	Same as in clause 4.16.13.1 step 3, replace AMF with (V-)PCF for the PDU session.

4.	Same as in clause 4.16.13.1 step 4.

5.	Same as in clause 4.16.13.1 step 5.

##### 4.11.0a.2a.9	UE Policy Association termination initiated by the PCF for a UE

The procedure is based on clause 4.16.13.2 (UE Policy Association Termination initiated by the PCF procedure) with the following differences:

In the non-roaming case, the V-PCF is not involved, the AMF is replaced by the PCF for the PDU Session, and the role of the H-PCF is performed by the PCF for the UE.

For the LBO roaming scenarios, the AMF is replaced by the V-PCF for the PDU Session, the role of the V-PCF is performed by the V-PCF for the UE.

In the Home Routed roaming scenario, the V-PCF is not involved. The AMF is replaced by the PCF for PDU Session in HPLMN, and the role of the H-PCF for the UE is performed by the PCF for the UE in HPLMN.

When the UE is in EPS, the UE Policy Association has been established by the (V-) PCF for PDU Session as described in clause 4.11.0a.2a.5.

-	Step 4: Change the AMF to the PCF for PDU Session. The PCF for UE may notify the PCF for PDU Session of the removal of the UE Policy Association via Npcf_UEPolicyControl_UpdateNotify service operation.

-	Step 5: Change the AMF to the PCF for PDU Session.

-	Step 6: Change AMF to the PCF for PDU Session and refer to the step 2 to step 5 in clause 4.11.0a.2a.8.

* * * Next Changes * * *

#### 4.11.0a.5	PDN Connection Establishment

During establishment of non-emergency PDN connection in the EPC, the UE and the SMF+PGW-C exchange information via PCO as described in clause 5.15.7 of TS 23.501 [2]. For UE with 5GC NAS capability disabled (i.e. N1 mode is disabled), the UE may also allocate a PDU Session ID and send it to the SMF+PGW-C via PCO. If the SMF+PGW-C supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, before the SMF+PGW-C provides an S-NSSAI to the UE, the SMF+PGW-C should check such that the selected S-NSSAI is among the UE's subscribed S-NSSAIs and that the S-NSSAI is not subject to Network Slice-Specific Authentication and Authorization, by retrieving the Subscribed S-NSSAI from UDM using the Nudm_SDM_Get service operation (the SMF+PGW-C discovers and selects a UDM as described in clause 6.3.8 of TS 23.501 [2]). If the SMF+PGW-C is in a VPLMN, the SMF+PGW-C uses the Nnssf_NSSelection_Get service operation to retrieve a mapping of the Subscribed S-NSSAIs to Serving PLMN S-NSSAI values. If the S-NSSAIs supported by the SMF+PGW-C are all subject to NSSAA, then the SMF+PGW-C should reject the PDN connection establishment. If the selected S-NSSAI is subject to NSAC and EPS counting is required for the S-NSSAI, the SMF+PGW-C uses the Nnsacf_NSAC_NumberOfUEsUpdate services operation and/or the Nnsacf_NSAC_NumberOfPDUsUpdate services operation to check if the selected S-NSSAI is available as described in clause 4.11.5.9. The SMF+PGW-C uses the Nudm_SDM_Subscribe service operation to subscribe the change of the Session Management Subscription data. If the SMF+PGW-C is notified from UDM with subscription data change, the SMF+ PGW-C takes actions for the PDN connection as described in clause 5.17.2.1 of TS 23.501 [2].

As described in TS 23.548 [74], during establishment of a PDN connection, a UE that hosts EEC(s) may indicate to the SMF+PGW-C, in the PCO, that it supports the ability to receive ECS address(es) via NAS and to transfer the ECS Address(es) to the EEC(s). If the UE indicated in the PCO that it supports the ability to receive ECS address(es) via NAS, the SMF+PGW-C may provide the ECS Address Configuration Information (as described in clause 6.5.2 of TS 23.548 [74]) to the UE in the PCO. The SMF+PGW-C may derive the Edge Configuration Server Information based on local configuration, the UE's location and/or UE subscription information.

The SMF+PGW-C may use the bearer modification procedure without bearer QoS update to send the UE a PCO with updated ECS Address Configuration Information as defined in clause 6.5.2 of TS 23.548 [74] to the UE.

During establishment of non-emergency PDN connection in the EPC, if PGW-C+SMF is selected for a UE that has 5GS subscription, the SMF may be configured to obtain the subscribed IP index from UDM as part of subscription data using the Nudm_SDM_Get service operation (the PGW-C+SMF discovers and selects a UDM as described in clause 6.3.8 of TS 23.501 [2]).

During establishment of non-emergency PDN connection in EPC, if PGW-C+SMF is selected for a UE that has 5GS subscription, the SMF may be configured to obtain the subscribed User Plane Security Policy from UDM as part of subscription data using the Nudm_SDM_Get service operation (the PGW-C+SMF discovers and selects a UDM as described in clause 6.3.8 of TS 23.501 [2]). The SMF uses the subscribed User Plane Security Policy as described in clause 5.10.3 of TS 23.501 [2].

During establishment of non-emergency PDN connection in the EPC, if SMF+PGW-C is selected for a UE that has 5GS subscription but does not support 5GC NAS and is accessing via EPC/E-UTRAN and if the SMF+PGW-C supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, the SMF+PGW-C+PGW-C may proceed as specified in first paragraph of this clause or select any S-NSSAI associated with the APN of the PDN connection. The SMF+PGW-C shall not provide any 5GS related parameters to the UE.

NOTE 1:	The SMF+PGW-C knows that the UE does not support 5GS NAS if the UE does not provide PDU Session ID in PCO (see clause 5.15.7 of TS 23.501 [2]).

During establishment of emergency PDN connection:

-	The SMF+PGW-C is to be derived from the emergency APN or to be statically configured in the Emergency Configuration Data in MME.

-	5GC interworking support with N26 or without N26 is determined based on UE's 5G NAS capability and local configuration (in the Emergency Configuration Data in MME).

-	The S-NSSAI configured for the emergency APN in SMF+PGW-C is not sent to the UE by the SMF+PGW-C. One S-NSSAI is configured for the emergency APN.

During establishment of non-emergency PDN connection and emergency PDN connection, if SMF+PGW-C is selected for a UE that does not support 5GC NAS, the SMF+PGW-C creates unique PDU Session ID for each PDN connection of the UE.

-	The unique PDU Session ID can be created based on the EPS Bearer IDs assigned by the MME for the PDN Connections associated with the UE and not be in the range of PDU Session ID values that can be created by a 5GC NAS capable UE.

-	If handover between EPS and EPC/ePDG (as specified in clause 8.6 of TS 23.402 [26]) is required, the SMF+PGW-C, based on operator configuration, may perform the following to ensure the uniqueness of PDU Session ID:

-	the SMF+PGW-C queries the UDM for any PDU Session ID(s) that are already registered in the UDM for the UE by invoking Nudm_UECM_Get service operation.

-	the SMF+PGW-C creates a PDU session ID that does not collide with the received PDU session ID(s).

-	When the SMF+PGW-C establishes the PDN connection successfully, the SMF+PGW-C performs UDM registration using the Nudm_UECM_Registration service operation.

NOTE 2:	If the scenario that a UE handovers a PDN connection with APN1 established via MME to ePDG and then establishes a second PDN connection with APN2 via MME needs to be supported in a given deployment, then, based on operator configuration, the SMF+PGW-C can query the UDM to avoid that the same PDU Session ID value of first PDN connection would be assigned by the SMF+PGW-C for the second PDN connection.

When the SMF+PGW-C establishes the PDN connection successfully, the SMF+PGW-C provides the ID of the PCF ID selected for the PDN connection in the UDM using the Nudm_UECM_Registration service operation.

A SMF+PGW-C may support L2TP as described in clause 4.3.2.4. In this case step 1 and step 7 of Figure 4.3.2.4-1 correspond to a PDN Connection establishment and a SMF+PGW-C replaces the SMF in that Figure.

To support User Plane Integrity Protection with EPS and policies that Require User Plane integrity protection to be used, at PDN connection establishment, the MME shall indicate to the SMF+PGW-C (via the Serving GW) whether the UE, the current eNB and the MME support User Plane Integrity Protection in EPS. If the MME and the UE support User Plane Integrity Protection, then the SMF+PGW-C informs the MME of the User Plane integrity protection policy (Required, Preferred, Not Needed) applicable to the PDN connection on a per-EPS bearer basis. In turn, the MME informs the eNB.

To support URSP Provisioning in EPS, during Initial Attach with default PDN connection establishment procedure in EPS, the UE provides the Indication of URSP Provisioning Support in EPS PCO in the PDN connectivity request to SMF+PGW-C. If the SMF+PGW-C supports URSP Provisioning in EPS and ePCO capability, it provides the Indication of URSP Provisioning Support in EPS PCO in the Create Session Response. If the UE receives the Indication of URSP Provisioning Support in EPS PCO from SMF+PGW-C in the PDN Connectivity Accept message, the UE initiates the UE requested bearer resource modification procedure and includes the UE Policy Container ePCO in the Request Bearer Resource Modification message, see clause 4.11.0a.x. If the default PDN connection is not established during Initial Attach procedure, the aforemetioned procedure happens during the first request for PDN connectivity.

#### 4.11.0a.X	UE requested bearer resource modification procedure

To support URSP Provisioning in EPS, the following enhancement to clause 5.4.5 of TS 23.401 [13] applies:

-	Steps 1, if the UE received the Indication of URSP Provisioning Support in eEPS in ePCO from SMF+PGW-C in the PDN Connectivity Accept message (as described in clause 4.11.0a.5), the UE includes the UE Policy Container ePCO in the Request Bearer Resource Modification message.

-	Step 4 (replaced by SM Policy Association Modification procedure as described in clause 4.11.0a.2a.2.1), SMF+PGW-C forwards the UE Policy Container received from the UE to the PCF for the PDU Session, which triggers UE Policy Association establishment as specified in clause 4.11.0a.2a.5.

#### 4.11.0a.X+1	SMF+PGW-C initiated bearer modification without bearer QoS update

To support URSP Provisioning in EPS, the following enhancement to clause 5.4.3 of TS 23.401 [13] applies:

-	Step 1 The PCF provides UE Policy Container, which is received from the PCF for the UE as specified in clause 4.11.0a.2a.6, to the SMF+PGW-C in (PCF-initiated) IP-CAN Session Modification procedure as specified in clause 4.11.0a.3.

-	Steps 2~5 The UE Policy Container in ePCO is sent by the SMF+PGW-C to the UE via SGW and MME

-	Steps 6~9 The UE sends UE Policy delivery result to the SMF+PGW-C via MME and SGW.

-	Steps 10 The SMF+PGW-C reports the UE Policy delivery result to the PCF as specified in clause 4.11.0a.2a.2.3.

* * * Next Changes * * *

#### 4.11.5.10	UE Policy Association control

##### 4.11.5.10.1	UE Policy Association Establishment for URSP delivery in EPS

The following impacts are applicable to clause 4.16.11 (UE Policy Association Establishment procedure) during initial registration or during UE registration with 5GS when the UE moves from EPS to 5GS:

In addition to step 3 in clause 4.16.11, if the indication about the support of provisioning of URSP rules in EPS is received in step 2 within Npcf_UEPolicyControl_Create Request, that indication is added in the list of parameters to be stored in UDR using Nudr_DM_Create.

* * * End of Changes * * *