3GPP TSG-WG SA2 Meeting #160	S2-2313741

# Chicago, US, Nov 13 – 17, 2023		(revision of S2-2312297)

# Source:	Huawei (Moderator)

# Title:	New WID on Study on enhancement of Timing Resiliency, TSC&URLLC, and LAN

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on enhancement of Timing Resiliency, TSC&URLLC, and LAN

Acronym: FS_eTRS_URLLC_LAN

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820019 | Enhancement of Ultra-Reliable Low-Latency Communication support in the 5G Core network | Related Work Item in Release 16 (SA2) |
| 900008 | Support of Enhanced Industrial IIoT | Related Work Item in Release 17 (SA2) |
| 970024 | Timing Resiliency and URLLC enhancements | Related Work Item in Release 18 (SA2) |
| 970069 | Generic group management, exposure and communication enhancements | Related Work Item in Release 18 (SA2) |

Dependency on non-3GPP (draft) specification:

None

3	Justification

Time Resilience related:

SA1 specified requirements for 5G System to remain resilient if there is GNSS failure and for 5G System to act as a backup and offer wireless and indoor-capable time synchronization service for other applications (e.g. financial, power grid systems) in Rel-18. These requirements were targeting to position the 5G timing synchronization services can be resilient to loss of GNSS and can be trusted as a time source for critical infrastructure.

In addition to the ongoing resiliency study for time synchronization service in Rel-18 and its impacts for monitoring and reporting the service status in the specification, SA2 has introduced time synchronization service support in the previous releases. SA2 specified solution for time synchronization support using gPTP in Rel-16, AF requested time synchronization as a service (based on access stratum time distribution, called ASTI, or based on user plane packets using (g)PTP) support in Rel-17 and introduced the ability for subscription control and/or AF to activate / deactivate ASTI/PTP time synchronization service in a certain coverage area in Rel-18. However, focusing on ASTI service, the current specification still assumes that the reference time information (RTI) is broadcasted by the RAN based on OA&M mainly, thus the UE moving into a time synchronization coverage area (where it is supposed to receive time synchronization services) may not receive 5G RTI and the RAN may also be broadcasting RTI in cells/gNBs even if it is not part of the time synchronization coverage area resulting in wasted radio resources. That is, it remains unspecified in Rel-18 how the time synchronization coverage area can be ensured for UEs in RRC_INACTIVE/IDLE state.

Besides, cooperation between multiple mobile robots/AGVs is commonly needed in industry environment. In order to support the time synchronization service for one or more multiple ad-hoc cooperative groups, the 5GS needs to support the management of the establishment/update of the ad-hoc cooperative groups and its grand master clock.

URLLC related:

3GPP specifies replication mechanisms since Rel-16 for URLLC however the drawback is that the URLLC replication is either done for all traffic or no traffic for a given application. However, the current replication mechanism assumes that application layer protocols are used to control the traffic over multiple user plane paths for high reliability, with no control available within 3GPP system for operators.

Besides, capacity utilization is a key point for supporting demanding applications and services such as URLLC applications and XR services. ). But currently the network can only support limited terminals (due to URLLC devices requiring more recourse). It is noticed 5GS has transferred many unnecessary bits: For example, Ethernet has a minimum frame size of 64 Bytes, comprising an 18-Byte header and a payload of 46 Bytes. The application-layer packet payload of industrial applications is usually small, e.g. 20 Bytes.

UE-UE communication is widely used in industry scenario. However some features, e.g. BAT adjustment, Survival Time, cannot be used in UE-UE communication. For BAT adjustment in the UE2UE scenario, if the RANs provides the feedback (e.g. BAT offset) independently, the gap between the two RANs may not be synchronized, and the adjustment cannot be performed correctly. For Survival Time in the UE2UE scenario, it is unclear how the RAN nodes coordinate the packet loss, especially the packet is lost in the second air interface.

Integration of 5G VN and Ethernet LAN:

IIn 5G-VN, two UEs (one at the begin and one at the end of a conveyor belt, may make a ring out of the linear topology of the Ethernet LAN over the 5GS. A loop prevention protocol e.g. MSTP avoids circling Multicast/Unicast frames. Additionally, this setup allows a media redundancy using the 5GS – if the wireline breaks, every node is still reachable through the 5GS. 5GS enhancements for support of MSTP needs to be studied in order to support prevention of loops.

4	Objective

Following are the objectives for this study:

WT-1: Time Synchronization enhancements

WT-1.1:   How to support activation/deactivation Access Stratum based time synchronization based on Time Synchronization coverage area.

-  Enable gNB to track the UE(s) (in RRC_INACTIVE) moving in and out of Time Sync coverage area based on RNA. This is to enable the interested UE(s) obtain time sync information, also gNB can stop broadcasting time sync when the interested UE(s) move out of time sync coverage area.

-  Assist gNB to determine the areas where Time Sync is needed for UE(s) in IDLE mode.

NOTE 1: This is only applicable for the UEs without high accuracy requirements of time synchronization.

NOTE 2: It is assumed this work task requires coordination with RAN3.

WT-1.2 Void

WT-1.3 Whether and how to support time synchronization service for a group of UEs using ASTI based mechanism.

- Assist gNB to broadcast time information related to one or multiple clock domains to the UE accessed to the gNB.

NOTE 3: This WT does not consider Clock synchronization to a group of UEs via ProSe direct/indirect communication.

NOTE 4: It is assumed this work task requires coordination with RAN2&3.

WT 1.4 Void

WT-1.5: Study how to enable reference time delivery based on subscription for WB-EUTRA. It assumes the accuracy will not be changed in E-UTRAN.

- EPS Subscription enhancements for control of reference time delivery to the UE

- MME and eNB enhancements to inform eNBs about need for reference time delivery for the UE

NOTE 5: WT-1.5 requires work in RAN3 since delivering reference time  to individual UEs in dedicated RRC is already supported for WB-EUTRA. It is assumed WB-EUTRA support delivering reference time  to individual UEs in dedicated RRC and there is no new requirement for broadcast in radio (e.g. new SIB information) for WB-EUTRA.

NOTE 6: NTN is out of scope.

NOTE 7: It is assumed this work task requires coordination with RAN3.

WT-1.6: Study how to enable clock quality information delivery based on subscription for WB-EUTRA

- EPS Subscription enhancements for control of clock quality delivery to the UE

- eNB enhancements to provide clock quality information to UEs

NOTE 8: It is assumed this work task requires coordination with RAN3 and RAN2.

NOTE 9: It is assumed this work task does not require PDC support for high accuracy. It is assumed that no RAN1 work is required as a result of this work task.

NOTE 10: The solution defined in Rel-18 for delivery of clock quality information in 5GS can be partly taken as a baseline but there will be no SIB enhancement in WB-EUTRA.

NOTE 11: NTN is out of scope.

WT-2: General URLLC enhancements

WT-2.1 Support for 3GPP system-controlled E2E replication/elimination for redundancy for both Ethernet and IP type PDU Sessions. It is assumed that the two PDU Sessions carrying the redundant traffic shall be as following:

- for a single UE, or

- for two UEs that are independently registered.​

The work task addresses architecture and protocol aspects for the 3GPP network operators on the above scenarios:

WT 2.1.1 For FRER:

- Ability to enable 3GPP entities (e.g. DS-TT, NW-TT) to support replication and elimination for redundancy, protocol aspects to replicate/eliminate for FRER.​

NOTE 12: Either the terminal or network side replication and elimination endpoint or both can be within 5GS.

WT 2.1.2 For DetNet:

- Ability to support replication and elimination for redundancy, protocol aspects to replicate/eliminate for DetNet.​

- Extended capability exposure framework to allow the entity outside the 5GS to discover, or request selective duplication, 5GS reliability and redundancy functionality for DetNet.

NOTE 13: Either the terminal or network side replication and elimination endpoint or both can be within 5GS. Deployments can support DetNet controller for replication/elimination that is outside 3GPP system but the solution for replication/elimination in this scenario is outside 3GPP system.

WT 2.1.3 new 3GPP specific replication and elimination protocol:

- Ability to support replication and elimination for redundancy, protocol aspects to replicate/eliminate.​

- Extended capability exposure framework to allow the entity outside the 5GS to discover, or request selective duplication, 5GS reliability and redundancy functionality

NOTE 14: It is assumed both the terminal and network side replication and elimination endpoint to be within 5GS.

NOTE 15: Configurations and mechanisms of IEEE TSN network or IETF DetNet will not be impacted by WT 2.1.

WT 2.2 Study whether and how to support for Ethernet paddings removal in the UE and the network for UL and DL, respectively to improve efficiency of transmission capacity.

WT 2.3 Void

WT 2.4 Study whether and how to support BAT adjustment (i.e. BAT offset) in UE-to-UE scenario considering two legs of the service.

NOTE 16: RAN2/3 WGs need to be involved to check the feasibility of the adjustment.

WT 2.5 Support adapting survival time considering UE-to-UE scenario considering two legs of the service.

NOTE 17: RAN2/3 WGs need to be involved to check the feasibility of the adapting.

WT-3: Void

WT-4: Integration of 5G VN and Ethernet LAN

WT-4.1:

- WT-4.1.1: Void

- WT-4.1.2: Void

- WT-4.1.3: How to support integration of 5G VN and Ethernet LAN to enable supporting activation/deactivation of Multiple Spanning Tree protocol in the UPF to ensure loop prevention.

WT-4.2 Void

WT-4.3 Void

WT-4.4 Void.

WT-4.5 Void.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 2.5 | 1.75 | Yes | WT#1 is self-contained |
| WT#1.1 | 1 | 0.5 | Yes |  |
| WT#1.3 | 0.5 | 0.25 | Yes |  |
| WT-1.5 | 0.5 | 0.5 | Yes |  |
| WT-1.6 | 0.5 | 0.5 | Yes |  |
| WT#2 | 4.5 | 4.5 |  | WT#2 is self-contained |
| WT#2.1.1 | 0.5 | 0.5 | No |  |
| WT#2.1.2 | 0.5 | 0.5 | No |  |
| WT#2.1.3 | 1 | 1 | No |  |
| WT#2.2 | 0.5 | 0.5 | Yes |  |
| WT#2.4 | 1 | 1 | Yes |  |
| WT#2.5 | 1 | 1 | Yes |  |
| WT#4 | 1 | 1 | No | WT#4 is self-contained |
| WT#4.1.3 | 1 | 1 | No |  |

Total TU estimates for the study phase: 8

Total TU estimates for the normative phase: 7.25

Total TU estimates: 8 +7.25 = 15.25

5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.abc | Study on enhancement of Timing Resiliency, TSC&URLLC, and LAN |  |  |  |

{Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

7	Work item leadership

SA2

8	Aspects that involve other WGs

Security aspects are considered by SA3.

RAN aspects are considered by RAN WGs.

9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

Samsung

NICT

China Unicom

SIA

China Mobile