3GPP TSG SA2 Meeting # 160	S2-23131396

# November 13 – 17, 2023, Chicago, USA

Source:	T-Mobile USA Inc

Title:	TEI19 on enhancing Emergency capability when CS is not supported.

Document for:	Information

Agenda Item: 	30.2

Work Item / Release:	Rel19

Abstract of the contribution: Supplies a draft of a Work Item description for a TEI 19 proposal to provide an indicator for UE to know when a network has no CS capability, and thus emergency retries using CS may be skipped.

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Emergency Services with Zero CS Network

Acronym:	EM0CS

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  |  |  |  |
| Don't know |  |  | X |  | X |

NOTE: AN Impact becomes “No” if NAS indicator is used and “Yes” if broadcast indicator is used.

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| NA | NA | NA | NA |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| None | None | None |

# 3	Justification

When IP Voice was initially introduced it was undetermined how effective they would be at meeting regulatory requirements for emergency services, and thus emergency service voice calls were specified to attempt emergency services using Circuit Switched (CS) capabilities in case the corresponding IP voice emergency services were not implemented or not available (according to a table in TS 23.294). While IP voice services have improved in terms of reliability and capability over time, this capability has remained as an effective means of ensuring a user is connected to emergency services if there are problems with the PS network (including congestion). However, an increasing number of operators are retiring their 2G and 3G access networks and thus these networks no longer have a CS option – the attempt to use the CS access network for emergency calls is now causing a delay before the next step in attempting an emergency call by any available network is attempted.  Currently, there is no means by which the UE knows the network it is connected to does not have CS voice capability, and since the UE may roam to networks that may have CS as well as networks that don’t, it is not prudent to always skip the step.  Hence some indication needs to be supplied to the UE that skipping the CS emergency attempt will avoid an unnecessary delay in moving to the next step in emergency service delivery.

Additionally, while TS 23.294 does allow for a UE to skip an attempt to use the CS network (when none is available), this is currently only defined for a single network configuration scenario (in Annex H of TS 23.294).  At the time of writing this section of TS, this was the only scenario in which CS could be absent, however as additional PS access technologies have been defined it is possible that other network configurations defined in this table may lack a corresponding CS network – thus this table needs to be re-visited and the capability to skip a CS network that is not present needs to be added to configurations where it may be applicable.

# 4	Objective

The objective of this work is to

- Specify an indicator from the network to the UE that indicates that re-attempting emergency services using CS may be skipped by the UE while attached to this network.

NOTE:	Whether to use a NAS indicator or a RAN broadcast indicator to inform the UE will be determined as part of this work.

- Update the network scenarios in which skipping a re-attempt using a CS network is permissible.

## TU estimates and dependencies

Total TU estimates for the study phase: 0 (TEI-19 has no study phase)

Total TU estimates for the normative phase: 1

Total TU estimates: 0 + 1 = 1

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| None | None | None | None | None | None |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS23.294 | Add use of UE indication of no CS emergency support by network. Allow skip of CS emergency attempt for additional network configurations cases in table in Annex H | TBD |  |
| TS23.501 | Add indication of no CS support for emergency voice in Emergency Services description | TBD | Assumes NAS indicator |
| TS23.502 | Add NAS indicator to registration flows | TBD | Assumes NAS indicator |

# 6	Work item Rapporteur(s)

Primary: Chris Joul (T-Mobile USA) chris <dot> joul <at> t <dash> mobile <dot> com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

T-Mobile USA