SA WG2 Meeting #S2-160	S2-2311924

13 - 17 November, 2023, Chicago, USA

SA WG2 Meeting #S2-159	S2-2310075

09 - 13 October, 2023, Xiamen, P.R. China

SA WG2 Meeting #S2-158	S2-2308295

21 - 25 August, 2023, Goteborg, Sweden

3GPP TSG-CT WG1 Meeting #142	C1-234389

# Bratislava 22 – 26 May 2023

# Title:	Reply LS on UPSI handling at the UE

# Response to:	LS on UPSI handling at the UE (S2-2211347)

# Release:	Rel-15, Rel-16, Rel-17, Rel-18

# Work Item:	5GS_Ph1-CT, 5GProtoc16, 5GProtoc17, 5GProtoc18

Source:	CT1

To:	SA2

Cc:	CT3

Contact Person:

Name:	Ivo Sedlacek

E-mail Address:	ivo dot sedlacek at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

CT1 would like to thank SA2 for the LS S2-2211347 on UPSI handling at the UE.

SA2 asked:

--------------

Question 1: How will the UE react if the PCF provides a Policy Section that contains only the PSI and there is no Policy Section stored in the UE with that PSI?

Question 2: Will the same UE behaviour be expected for a Rel-15, Rel-16 and Rel-17 UE?

--------------

There are different possible interpretations from CT1 specification within CT1 WG regarding what the UE behavior is.

When the PCF provides a Policy Section that contains only the PSI and there is no Policy Section stored in the UE with that PSI, and the UE is compliant to TS 24.501 Rel-15 only, Rel-16 only, Rel-17 only, or present Rel-18 version, depending on interpretation of the specification, the UE can:

- send a MANAGE UE POLICY COMMAND REJECT message; or

- ignore the instruction (i.e. the Policy Section that contains only the PSI), not diagnose an error, and continue processing MANAGE UE POLICY COMMAND message.

2. Actions:

To SA2 group.

ACTION:	CT1 would like to ask SA2 to take the above into consideration.

3. Date of Next CT1 Meetings:

CT1#143	21st  – 25th August 2023 		Goteborg

CT1#144	9th   – 13th October 2023 		China (TBC)

CT1#145	13th   – 17th November 2023 		Chicago