SA WG2 Meeting #S2-160	S2-2311925

13 - 17 November, 2023, Chicago, USA

SA WG2 Meeting #S2-159	S2-2310080

09 - 13 October, 2023, Xiamen, P.R. China

SA WG2 Meeting #S2-158	S2-2308322

21 - 25 August, 2023, Goteborg, Sweden

3GPP TSG-RAN3 Meeting #120                                                                 R3-233432

Incheon, KR, 22th - 26th May 2023

# Title:	Response to Reply LS on Support of network slices which have area of service not matching deployed tracking areas

# Response to:	R3-232509/S2-2306045

# Release:	Rel-18

# Work Item:	eNS_Ph3

Source:	RAN3

To:	SA2

Cc:	SA5

Contact Person:

Name:	Angelo Centonza

E-mail Address:	angelo (dot) centonza (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: none

1. Overall Description:

RAN3 would like to thank SA2 for their LS on Support of network slices which have area of service not matching deployed tracking areas.

RAN3 has further discussed how to prepare NG-RAN resources to serve the alternative S-NSSAI and would like to request feedback on the following approaches proposed:

Approach 1: The resources associated to the RRMPolicyRatio(s) (See TS28.541) of which the alternative S-NSSAI is member are adjusted (by OAM) in order to take into account the additional load potentially generated by remapping one or more slices into the alternative S-NSSAI.

Approach 2: By retrieving data from the OAM and making them available to the AMF, the AMF can determine the RRMPolicyRatio utilisation for the resources of potential Alternative S-NSSAIs. In this way, an appropriate Alternative S-NSSAIs can be selected, whose RRMPolicyRatio(s) has free resources. The AMF/SMF is expected to select different Alternative S-NSSAIs on a per UE basis to achieve load distribution.

Approach 3: Coordination is taken to ensure that both the original slice and the one or more potential remapped slices are member of the same RRMPolicyRatio(s) (See TS28.541). This ensures that the resources that would have been used at RAN for the original slice may be used instead for the alternative slice.

2. Actions:

To SA2

ACTION: 	
RAN3 kindly asks SA2 to provide feedback accordingly.

3. Date of Next RAN3 Meetings:

Please check the RAN3 calendar here