SA WG2 Meeting #S2-160	S2-2311926

13 - 17 November, 2023, Chicago, USA

SA WG2 Meeting #S2-159	S2-2310098

09 - 13 October, 2023, Xiamen, P.R. China

3GPP TSG-CT WG1 Meeting #143	C1-236562

# Goteburg, 21– 25 August 2023

# Title:	LS on LPP message and supplementary service event report over a user plane connection between UE and LMF

# Response to:	LS (S2-2301857) on LPP message and supplementary service event report over a user plane connection between UE and LMF

# Release:	Rel-18

# Work Item:	5G_eLCS_Ph3

Source:	CT1

To:	SA2

Cc:	SA3, RAN2, CT4

Contact person:

Name:	Mikael Wass

Tel. Number:

E-mail Address:	mikael.wass@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None.

1. Overall Description:

CT1 would like to thank SA2 for the LS on LPP message and supplementary service event report over a user plane connection between UE and LMF. In addition to the initial response provided in reply-LS C1-231129 CT1 would like to add the following response.

SA2 Question:

“SA2 kindly asks CT1 and CT4 to comment on suitable types of user plane address for the LMF supporting user plane positioning (e.g. such as an IP address and/or FQDN) and whether, which or what the transport protocols and application level messages to transfer LPP messages and supplementary service event report could be identified or defined by CT1 or CT4.”

CT1 Answer:

For specification of transport of LPP messages and supplementary service event reports over a user plane connection between UE and LMF, CT1 has decided to specify a new protocol, LCS-UPP, in 3GPP TS 24.572. The LCS-UPP protocol provides connectivity on a secure user plane connection and supports both LPP and supplementary services.

CT1 further noted that user plane positioning using SUPL is described in 3GPP TS 38.305 and 3GPP TS 23.271 since Rel15, and it is the view of CT1 that this option remains unchanged even after introducing the user plane positioning based on LCS-UPP. CT1 intends to specify requirements for co-existence between the SUPL option and the LCS-UPP option as part of 3GPP TS 24.572.

2. Actions:

To SA2 group.

ACTION: 	CT1 asks SA2 group to take the above information into account and, if needed, update related specification accordingly.

3. Date of Next CT1 Meetings:

CT1#144	9th   – 13th October 2023 		Xiamen

CT1#145	13th   – 17th November 2023 		Chicago