SA WG2 Meeting #S2-160	S2-2311929

13 - 17 November, 2023, Chicago, USA

SA WG2 Meeting #S2-159	S2-2310107

09 - 13 October, 2023, Xiamen, P.R. China

3GPP TSG-CT WG4 Meeting #117	C4-233550

# Göteborg, Sweden, 21st– 25th August 2023

Title:	LS On EAC Mode Subscription Optimization

Response to:	-

Release:	Rel-18

Work Item:	TEI18

Source:	CT4

To:	SA2

Cc:	-

Contact person:	Jones Lu

Jones (dot) Lu (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

During implementation of the NSAC, CT4 has observed certain limitations of the current service operations:

- In the current design, the AMF implicitly subscribes to the NSACF for the EAC mode status and change of certain S-NSSAI in the service operation to perform NSAC for certain UE. The AMF cannot get the EAC mode from the NSACF before the first UE NSAC service operation for the S-NSSAI. After the subscription was established, when the AMF needs to update the EAC notification URI or is no longer interested in receiving EAC modes, the AMF can only make the change via a subsequent service operation to perform UE NSAC. Before the NSACF is aware of the change, the NSACF may retry the EAC notification to the AMF and continuously receive negative responses or timeouts.

- When Access Type specific NSAC is configured in NSACF, the number of UEs per S-NSSAI is counted per access type, but EAC mode currently is access type agnostic.

CT4 has the following questions to SA2 for clarification:

Q1: Whether the AMF should be allowed to create and/or modify the EAC mode subscription independently, instead of always relying on a service operation for UE NSAC?

Q2: Will it be considered to have EAC Mode per access type, when per access type NSAC is configured in the NSACF?

2. Actions:

To SA WG2 group:

ACTION: 	CT4 kindly asks SA2 to answer the listed questions.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm