SA WG2 Meeting #S2-160	S2-2311931

13 - 17 November, 2023, Chicago, USA

SA WG2 Meeting #S2-159	S2-2310111

09 - 13 October, 2023, Xiamen, P.R. China

3GPP TSG-CT WG4 Meeting #117	C4-233692

# Göteborg, Sweden, 21st– 25th August 2023

# Title:	Reply LS on Proposed method for AMF discovery and subscription by the TSCTSF

# Response to:	LS (S2-2308233) on Proposed method for AMF discovery and subscription by the TSCTSF from SA2

# Release:	Rel-18

# Work Item:	TRS_URLLC

Source:	CT4

To:	SA2

Cc:	CT3

Contact Person:

Name:	Bruno Landais

Tel. Number:

E-mail Address:	bruno.landais@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

CT4 thanks SA2 for their LS on Proposed method for AMF discovery and subscription by the TSCTSF.

SA2 question: There were concerns related to excessive signaling and failure issues thus SA2 would like to kindly request CT4 to provide feedback regarding the two methods from stage 3 perspective.

Both alternatives a) (NRF based AMF discovery and subscription) and b) (UDM based AMF discovery and subscription) described in the SA2 LS (S2-2308233) can be used by the TSCTSF to discover and subscribe to AMFs, and which one is preferable may depend on deployment and service-related factors.

The alternative a) may be preferred for AF requests targeting a group of UEs. In this case, the TSCTSF needs to create an AMF event subscription targeting a group of UEs (identified by an Internal Group ID) at every AMF instance of every AMF set covering (part of) the AoI, including at AMF instances that may restart later on and at new AMF instances that may register later on and that support the AoI (like for any UE group subscriptions to any NF of an NF set).

The alternative b) may be preferred for AF requests targeting a UE or a few specific UEs. In this case, the TSCTSF needs to create an AMF event subscription for each target UE. For AF requests targeting specific UEs, alternative a) would require the TSCTSF to create an AMF event subscription targeting ANY UE, which could cause intensive processing and event reporting signaling in the 5GC, including for UEs that do not require the support of time synchronization services.

The following potential enhancements might be considered by SA2 (for the alternative a)):

- to keep the event subscription and notification signaling only to the AMFs serving the AoI, the Namf_EventExposure Subscribe request could support an optional parameter indicating the AMF to not propagate the group subscription to any target AMF during inter AMF mobility of group members.

- to enable alternative a) for AF requests targeting specific UEs, an Namf_EventExposure Subscribe request targeting ANY UE could contain further filter information, e.g. a list of SUPIs, DNN/S-NSSAI dedicated for time synchronization service, or an indication to apply the subscription to any UE having user subscription enabling Time Synchronization service.

2. Actions:

To SA2 group.

ACTION: 	CT4 asks SA2 group to take the above information into account.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm