SA WG2 Meeting #S2-160	S2-2311932

13 - 17 November, 2023, Chicago, USA

SA WG2 Meeting #S2-159	S2-2310112

09 - 13 October, 2023, Xiamen, P.R. China

3GPP TSG-CT WG4 Meeting #117	C4-233776

# Göteborg, Sweden, 21st– 25th August 2023                                                      was C4-233633

Title:	LS on Clarification related to Network Slicing enhancement

Response to:	NA

Release:	Rel-18

Work Item:	eNS_Ph3

Source:	3GPP CT4

To:	3GPP SA2

Cc:	3GPP CT1, 3GPP CT3

Contact person:	Rajesh Babu Natarajan

rajesh underscore babu dot natarajan at nokia dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

In the current TS 23.501,

Clause 5.15.6 states:

If the serving AMF supports the Network Slice Replacement feature and is configured to use the NSSF for Network Slice Replacement triggering, the AMF subscribes with the NSSF of the VPLMN for notifications when an HPLMN S-NSSAI needs to be replaced with an Alternative S-NSSAI, in addition to notifications for the Serving PLMN S-NSSAIs. The NSSF of the VPLMN shall subscribe with the NSSF of the HPLMN for notifications when an HPLMN S-NSSAI needs to be replaced with an Alternative S-NSSAI.

Question 1: Does the above requirement apply only to Home Routed roaming use case? Or also to LBO roaming use case?

Clause 5.15.19 states:

, the AMF provides the Alternative S-NSSAI for this S-NSSAI in the Allowed NSSAI and in the Configured NSSAI, if not included yet, and the mapping between S-NSSAI(s) to Alternative S-NSSAI(s) to the UE in UE Configuration Update message as follows:

-    for non-roaming UEs, the AMF provides the mapping of the S-NSSAI to the Alternative S-NSSAI to the UE.

-    for roaming UEs when the VPLMN S-NSSAI has to be replaced by a VPLMN Alternative S-NSSAI, the AMF provides the mapping of the VPLMN S-NSSAI to the Alternative VPLMN S-NSSAI to the UE.

-    for roaming UEs when the HPLMN S-NSSAI has to be replaced by an Alternative HPLMN S-NSSAI, the AMF provides the mapping of the HPLMN S-NSSAI to the Alternative HPLMN S-NSSAI to the UE.

Question 2: In the above requirement, HPLMN S-NSSAI replaced with HPLMN Alternative S-NSSAI is applicable only in case of Home routed roaming use case? Or also to LBO roaming use case?

In the current TS 23.502,

Clause 4.3.2.2.1 states:

For roaming scenario in local breakout (LBO), the AMF also sends the corresponding S-NSSAI of the HPLMN from the Mapping Of Allowed NSSAI or Mapping Of Partially Allowed NSSAI to the SMF. If the the AMF determines to replace the HPLMN S-NSSAI received from the UE with the HPLMN Alternative S-NSSAI or the AMF receives the HPLMN Alternative S-NSSAI and the HPLMN S-NSSAI provided by the UE, the AMF sends both HPLMN S-NSSAI and HPLMN Alternative S-NSSAI to the SMF.

Question 3: In the above requirement, why would AMF replace HPLMN Alternative S-NSSAI in case of LBO roaming? (for example, why would congestion or unavailability of a slice in HPLMN impact an equivalent slice being used in VPLMN by an LBO roaming UE?)

# 2	Actions

To SA2

ACTION: 	CT4 kindly asks SA2 to answer the above questions and update the SA2 specifications if necessary.

# 3	Dates of next TSG CT WG 4 meetings

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm