3GPP TSG-WG SA2 Meeting #160	S2-2312019

# Chicago, US, Nov 13 – 17, 2023		(revision of S2-231xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4552 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the MemberUESelectionAssistance service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The format of the UE filtering information for different Member UE filtering criteria is not aligned. The DNN/S-NSSAI could be in the optional information of QoS filtering criteria. As described in clause 4.15.13.3.1, the traffic descriptor (e.g. Application ID) is included in the QoS filtering criteria, so the figure and description in clause 4.15.13.3.3 should be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some editorial changes to the Table of Member UE filtering criteria. Add DNN/S-NSSAI as the optional information of QoS filtering criteria. Update the figure and description in clause 4.15.13.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some descriptions are not aligned between the procedures and the MemberUESelectionAssistance service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.2, 4.15.13.3.1, 4.15.13.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.15.13.2	Member UE Filtering Criteria for 5GS assistance to Member UE selection

Table 4.15.13.2-1 provides a summary of the Member UE filtering criteria that the AF may request.

Table 4.15.13.2-1: Description of Member UE filtering criteria

| Member UE filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the selected UEs match or exceed the QoS of the filtering criteria | Service operation: Nsmf_EventExposure or Nudm_EvenExposure Filter: target=SUPI, traffic descriptor (e.g. Application ID), DNN/S-NSSAI Event ID: QoS Monitoring | 4.15.13.3 |
| Access Type and/or RAT Type of the PDU Session | Indicate the Access Type and/or RAT Type of the selected UE for the PDU Session used by the application (e.g. 3GPP/NR, Non-3GPP/WLAN, additional Access Type and RAT Type for MA PDU session) | Service operation: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI Event ID: Change of Access Type and/or Change of RAT Type | This can be provided by existing service operations, events and procedures. |
| End-to-end data volume transfer time | Indicate the end-to-end data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE and AF, e.g. the average and variance of End-to-end data volume transfer time | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: target = GPSI(s) or SUPI(s) Analytics ID: End-to-end data volume transfer time | 4.15.13.6 |
| UE current location | Indicate the certain area that the selected UE currently located in. | Service operation: Namf_EventExposure Filter: a list of GPSI(s) or SUPI(s) Event ID: Location Report | 4.15.13.4 |
| UE historical location | Indicate the certain area that the selected UE appeared in a historical period of time | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Visited AoI = Target AOI, target period = historical nomadic period Analytics ID: UE mobility | 4.15.13.4 |
| UE direction | Indicate the selected UEs should include different moving direction | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Direction Analytics ID: UE Mobility | 4.15.13.4 |
| UE separation distance (NOTE 1) | Indicate the selected UEs should comply with a minimum separation distance between each other | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Proximity Attributes Analytics ID: Relative Proximity | 4.15.13.4 |
| Service Experience | Indicates selected UEs fulfilling ceratin Service Experience criteria e.g., MOS value | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: S-NSSAI, DNN, Application ID, DNAI, AoI, Service Experience Contribution weight, reporting threshold (NOTE 2), Service Experience Type (NOTE 3) Analytics ID: Service Experience | 4.15.13.5 |
| DNN | Indicate the DNN of the selected UE for the PDU Session used by the application | Service operation: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s) Event ID: QFI allocation | 5.2.8.3 |
| NOTE 1: This criterion should only be applied when the number of UEs is in the range of 10's or less. NOTE 2: The Service Experience Contribution Weights signal the relative importance of each UE's Service Experience value (i.e. MOS), as defined in TS 23.288 [50]. For example, it might be that the service experience of a UE in relation to other UEs may not be as important e.g. because the data provided by such UE is not as critical to the service. NOTE 3: Indicates the type of service experience analytics, e.g. AIML traffic where a customized MoS apply. |  |  |  |

# * * * * Next change * * * *

##### 4.15.13.3.1	General

An AF may invoke Nnef_MemberUESelectionAssistance_Subscribe service operation with a QoS filtering criteria for receiving a list of UEs that match or exceed such criteria.

In addition to the mandatory parameters, the AF also includes in the request:

-	QoS filtering criteria.

-	Optionally, an Area of Interest: location area of the candidate UEs.

The QoS filtering criteria includes:

-	a traffic descriptor (e.g. Application ID).

-	Optionally, DNN/S-NSSAI.

And one or more of the QoS parameters subject to QoS monitoring (see list in clause 5.45 of TS 23.501 [2]).

The current list of UEs that match or exceed the QoS filtering criteria and are optionally located in the AoI in real time, for the duration of the subscription, is further described in clause 4.15.13.3.3.

# * * * * Next change * * * *

##### 4.15.13.3.3	Member UE Selection Assistance with QoS filtering criteria for real-time QoS Monitoring

At the reception of Nnef_MemberUESelectionAsistance_Subscribe request of step 1, in order to detect the list of UEs that fulfil the QoS filtering criteria in real time, NEF performs real-time QoS monitoring. Unless the QoS flow to be monitored is associated with a default QoS flow, QoS Monitoring needs to be activated in SMF for that QoS Flow before (e.g. by AF Session with required QoS) (see clause 4.15.4.5.1 for details).

AF

NEF

1. Nnef_MemberUESelectionAssistance_Subscribe

(AF ID, a list of target UE(s), QoS filtering criteria)

8. Nnef_MemberUESelectionAssistance_Notify

(list(s) of candidate UE(s))

4a. Nsmf_EventExposure_Subscribe (Event: ‘QoS

Monitoring’, a list of target UE(s), optional App ID,

S-NSSAI, DNN, …)

SMF 1

SMFn

4b. Nsmf_EventExposure_Subscribe (Event: ‘QoS

Monitoring’, a list of target UE(s), optional App ID,

S-NSSAI, DNN, …)

6a. Nupf_EventExposure_Notify (target UE, QoS monitoring parameters)

6b. Nupf_EventExposure_Notify (target UE, QoS monitoring parameters)

2. Service authorization for AF request, mapping the QoS filtering criteria to the corresponding service operation

7. Consolidation of the candidate UE list

3. Discovery and selection of target SMFs

UPF 1

UPFn

5a. N4 session modification

5b. N4 session modification

Figure 4.15.13.3.3-1: 5GC assistance to Member UE selection for real-time QoS monitoring

1.	AF subscribes to the Member UE selection assistance functionality by sending Nnef_MemberUESelectionAssistance_subscribe request including its AF ID, a list of target UE(s) and the QoS filtering criteria.

2.	NEF uses the AF ID to verify the authorization of the AF request and identifies which information needs to be collected and executed based on the QoS filtering criteria provided by the AF.

3.	If an S-NSSAI/DNN is not included in the request from the AF, the NEF derives the S-NSSAI and DNN to which this e.g. Application ID have access. NEF discovers the SMFs that are deployed in the Area of Interest by querying UDM and NRF. The NEF may also restrict the discovery to those SMFs that serve some S-NSSAI and DNN combination.

4.	For each target SMF, NEF sends an Nsmf_EventExposure_Subscribe request (Event: 'QoS monitoring', target UEs, optionally a traffic descriptor (e.g. Application ID), optionally indication for default QoS flows monitoring, S-NSSAI, DNN, Notification Target Address set to NEF, etc.). Alternatively, NEF may send an Nudm_EventExposure_Subscribe request to the UDM (not shown in Figure 4.15.13.3.3-1).

5.	The SMF may need to send an N4 Session Modification request to UPF for requesting the QoS monitoring for certain flows.

6.	UPF sends an Nupf_EventExposure_Notify request to NEF including an Event Exposure notification, according to the subscription received from SMF.

7.	Based on the Event Exposure reports received from the UPFs, NEF consolidates the received results and derives the list(s) of candidate UE(s) and additional information which fulfil the QoS filtering criteria provided by the AF.

8.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and additional information.

# * * * * End of changes * * * *