3GPP TSG-WG SA2 Meeting #160	S2-2312025

# Chicago, USA, November 13 – 17, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0978 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Ndccf_DataManagement_Transfer service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the procedure for DCCF relocation initiated by DCCF, the source DCCF using Ndccf_DataManagement_Transfer Request service operation to transfer of UE data subscription context to the target DCCF. However, the Ndccf_DataManagement_Transfer is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the Ndccf_DataManagement_Transfer service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Incomplete of the procedure for DCCF relocation initiated by DCCF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.3.8, 8.1, 8.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.2.6.3.8	DCCF and MFAF relocation initiated by DCCF

The procedure depicted in Figure 6.2.6.3.8-1 is used by a data consumer (e.g. NWDAF or central DCCF) to obtain UE data and to support DCCF and MFAF reselection when the DCCF or MFAF can no longer serve the UE.

Data Consumer

Source DCCF

Target DCCF

11. Ndccf_DataManagement_Transfer response

16. Ndccf_DataManagement_Notify

Source MFAF

14. Nnf_EventExposure unsubscribe

8. Ndccf_DataManagement_Transfer Request

13. Ndccf_DataManagement_Notify

AMF

0. Ndccf_DataManagement_Subscribe

Target MFAF

Data source

3. Discovers and selects target DCCF and/or MFAF

4. Nmfaf_3daDataManagement_TransferInitiation Request

5. Nmfaf_ContextManagement_Transfer request / response

7. Nmfaf_3daDataManagement_TransferInititation Response

(conditional)

If MFAF Transfer

(conditional)

If DCCF Transfer

9. DCCF takes over data subscription

17 Nmfaf_3caDataManagement_Notify

15. Target DCCF Or MFAF collects new data from data source(s).

10 Nmfaf_3daDataManagement_ Configure

12.  Nnf_EventExposure subscribe

2. Determines UE data subscription transfer for DCCF and/or MFAF

1. Subscribes UE mobility events.

6. MFAF Takes over UE Context

Figure 6.2.6.3.8-1: Procedure for DCCF relocation initiated by DCCF

0.	The data consumer subscribes to source DCCF. The data consumer may indicate in the subscription request with an indicator that the DCCF may execute the relocation procedure.

NOTE 1:	If the source DCCF or target DCCF does not support relocation, the consumer may execute DCCF relocation based on internal logic.

1.	Source DCCF subscribes UE mobility events from AMF. The UE ID is provided by the data consumer in step 0.

2.	If UE moves out of the service area of the source DCCF, source DCCF determines UE DCCF subscription context to be transferred to target DCCF, e.g., triggered by a UE mobility event notification from AMF. If UE moves out of the service area of the source MFAF, source DCCF determines UE MFAF data subscription to be transferred to target MFAF.

3.	Source DCCF may query the NRF to discover and select the target DCCF and/or MFAF, e.g. based on the UE location information received from AMF.

Conditional on MFAF transfer:

4.	Source DCCF uses the Nmfaf_3daDataManagement service to request transfer of UE MFAF data subscription context to the target MFAF.

5.	Target MFAF retrieves MFAF subscription context from Source MFAF.

6.	Target MFAF accepts the data subscription(s) context transfer.

7.	Target MFAF responds to the DCCF indicating the transfer is complete. The response may contain a Transaction Reference ID from the Target MFAF.

Conditional on DCCF transfer:

8.	Source DCCF using Ndccf_DataManagement_Transfer Request service operation to transfer of UE data subscription context to the target DCCF.

9.	Target DCCF accepts the data subscription(s) context transfer.

10.	If an MFAF is being used, the Target DCCF uses the Nmfaf_3daDataManagement_Configure service to configure the target MFAF.

11.	Target DCCF confirms UE data subscription context transfer to the source DCCF. The confirmation includes the Subscription Correlation ID used by the Target DCCF.

For DCCF or MFAF transfer:

12.	[Optional] Target DCCF subscribes to the relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the data subscription context and repeat step 1 to subscribes for UE mobility events from AMF.

NOTE 2:	For MFAF context transfer without DCCF context transfer, the Target DCCF is also the Source DCCF.

13.	Source DCCF informs the data consumer about the successful UE DCCF or MFAF data subscription context transfer using a Ndccf_DataManagement_Notify message. The notification may contain a Subscription Correlation ID provided by the target DCCF.

14.	[Optional] Source DCCF unsubscribes with the data source(s) that are no longer needed for the remaining UE data subscriptions.

NOTE 3:	At this point, UE data subscription transfer is deemed completed.

15-17.	Target DCCF or MFAF collects data from the data source(s) and notifies the data consumer using a Ndccf_DataManagement_Notify message.

# * * * *Second change* * * *

## 8.1	General

Table 8.1-1 shows the DCCF services and DCCF service operations.

Table 8.1-1: NF services provided by DCCF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndccf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF |
|  | Unsubscribe |  | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF |
|  | Notify |  | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF |
|  | Fetch | Request / Response | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF |
|  | Transfer | Request / Response | DCCF |
| Ndccf_ContextManagement | Register | Request / Response | NWDAF, ADRF |
|  | Update | Request / Response | NWDAF, ADRF |
|  | Deregister | Request / Response | NWDAF, ADRF |

# * * * *Third change* * * *

### 8.2.X	Ndccf_DataManagement_Transfer service operation

Service operation name: Ndccf_DataManagement_Transfer.

Description: Source DCCF transfers UE data subscription context to the target DCCF.

Inputs, Required: UE data subscription context(UE ID，Data Specification, Notification Target Address(es) (+ Notification Correlation ID(s))).

Inputs, Optional: Service Operation, Time Window, NF (or NF-Set) ID, ADRF or NWDAF hosting ADRF information where data are to be stored.

Outputs Required: Subscription Correlation ID, Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *