3GPP TSG-SA WG2 Meeting #160	S2-2312082

# November 13 – 17, 2023, Chicago, USA									 (revision of S2-231xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0388 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on handling location information from NG-RAN in multi-path operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN3 LS on handling of location information in multi-path operation (S2-2311965/R3-235761) asks the following question to SA2. Question: Whether it is required for NG-RAN node to report both cell IDs of direct path and indirect path to AMF, in case the direct path and the indirect path connect with different cells of same gNB? Therefore, clarification on this aspect is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §6.9.3 - Clarify that how for NG-RAN to handle location information for the case of difference cells in multi-path operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How for NG-RAN to handle location information is unclear for the case of difference cells in multi-path operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.9.3	Multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay

When the direct Uu path and indirect path via 5G ProSe Layer-2 UE-to-Network Relay connect with different cells of same gNB, the gNB provides cell IDs of both paths to the AMF, that is, cell ID of the indirect path is provided as additional cell ID. The AMF may consider the additional cell ID when it determines UE location.

Editor's note:	Alignment with RAN WGs is to be done.

* * * * End of Changes * * * *