3GPP TSG-SA WG2 Meeting #160	S2-2312086

# 13 – 17 November, 2023, Chicago, USA

Title:	[DRAFT] Reply LS on 5G ProSe UE-to-UE relay discovery with security aspects

Response to:	LS on 5G ProSe UE-to-UE relay discovery with security aspects (S2-2311951/C1-237897)

Release:	Rel-18

Work Item:	5G_ProSe_Ph2

Source:	SA2

To:	CT1

Cc:	SA3

Contact Person:

Name:	Kisuk Kweon

Tel. Number:

E-mail Address:	kisuk.kweon AT samsung DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.304 CR2199 CR2201 CR2202 CR2203

1. Overall Description:

SA2 would like to thank CT1 on LS on 5G ProSe UE-to-UE relay discovery with security aspects (S2-2311951/C1-237897).

SA2 has discussed the questions and would like to provide the answers as followings.

- As specified in TS 33.503 clause 6.1.3.3.3.2, for 5G ProSe UE-to-UE Relay Discovery with Model B, the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE are included in a protected direct discovery set and sent between the discoverer 5G ProSe End UE and the discoveree 5G ProSe End UE via the 5G ProSe UE-to-UE Relay UE.

Question: How does the 5G ProSe UE-to-UE Relay UE send the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE to the corresponding discoverer 5G ProSe End UE, considering 1) the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in the protected direct discovery set are transparent to the 5G ProSe UE-to-UE Relay UE and 2) the 5G ProSe UE-to-UE Relay UE may serve more than one discoverer 5G ProSe End UE simultaneously?

SA2 answer to Question: SA2 has discussed this issue and agreed to include an indication/parameter other than the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in both the 5G ProSe UE-to-UE Relay Discovery Solicitation message from the 5G ProSe UE-to-UE Relay and the 5G ProSe UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE (see the attached CRs).

With additional indication/parameter, the 5G ProSe UE-to-UE Relay can identify which discoverer 5G ProSe End UE is destination of the 5G ProSe UE-to-UE Discovery Reponse message from the discoveree 5G ProSe End UE.

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to take the above answers into account and develop the corresponding stage 3 specifications.

3. Date of Next SA2 Meetings:

TSG-SA2 Meeting #Ad Hoc	21-29 January 2024 	Electronic

TSG-SA2 Meeting #162		26 February – 1 March 2024	Athens, Greece