3GPP TSG-WG SA2 Meeting #160	S2-2312101

Chicago, USA, 13 - 17 November, 2023

Title:	[Draft]LS Reply on Provisioning separate DL and UL PDU Set QoS Parameters to NG-RAN

Response to:	S2-2311967/ R3-235890

Release:	Rel-18

Work Item:	XRM

Source:	[CATT]3GPP SA2

To:	3GPP RAN3

Cc:	3GPP RAN2

Contact person:	Chunshan Xiong

chushan.xiong@cictmobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall description

SA2 thanks RAN3 WG for the liaison (S2-2311967/ R3-235890) on Provisioning separate DL and UL PDU Set QoS Parameters to NG-RAN.

Regarding the questions raised by RAN3, SA2 would like to provide the following replies:

Question 1:	Majority companies in RAN3 see benefits and RAN3 kindly asks SA2 to provide the feedback on whether separating all DL and UL PDU Set QoS parameters or some of them from 5GC is necessary

Answer:  	These 3 PDU Set QoS Parameters (i.e. PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER) and PDU Set Integrated Handling Information (PSIHI)) are specified in TS23.501. The QoS Profile may include the PDU Set QoS Parameters and the SMF sends all or part of PDU Set QoS Parameters to NG-RAN as part of the QoS Profile.

A QoS Flow is associated with only one PDU Set Delay Budget. The value of the PDU Set Delay Budget is the same in UL and DL.

A QoS Flow is associated with only one PDU Set Error Rate. The value of the PDU Set Error Rate is the same in UL and DL.

The 3 PDU Set QoS parameters are optional, but at least one of them needs to be included in the QoS profile to support the PDU Set based QoS Handling in the RAN.

2	Actions

To RAN3:

ACTION: 	SA2 kindly asks RAN3 to take the above into account into their standardization work.

3	Dates of next TSG SA WG2 meetings

TSG-SA2 Ad hoc Meeting	22-29 January 2024                    e-meeting

TSG-SA2 Meeting #161		26 February -1 March 2024         Athens, GR