3GPP TSG-WG SA2 Meeting #160	S2-2312116

Chicago, US, November 13 – 17, 2023	(revision of S2-230xxxx)

Source:	SK Telecom

Title:	New KI: Procedure enhancement based on energy related information

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys/Rel-19

Abstract: This document proposes a new Key Issue description regarding the selection of Network Function with considering the energy saving and energy efficiency.

1. Introduction

This document proposes new Key Issues description regarding the following aspects:

- Selection of Networking Function with considering the energy saving and energy efficiency. As stated in SA1 EnergyServ in TR 22.882 and WT#3 highlights the following parts:

- Subject to operator policy and regulatory requirements, the 5G system shall be able to provide a mechanism for one or more network functions to operate based on energy consumption to meet various end-user’s service requirements. - Subject to operator policy and regulatory requirements, the 5G system shall be able to provide means to configure and coordinate the operation of one or more network functions to operate based on MNO's energy consumption requirements and targets. - WT #3. Study 5GS enhancements (e.g., energy usage adjustment for NF from CN aspect, energy saving related decision making, NF selection leveraging NF energy states) for network energy saving including 5GC(NFs) and NG-RAN interactions, analytics, etc. Impacts on the UE are not ruled out e. g., for scenarios specified in TR 22.882 by SA1 EnergyServ.

* Start of change *

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-66

5.x	Key Issue #x: Procedure enhancement based on energy related information

5.x.1	Description

In 5GC architecture, NF needs a mechanism to perform discovery and selection based on energy consumption efficiently. A set of NFs responsible for the call-procedures needs to be aware of overall energy consumption requirements, and how they interact together as a coordinated operation is needed. The NF’s reduced capabilities based on various energy consumption level may need to be known to other NFs for efficient and optimal selection. Furthermore, energy related information exchange between NFs maybe needed to achieve a more holistic selections and decisions.

This key issue will study how to enhance the mechanisms accounting for network energy saving. In particular, this KI will address:

- Whether and how to enhance the NF discovery and selection related functionalities/policy for energy saving or energy consumption, based on e.g., energy related information of the NFs, this also includes:

-	Whether and how to obtain energy saving states and exchange energy related information between NFs.

-	Whether and how to define NF’s reduced capability from different energy saving states during the NF discovery and/or selection.

- Whether and how to control the NF’s traffic processing based on energy saving states of the NF and other criteria (NF’s QoS, overload control, packet-rate control, etc), and

-	Whether, how and when to notify other NFs due to the traffic processing.

NOTE:	The energy saving attributes/states defined by SA5 will be considered during the study of this Key Issue.

* End of change *