SA WG2 Meeting #160	S2-2312143

13 - 17 November, 2023, Chicago, USA

Source:	CableLabs, Charter Communications, Nokia, Nokia Shanghai Bell, Lenovo, InterDigital Inc., ZTE, Meta USA

Title:	pCR 23.700-70: New KI for WT#3.2 on PDU Set based QoS handling support in non-3GPP access

Document for:	Approval

Agenda Item:	19.3

Work Item / Release: 	FS_XRM_Ph2 / Rel-19

Abstract: This paper proposes a new KI for extending PDU Set based QoS handling support to non-3GPP access.

# 1. Introduction

The new approved SID titled Study on Architecture enhancement for XRM Ph2 (SP-231198) includes the below non-3GPP aspects to be studied as part of Rel-19:

WT#3 Further enhancement to support XR based on non-3GPP access.

WT#3.2 Study how PDU Set QoS Control mechanisms can be extended to non-3GPP access networks.

-	Support PDU Set QoS in untrusted/trusted access (e.g. N3IWF, TNGF).

-	Support PDU Set QoS in wireline access (e.g. W-AGF).

This pCR proposes a key issue related to WT#3.2 for PDU Set based QoS handling support in non-3GPP access.

# 2. Proposal

This contribution proposes to include Key Issue x in TR 23.700-70 V0.1.0.

Start of changes

## 5.x	Key Issue #x: Support for PDU Set in non-3GPP access

5.X.1	Description

Support for PDU Set mechanisms are specified in TS 23.501 clause 5.37.5. 5G system supports PDU Set based QoS handling in NG-RAN with the PSA UPF identifying PDUs that belong to PDU Sets based on Protocol Description for PDU Set identification and providing PDU Set Information to the RAN in the GTP-U header. Rel-18 5GS support for PDU Set based handling is limited to NG-RAN access. However, the interaction between the application and 5GS via non-3GPP access is also necessary to enhance efficiency and promote user experience. The user may be serviced by the 5GC via non-3GPP accesses such as trusted, untrusted, or wireline access, such as a device behind RG.

In particular, this KI will study the necessary enhancements/mechanisms to support PDU Set based handling over non-3GPP access:

- The mechanisms needed to provide PDU Set QoS parameters to intermediate non-3GPP access nodes (e.g., N3IWF, TNGF, W-AGF, RGs, etc.), including PDU Set QoS policy control handling at the intermediate non-3GPP access nodes, to support PDU Set based QoS handling in UL and DL in non-3GPP accesses.

- The mechanisms needed to exchange PDU Set Information between 5GC and the intermediate non-3GPP access nodes (e.g., N3IWF, TNGF, W-AGF, RGs, etc.) to support PDU Set based QoS handling in UL and DL in non-3GPP accesses.

End of changes