3GPP TSG-WG SA2 Meeting #160	S2-2312181

Chicago. Nov 13 – 17 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on flow description information

Document for:	Discussion

Agenda Item:	9.9.2

Work Item / Release:	AIMLsys / Rel-18

Abstract: discussing if flow description information should be provided per UE or common to all UEs in the list when a list of UEs is provided by AF over Nnef_AFsessionWithQoS API.

# 1. Introductions

The Nnef_AFsessionWithQoS API was initially designed to accommodate a single UE.

Flow description information or External Application identifier is one of the required inputs.

Flow description information is used to enable the binding functionality and the generation or selection of the service data flow filter(s) in the PCC rules. Flow description information typically comprises so-called the 5-tuple data with source and destination IP addresses, source and destination port numbers, and the protocol information. If an External Application identifier is provided instead of the Flow description information, it is assumed that all the required details are already available in the 5GC that can be mapped to the given External Application identifier.

During R18 AIMLsys work, the Nnef_AFsessionWithQoS API was extended to support a list of UEs for AI/ML use cases, such as FL. This extension has led to some confusion within CT3 regarding whether the flow description info should be provided separately for each UE, or it should be common to all the UEs in the list participating in the same application.

With this context, we received a LS (S2-2311928/C3-233721) from CT3 in SA2#159, which asked the following question:

- Question 1: In case list of UE is provided, whether the flow description information(s) will be provided per UE or the flow description information(s) is common for list of UEs?

During the meeting, there were two competing proposals to address this confusion.

- S2-2310536/S2-2310537 proposed to make the flow description information common to all the UEs in the list by allowing the AF to provide only 3-tuple information (i.e., only server-side IP address and port number, and protocol information), and to make PCF supplement the rest of UE-side information when generating the PCC rules.

- S2-2310227/S2-2310228 proposed that the flow description information should be provided separately for each UE.

This discussion paper is prepared to summarize and analyse the discussion points and make proposals to conclude this issue.

# 2. Discussion points.

- API optimization aspect

- The main purpose of extending the API to accommodate a list of UEs was to optimize signalling between AF and NEF.

- Please note that substantial optimization has already been accomplished by employing common QoS parameters, which number over 20, for all the UEs in the list.

- If using a single, common flow description information can indeed work, it would be more advantageous as it would further optimize the signalling by reducing the size of the request message from the AF.

- Can the flow description information be common to all UEs in the list?

- The server IP address can be common because we are considering the scenario where all the UEs in the list are communicating with the same application server.

- The IP addresses for the list of UEs cannot be common because they must have independent addresses to be identifiable.

- The server port number cannot be common for all cases.

  - If the server uses the same port as source for comparable QoS flows towards all UEs, common server-side port information together with UE IP address and/or the protocol information should be sufficient to distinguish the QoS flows.

  - However, for transport protocols with feedback such as TCP or RTP (with RTCP feedback), different source port numbers (or the next higher port number for RTCP) are typically used because it is essential to use different source port numbers to distinguish between data packets and ACKs.

- The UE port number cannot be common for all cases.

  - If we consider the case where some reserved port numbers are used, then the UE port number can be common to all the UEs in the list. Also, using a wildcard value as the common UE port number might be possible when all the UEs in the list are running only one service or multiple services that uniformly demand the same QoS with the application server.

  - However, because the UE port numbers are typically allocated dynamically by the application, it is frequently not possible that all the UEs use the same port number. In addition, there are many cases where IP flows with the same UE IP address but using distinct UE port numbers are required, and each of these IP flows requires a different QoS. An example would be the case where transport protocols with feedback such as TCP or RTP (with RTCP feedback) are used.

- The protocol information cannot be common for all cases.

  - The protocol information depends on the application design. For example, the application server may support both TCP and QUIC, and the UEs can use either TCP or QUIC to communicate with the server based on its capability. 3GPP shall not mandate that a single transport protocol must be used commonly by all the UEs even for the same application because it should be a decision left to the application as exemplified below.

    - UE1 with the AI/ML model transfer over TCP 14567 or,

    - UE2 with the AI/ML model transfer over QUIC 14567

- Can 5GC supplement the info that cannot be common and allow the AF to provide only the info that can be common?

- If AF provides only the info that can be common, which is only the server IP address, then 5GC should find out the remaining information.

  - The UE IP addresses and the UE port numbers can be known, thus can be supplemented by 5G because they are supposed to be provided by the AF as the target addresses according to clause 4.1.6.13 in TS 23.502. (“For every UE in the set, this list contains the IP address and the port number that is used by the UE for the communication with the AF.”)

  - However, 5GC cannot know the protocol information and the server port number unless they are not provided from the AF.

- Therefore, the answer should be no. 5GC cannot supplement all the information that cannot be common while it is required to setup the PCC rules.

# 3. Conclusions and Proposals

Conclusion 1: The primary objective of extending the API to support a list of UEs, which involves optimizing the signaling between AF and NEF, has already been accomplished by incorporating common QoS parameters and enabling the AF to send a single request instead of multiple requests corresponding to the number of target UEs.

Conclusion 2: As discussed in clause 2, allowing the AF to provide only a single, common flow description information is not possible for all the cases. It is possible only when the server only sends a single QoS stream to multiple UEs.

Proposal 1: It is proposed to endorse the approach that the flow description information should be provided separately for each UE when the server side information and the protocol information are different across the UEs.

Proposal 2: It is proposed to agree on the corresponding CR (S2-2312182) and the reply LS to CT3 (S2-2312180).