3GPP TSG-WG SA2 Meeting #160	S2-2312198

Chicago, USA, 13 - 17 November, 2023	(revision of S2-230xxxx)

Title:	[Draft] LS Reply on Issues on QoS Monitoring parameters for XR service

Response to:	S2-2311927/C3-233649

Release:	Rel-18

Work Item:		XRM

Source:	[CATT] 3GPP SA2

To:	3GPP CT3

Cc:	-

Contact person:	Chunshan Xiong

chushan.xiong@cictmobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall description

SA2 thanks CT3 WG for the liaison (S2-2311927/C3-233649) on QoS Monitoring parameters for XR service.

Regarding the questions raised by CT3, SA2 would like to provide the following replies:

Question 1:	If NG-RAN provides congestion measurements based on NG-RAN implementation, do the reporting period, and periodic and event triggered reporting frequencies apply to the monitoring and exposure of congestion information?

Answer:  	Yes.

Question 2:	If the answer to question 1 is yes, for event triggered reporting, does it mean that the minimum and the maximum congestion reports provided by NG-RAN also need to be reported, if received within the minimum waiting time?

Answer:  	Only one congestion report is provided by the NG-RAN for the reporting.

Question 3:	For data rate monitoring, if the averaging window is shorter than the waiting time, does the UPF report the minimum and maximum monitored data rate or should the UPF report the average?

Answer:  	Only one data rate is reported for the reporting before the expiration of the waiting time.

Question 4:	Do the minimum and the maximum PDV measurement results also need to be reported by the PCF?

Answer:  	Only one PDV measurement is reported by the PCF during each period reporting or event reporting.

Question 5:	For the Congestion information, Data Rate and PDV, does it mean that the measurement failure also needs to be reported if no measurement result is available?

Answer:  	“No measurement result is available” can be reported, and it should be treated as normal case.

Question 6:	If the answer to question 5 is yes, how does the UPF detect that no Congestion or Data Rate measurement result is available?

Answer:  	The UPF can detect congestion as described in clause 5.45.3 and data rate as described in clause 5.45.4. If the UPF cannot get the measurement results (e.g. no congestion state from RAN, the minimum waiting time is shorter than the measurement averaging window), it will provide no measurement available.

Question 7:	Whether all TSC QoS requirements are applicable to multi-modal/XRM communications or not?

Answer:  	In Table 5.27.2-1: TSC Assistance Information (TSCAI) of TS23.501, only two parameters (Flow Direction and Periodicity) are mandatory for TSC and XRM services, all other parameters are optional and N6 Jitter Information can be used for the XRM services. In the R18, the TSC special QoS parameters (e.g. Burst Arrival Time, Survival Time, etc.) are not required to be supported for the XRM services, but the 5G network can support such TSC QoS parameters for both TSC and XRM services based on its local configuration and local policies (e.g. the TSN/TSC network can be used to transport the XR traffic) which is out of scope of 3GPP.

2	Actions

To CT3:

ACTION: 	SA2 kindly asks CT3 to take the above into account into their standardization work.

3	Dates of next TSG SA WG2 meetings

TSG-SA2 Ad hoc Meeting	22-29 January 2024                    e-meeting

TSG-SA2 Meeting #161		26 February -1 March 2024         Athens, GR